# Water Framework Directive 2000/60/EC (WFD) reporting requirements (Groundwater):

- Analysis of water body  characteristics, human impacts/pressures and economic analysis

- Monitoring

- Chemical status

- Quantitative status

- River Basin Management Plan (RBMP)

- After submitting RBMP each 3 years report on progress and plan implementations

1. Reports required by WFD on analysis of water body characteristics, human impacts/pressure, economic analysis

1.1 Initial characterisation:

- Water body identification

Location and boundaries of the groundwater body

- The pressures:

- diffuse sources of pollution

- point sources of pollution

- abstraction

- artificial recharge

- Catchment area

General character of the overlying strata in the catchment area, from which the groundwater body receives its recharge,

- Ecosystem dependency

If ground water are directly dependent on surface water ecosystems or terrestrial ecosystems

| GWB_GroundWaterBody* |  |
| --- | --- |
| FK1,I1 | EURBDCode* |
| U1 | EUGroundWaterBodyCode* |
|  | GWB_MS_CD* |
|  | LAT* |
|  | LON* |
|  | GWB_NAME |
|  | OUT_OF_RBD |
|  | TRANSBOUNDARY |
|  | PROT_AREA_ASSOC* |
|  | METADATA |
|  | URL |

1.2. Further characterisation for w.b. that are identified as being at risk to not comply with WFD:

-geological characteristics

-hydrogeological characteristics of the groundwater body including hydraulic conductivity, porosity and confinement,

-characteristics of the superficial deposits and soils in the catchment from which the groundwater body receives its recharge, including the thickness, porosity, hydraulic conductivity, and absorptive properties of the deposits and soils

-stratification characteristics of the groundwater within the groundwater body,

-an inventory of associated surface systems, including terrestrial ecosystems and bodies of surface water, with which the groundwater body is dynamically linked,

-estimates of the directions and rates of exchange of water between the groundwater body and associated surface systems

-sufficient data to calculate the long term annual average rate of overall recharge

-characterisation of the chemical composition of the groundwater, including specification of the contributions from human activity

| GWB_Hydrogeological_Characteristics |  |
| --- | --- |
| FK1,U1 | EUGroundWaterBodyCode |
|  | Layered |
|  | AREA |
|  | Scale |
|  | ScaleExplanation |
|  | AverageDepth |
|  | AverageThickness |
|  | DepthRange |
|  | LinkSurfaceWaterBodies |
|  | LinkTerrestrialEcosystems |
|  | GeologicalFormation |
|  | VerticalOrientation |
|  | CAPACITY |

1.3 Review of the impact of human activity on groundwaters for water body identified as being at risk of failing to meet the WFD objectives

- location of points in the groundwater body used for the abstraction of water

- annual average rates of abstraction from such points (only if  > 10 m3 /day)

- chemical composition

- location of points in the groundwater body into which water is directly discharged

- rates of discharge at such points

-chemical composition of discharges

-land use in the catchment or catchments from which the groundwater body receives its recharge and pollutant inputs and anthropogenic alterations

| GWB_Pressures |  |
| --- | --- |
| FK1,I1 | EUGroundWaterBodyCode |
|  | SignificantPresureTypes |

1.4 Review of the impact of changes in groundwater levels

-surface water and associated terrestrial ecosystems

-water regulation, flood protection and land drainage

-human development

| GWB_Impacts |  |
| --- | --- |
| FK1,I1 | EUGroundWaterBodyCode |
|  | SignificantImpactTypes |

1.5 Review of the impact of pollution on groundwater quality for w.b. where groundwater is so polluted that achieving good groundwater chemical status is infeasible or disproportionately expensive

-not specified

| GWB_Pressure_Impact_Other |  |
| --- | --- |
| FK1,U1 | EUGroundWaterBodyCode |
|  | OtherPressureDescription |
|  | OtherImpactDescription |

1.6 Economic analysis

- make the relevant calculations for recovery, costs of water services and long term forecasts of supply:

- estimates of the volume, prices and costs associated with water services

- estimates of relevant investment including forecasts of such investments

-make judgments about the most cost-effective combination of measures

2. Reports on monitoring

2.1 Monitoring quantitative status

- Groundwater level regime

| GWB_Quantitative_Status* |  |
| --- | --- |
| FK1,U1 | EUGroundWaterBodyCode* |
|  | QuantitativeStatusValue* |
|  | CommentQuantitativeStatusValue |
|  | GoodQuantitativeStatusExemptionComment |

2.2 Monitoring chemical status

Parameters:

- Conductivity

- Conc. Of pollutants (priority substances - heavy metals and PAHs)

| GWB_Chemical_Status* |  |
| --- | --- |
| FK1,U1 | EUGroundWaterBodyCode* |
|  | ValueChemicalStatus* |
|  | CommentChemicalStatusValue |

Update from Directive on the protection of groundwater against pollution and deterioration 2006/118/EC

(The new Groundwater Directive)

2.3 Monitoring chemical status - Identification of significant upward (/reversal) trends

- M.S. should identify any significant upward(/reversal) and sustained trend in conc. of pollutions and this should be summarized in RBMP

| GWB_TrendReversal* |  |
| --- | --- |
| FK1,U1 | EUGroundWaterBodyCode* |
|  | TrendReversal* |
|  | TrendReversalComment |

| GWB_UpwardTrend* |  |
| --- | --- |
| FK1,U1 | EUGroundWaterBodyCode* |
|  | UpwardTrend* |
|  | UpwardTrendComment |