Model Documentation

# Model Detail

This document provides a complete overview of all element details. For simpler and more focused reports, simply copy this initial template and turn off the sections not required.

## Hydrogeology_v.0.4

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Geology

Detail:		Created on 2011-03-15. Last modified on 2011-04-13

GUID:		{1D131421-002C-43be-B01F-FF2A3006A695}

Hydrogeology_v.0.4 - (Package diagram)

Created By:	Janusz Michalak, Tomasz Nałęcz, Piotr Wojda on 2011-03-12

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{B6B5356A-7145-45b0-95BB-10271DD76B7F}

Figure: 1

### <anonymous>

Type:		Text

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_v.0.4     Keywords:

Detail:		Created on 2007-07-19.  Last modified on 2007-07-19.

GUID:		{F818D41D-0631-4dc4-90E8-A402588FA27D}

### Hydrogeology_Core

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_v.0.4

Detail:		Created on 2011-03-05. Last modified on 2011-03-11

GUID:		{6DCBEC63-B54B-410a-8826-040772798F4E}

HydrogeologyCore_Aquifers - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-06

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{93BF1C87-3F82-42b9-BA34-4C1614EC531B}

Figure: 2

HydrogeologyCore_CodeListsAndEnumerations - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-09

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{A67E5173-619B-4aff-9F3F-7B535C715AF2}

Figure: 3

HydrogeologyCore_Features - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-05

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{F2073104-580C-4e54-B778-B815E52456CF}

Figure: 4

HydrogeologyCore_GenericAndExternalTypes - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-06

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{4B7F4FEA-F359-4653-AB65-52B2F6A8C94C}

Figure: 5

HydrogeologyCore_GeologyMain - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-15

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{97D27D80-89C6-47b3-B6C2-DE12F530C035}

Figure: 6

HydrogeologyCore_Objects - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-05

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{8F4C57E5-8E64-48e1-943F-60E825E9DAD5}

Figure: 7

HydrogeologyCore_Wells - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-05

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{D884CAE8-F215-48b3-8237-8024D6495476}

Figure: 8

#### ActiveWell (or AbstractionWell)

Type:		Class    Well

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-03-07.  Last modified on 2012-02-16.

GUID:		{6CC00DAA-A666-4c7d-B302-AF803FD0B3FB}

-- Definition --

A WaterWell used for the of abstraction of water from a groundwater body

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public ActiveWell (or AbstractionWell) | Public Well |  |
| Dependency Source -> Destination | Public ActiveWell (or AbstractionWell) | Public WaterUseCode |  |
| Generalization Source -> Destination | Public MineralWaterAbstractionWell | Public ActiveWell (or AbstractionWell) |  |
| Association LinkForNavigation Source -> Destination | Public ActiveWell (or AbstractionWell) | Public WaterAppurtenance | Invariant {if ( GroundWaterAbstractionWell.waterUse == forDrinking && WaterAppurtenance. type == well )} |
| Association Bi-Directional | Public isProtectedBy ActiveWell (or AbstractionWell) | Public protects HydrogeologicalZone |  |
| Generalization Source -> Destination | Public ThermalWaterAbstractionWell | Public ActiveWell (or AbstractionWell) |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| quantityOfAbstraction CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |
| waterUse WaterUseCode Public |  | Default: |
| isPartOfComplexIntake boolean Public «voidable» |  | Default: |

#### Aqiuclude

Type:		Class    HydrogeologicalFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-03-09.

GUID:		{33933D61-7C0D-4337-93EE-D64C551FD234}

-- Definition --

An impermeable body of rock or stratum of sediment that acts as a barrier to the flow of groundwater

(From GWML) Impermeable beds of geologic material that hinder or prevent groundwater movement.

http://www.groundwater.org/gi/gwglossary.html

- Note --

(From GWML) A formation which, although porous and capable of absorbing water slowly, will not transmit water fast enough to furnish an appreciable supply for a well or spring. Aquicludes are characterized by very low values of "leakage" (the ratio of vertical Hydraulic Conductivity to thickness), so that they transmit only minor inter-aquifer flow and also have very low rates of yield from compressible storage. Therefore, they constitute boundaries of aquifer flow systems.

http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-a.pdf

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Destination -> Source | Public encloses Aqiuclude | Public isEnclosedBy AquiferSystem |  |
| Generalization Source -> Destination | Public Aqiuclude | Public HydrogeologicalFeature |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| hasLargeHorizontalExtend boolean Public |  | Default: |
| Tickness CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |
| orderNumberFromSurface int Public |  | Default: |

#### Aquifer

Type:		Class    HydrogeologicalFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-03-09.

GUID:		{F16A72D6-43E2-4eab-956B-1792D95EF6BA}

-- Definition --

(From GWML) A formation, group of formations, or part of a formation that contains sufficient saturated, permeable material to yield significant quantities of water to wells and springs. (USGS)

-- Note --

(From GWML) An underground geological formation able to store and yield water.

http://www.groundwater.org/gi/gwglossary.html

An aquifer is an underground layer of water-bearing permeable rock or unconsolidated materials (gravel, sand, silt, or clay) from which groundwater can be usefully extracted using a water well.

http://en.wikipedia.org/wiki/Aquifer"

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public isPartOf Aquifer | Public isCollectionOf AquiferSystem |  |
| Generalization Source -> Destination | Public ThermalWaterAquifer | Public Aquifer |  |
| Generalization Source -> Destination | Public Aquifer | Public HydrogeologicalFeature |  |
| Generalization Source -> Destination | Public MineralWaterAquifer | Public Aquifer |  |
| Aggregation DescribesParametersOf Source -> Destination | Public AquiferDescription | Public Aquifer |  |
| Association BaseOfDrainage (LinksForNavigation) Source -> Destination | Public isDrained Aquifer | Public draines SurfaceWater |  |
| Generalization Source -> Destination | Public KarsticAquifer | Public Aquifer |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| hasLargeHorizontalExtend Boolean Public |  | Default: |
| isConfined Boolean Public |  | Default: |
| isExploited Boolean Public |  | Default: |
| IsMainInSystem Boolean Public |  | Default: |
| PermeabilityCoefficient CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |
| Tickness CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |
| orderNumberFromSurface int Public |  | Default: |
| vulnerabilityToPollution CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |

#### AquiferSystem

Type:		Class    HydrogeologicalFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-03-09.

GUID:		{337F05A0-3C7C-4bed-8D63-3CD8D01D893E}

-- Definition --

(From GWML) Collection of aquifers composing a system of interacting components.

Aquifer System - A collection of all hydrogeological system elements, which together constitute the environment of groundwater - a " communicating vessels"  filled or can be filled with water. Attributes of Aquifer System and its components determine the feasibility of water collection, its movement, as well as the impact on its chemical state.

-- Note --

Aquifer System components and their attributes (including geometry) are the most stable over time except special cases.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Destination -> Source | Public encloses Aqiuclude | Public isEnclosedBy AquiferSystem |  |
| Aggregation Source -> Destination | Public isPartOf Aquifer | Public isCollectionOf AquiferSystem |  |
| Aggregation Source -> Destination | Public isPartOf Aquitard | Public isCollectionOf AquiferSystem |  |
| Aggregation belongsTo Source -> Destination | Public AquiferSystem | Public formedBy HydrogeologicalSystem |  |
| Generalization ConstantInTime Source -> Destination | Public AquiferSystem | Public HydrogeologicalFeature |  |
| Generalization Source -> Destination | Public PorousAquiferSystem | Public AquiferSystem |  |
| Generalization Source -> Destination | Public CompoudAquiferSystem | Public AquiferSystem |  |
| Generalization Source -> Destination | Public FracturedAquiferSystem | Public AquiferSystem |  |
| Generalization Source -> Destination | Public KarsticAquiferSystem | Public AquiferSystem |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| isLayered Boolean Public «voidable» |  | Default: |
| isComplex boolean Public «voidable» |  | Default: |
| mediaType AquiferMediaType Public «voidable» |  | Default: |
| aquiferType AquiferType Public «voidable» |  | Default: |

#### Aquitard

Type:		Class    HydrogeologicalFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-03-09.

GUID:		{4FCBF9D0-CBC9-4454-83AE-8DE1EB6D297A}

-- Definition --

(From GWML) A saturated, but poorly permeable bed that impedes ground-water movement and does not yield water freely to wells, but which may   transmit appreciable water to or from adjacent aquifers and, where sufficiently thick, may constitute an important ground-water storage unit.

-- Note --

(From GWML) Aquitards are characterized by values of leakance that may range from relatively low to relatively high. Areally extensive aquitards of relatively low leakance may function regionally as boundaries of aquifer flow systems.

http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-a.pdf"

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation DescribesParamitersOf Source -> Destination | Public AquiferDescription | Public Aquitard |  |
| Aggregation Source -> Destination | Public isPartOf Aquitard | Public isCollectionOf AquiferSystem |  |
| Generalization Source -> Destination | Public Aquitard | Public HydrogeologicalFeature |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| hesLargeHorizontalExtend Boolean Public |  | Default: |
| PermeabilityCoefficient CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |
| Tickness CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |
| orderNumberFromSurface int Public |  | Default: |

#### GroundWaterBody

Type:		Class    HydrogeologicalFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-03-13.

GUID:		{67A1D683-550E-47bf-B05A-A2332AD009D7}

-- Definition --

(From GWML) A distinct volume of groundwater within an aquifer or aquifers (Vogt, 2002). The equivalent in the European Water Framework Directive (2000/60/CE, 2000) is "Body of groundwater" defined exactly in the same way.

-- Note --

GroundWaterBody may consist of smaller occurrence GroundWaterBody and consequently can form a hierarchical structure (ComplexGroundWaterBody). Part of the occurrences in the hierarchy is WFDGroundWaterBody in AM theme.

GroundWaterBody adjacent to the other GroundWaterBodys.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association neighbourhoot Bi-Directional | Public GroundWaterBody | Public GroundWaterBody |  |
| Generalization VariableInTime Source -> Destination | Public GroundWaterBody | Public HydrogeologicalFeature |  |
| Aggregation Aggregate Source -> Destination | Public belongsTo GroundWaterBody | Public contains GroundWaterFlowSystem |  |
| Aggregation nesting Bi-Directional | Public GroundWaterBody | Public GroundWaterBody |  |
| Dependency Source -> Destination | Public GroundWaterBody | Public ConditionOfGrounwater |  |
| Generalization Source -> Destination | Public ThermallGroundWaterBody | Public GroundWaterBody |  |
| Generalization Source -> Destination | Public MineralGroundWaterBody | Public GroundWaterBody |  |
| Generalization Source -> Destination | Public WFD_GroundWaterBody | Public GroundWaterBody |  |
| Generalization Source -> Destination | Public KarsticGroundWaterBody | Public GroundWaterBody |  |
| Dependency Destination -> Source | Public WaterType | Public GroundWaterBody |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| mineralization WaterType Public «voidable» |  | Default: |
| conditionOfGroundWaterBody ConditionOfGrounwater Public «voidable» |  | Default: |

#### GroundWaterFlowSystem

Type:		Class    HydrogeologicalFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-03-09.

GUID:		{D09903FA-D6C4-46fc-B048-63D615045305}

-- Definition --

GroundWaterFlowSystem is created by groundwater bodies and both GroundWaterFlowSystem and GroundWaterFlowSystem change over time, and these changes are caused by external factors.

-- Note --

Variable GroundWaterFlowSystem state defines the entire state of HydrogeologicalSystem. Frequently, such factors related to HydrogeologicalSystem and highly impact on the GroundWaterFlowSystem are the different human impacts, for example, groundwater intakes, drainage systems and hydro-weirs.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization VariableInTime Source -> Destination | Public GroundWaterFlowSystem | Public HydrogeologicalFeature |  |
| Aggregation Aggregate Source -> Destination | Public belongsTo GroundWaterBody | Public contains GroundWaterFlowSystem |  |
| Dependency Destination -> Source | Public ConditionOfGrounwater | Public GroundWaterFlowSystem |  |
| Aggregation belongsTo Source -> Destination | Public GroundWaterFlowSystem | Public contains HydrogeologicalSystem |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| conditionOfFlowSystem ConditionOfGrounwater Public |  | Default: |

#### GroundWaterMonitoringStation

Type:		Class    HydrogeologicalObjectManMade

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-03-07.  Last modified on 2011-04-12.

GUID:		{F92489D6-33EA-4be1-9E70-B17B4C6DB60E}

-- Definition --

Class GroundWaterMonitoringStation realizes class Station from ISO 19156 standard in hydrogeology domain.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public GroundWaterMonitoringStation | Public StationType |  |
| Dependency Source -> Destination | Public GroundWaterMonitoringStation | Public StationLevel |  |
| Generalization Source -> Destination | Public GroundWaterMonitoringStation | Public HydrogeologicalObjectManMade |  |
| Aggregation Source -> Destination | Public ObservationWell | Public GroundWaterMonitoringStation |  |
| Association LinkForNavigation Source -> Destination | Public GroundWaterMonitoringStation | Public MonitoringFacility | Invariant {if ( MonitoringFacility.monitoredMedium == groundWater )} |
| Generalization Source -> Destination | Public WFD_GW_MonitoringStation | Public GroundWaterMonitoringStation |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| level StatonLevel Public «voidable» |  | Default: |
| type StationType Public «voidable» |  | Default: |

#### HydrogeologicalFeature

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-02-15.  Last modified on 2011-04-13.

GUID:		{4B4A6F69-0E8D-4d7b-9533-67F4953E70D8}

-- Definition --

Top level abstract class for everything hydrogeological what is feature, but what is not hydrogeological object - what not conform definition of object.

-- Notes --

The most often, it is part of underground space delineated with physical, lithological, stratygraphic or other criterion.

This meaning is slight different then one in GWML model. Bodies of groundwater filling these spaces are hydrogeological features too.

Examples of hydrogeological features: hydrogeological unite, aquifer, aquitard, aquiclude, aquifer system, hydrogeological system, groundwater flow system and groundwater body.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association SpatialApparence Bi-Directional | Public presents HydrogeologicalFeatureAppearence | Public isPresented HydrogeologicalFeature |  |
| Generalization VariableInTime Source -> Destination | Public GroundWaterFlowSystem | Public HydrogeologicalFeature |  |
| Generalization VariableInTime Source -> Destination | Public GroundWaterBody | Public HydrogeologicalFeature |  |
| Generalization Source -> Destination | Public Aquifer | Public HydrogeologicalFeature |  |
| Generalization TheMostComplexFeature Source -> Destination | Public HydrogeologicalSystem | Public HydrogeologicalFeature |  |
| Generalization Source -> Destination | Public WFD_ProtectedArea | Public HydrogeologicalFeature |  |
| Generalization Source -> Destination | Public Aqiuclude | Public HydrogeologicalFeature |  |
| Generalization Source -> Destination | Public Aquitard | Public HydrogeologicalFeature |  |
| Association Bi-Directional | Public HydrogeologicalFeature | Public HydrogeologicalNamedPlace |  |
| Generalization ConstantInTime Source -> Destination | Public AquiferSystem | Public HydrogeologicalFeature |  |
| Generalization Source -> Destination | Public HydrogeologicalZone | Public HydrogeologicalFeature |  |
| Generalization Source -> Destination | Public ConeOfDepression | Public HydrogeologicalFeature |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| beginLifespanObject DateTime Public «lifeCycleInfo - voidable» |  | Default: |
| description PT_FreeText Public «voidable» |  | Default: |
| endLifespanObject DateTime Public [0..1] «lifeCycleInfo - voidable» |  | Default: |
| inspireId Identifier Public |  | Default: |
| name PT_FreeText Public «voidable» |  | Default: |

#### HydrogeologicalFeatureAppearence

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-02-15.  Last modified on 2011-04-12.

GUID:		{ACC15AB6-8C14-4242-83DE-08D14DF17D23}

-- Definition --

HydrogeologicalFeatureAppearence class is a geometric representation of HydrogeologicalFeature calaas. HydrogeologicalFeature instance (object) could be represented by many various instances of HydrogeologicalFeatureAppearence class.

-- Note --

Class equivalent to HydrogeologicMappedFeature in GWML model and GeologicMappedFeature in GeoSciML:

"Mapped feature more oriented on spatial temporal (although geometries could handle this).

A physical hydrogeologic feature represented on a map."

Here, meaning of this class is wider then in GWML and similar to MappedFeature class in GeoSciML:

A MappedFeature is part of a geological interpretation.

It provides a link between a notional feature (description package) and one spatial representation of it, or part of it. (Exposures, Surface Traces and Intercepts, etc)

* the specific bounded occurrence, such as an outcrop or map polygon

* the Mapped Feature carries a geometry or shape

o the association with a Geologic Feature (legend item) provides specification of all the other descriptors

o the association with a Sampling Feature provides the context and dimensionality

A Mapped Feature is always associated with some sampling feature - e.g. a mapping surface, a section, a Borehole (see BoreHolesAndObservation) etc.  As noted on the diagram, if the associated sampling feature is a Borehole, then the shape associated with the MappedFeature will usually be either a point or an interval. This reconciles the 2-D ("map", section) and 1-D (borehole, traverse) viewpoints in a common abstraction."

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association SpatialApparence Bi-Directional | Public presents HydrogeologicalFeatureAppearence | Public isPresented HydrogeologicalFeature |  |
| Association Source -> Destination | Public usesDataFrom HydrogeologicalFeatureAppearence | Public canBeApplied MappedFeature | Invariant {if HydrogeologicalFeature is GeologicFeature} |
| Dependency Destination -> Source | Public TypeOfSourceForGeometry | Public HydrogeologicalFeatureAppearence |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| geometry GEh_GeometryType Public |  | Default: |
| geometrySource TypeOfSourceForGeometry Public |  | Default: |
| inspireId Identifier Public |  | Default: |
| description PT_FreeText Public «voidable» |  | Default: |

#### HydrogeologicalObject

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-02-15.  Last modified on 2011-03-13.

GUID:		{D6C8AA4D-9FFB-4abd-AAE4-7575F37DA97C}

-- Definition --

An abstract class for everything that     meets the definition of     hydrogeological as well as being an     object:

An  object is an abstract entity     representing or describing a thing or     concept seen in the real world. The     object is distinguishable from other     objects, has a name and a well defined borders. (Subieta, 1999)

Each object has four  characteristics:

* identity, a feature to identify it  and distinguish it from other objects;

* state, which is the current state of data elements;

* clearly definined boundary;

* behavior.

Hydrogeological objects may be     natural (eg. spring) and the     manmade (eg. wells). The vast     majority of hydrogeological objects     is manmade.

The object is not part of the     underground space extracted based     on some physical or litho-    sthratigraphic characteristics, for     example, Aquifer, Groundwater     body or Groundwater table.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public HydrogeologicalObjectNatural | Public HydrogeologicalObject |  |
| Generalization Source -> Destination | Public HydrogeologicalObjectManMade | Public HydrogeologicalObject |  |
| Association Bi-Directional | Public HydrogeologicalObject | Public HydrogeologicalNamedPlace |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| inspireId Identifier Public |  | Default: |
| geometry GM_Primitive Public |  | Default: |
| name PT_FreeText Public «voidable» |  | Default: |
| description PT_FreeText Public «voidable» |  | Default: |
| beginLifespanObject DateTime Public «lifeCycleInfo - voidable» |  | Default: |
| endLifespanObject DateTime Public [0..1] «lifeCycleInfo - voidable» |  | Default: |

#### HydrogeologicalObjectManMade

Type:		Class    HydrogeologicalObject

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-02-15.  Last modified on 2011-04-13.

GUID:		{0EB16068-3820-499e-BC7B-392F0C7EEB42}

Examples of manmade hydrogeological objects are: well, groundwater intake, groundwater monitoring station or monitoring well.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public Well | Public HydrogeologicalObjectManMade |  |
| Generalization Source -> Destination | Public HydrogeologicalObjectManMade | Public HydrogeologicalObject |  |
| Generalization Source -> Destination | Public GroundWaterMonitoringStation | Public HydrogeologicalObjectManMade |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| validFrom DateTime Public «lifeCycleInfo - voidable» |  | Default: |
| validTo DateTime Public [0..1] «lifeCycleInfo - voidable» |  | Default: |

#### HydrogeologicalObjectNatural

Type:		Class    HydrogeologicalObject

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-02-15.  Last modified on 2011-03-14.

GUID:		{77D5F62C-04BE-4a4d-8BAF-557EC510D4D5}

Examples of natural hydrogeological objects are: a source, vanishing point and geyser.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public HydrogeologicalObjectNatural | Public HydrogeologicalObject |  |
| Dependency Source -> Destination | Public HydrogeologicalObjectNatural | Public WaterPersistenceValue |  |
| Association LinkForNavigation Source -> Destination | Public HydrogeologicalObjectNatural | Public ProtectedSite | Invariant {if(hydrogeologicalObjectNatural is protected)} |
| Dependency Source -> Destination | Public HydrogeologicalObjectNatural | Public NaturalObjectType |  |
| Generalization Source -> Destination | Public Spring | Public HydrogeologicalObjectNatural |  |
| Generalization Source -> Destination | Public Seep | Public HydrogeologicalObjectNatural |  |
| Generalization Source -> Destination | Public SwallowHole | Public HydrogeologicalObjectNatural |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| naturalObjectType NaturalObjectType Public «voidable» |  | Default: |
| waterPersistance WaterPersistanceValue Public «voidable - estimatedProperty» |  | Default: |
| quantityOfFlow CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |

#### HydrogeologicalSystem

Type:		Class    HydrogeologicalFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-03-10.

GUID:		{27504610-6C9E-4f98-8A37-DB4BC283F6B5}

-- Definition --

Part of the Earth's crust which is the environment consisting of a groundwater aquifer system (Aquifer System - invariable in time) and filled with the groundwater flow system (Groundwater Flow System - variable in time). Both these components are complex, interrelated and their components are also interrelated.

-- Note --

General hydrogeological relationships in the system:

- External Factors (External Factors, EF) influence the HydrogeologicalSystem.

- The HydrogeologicalSystem response to these factors (EF) determined by the properties of the system   (Aquifer System).

- The answer to the HydrogeologicalSystem state of EF shall be expressed by GWFS.

A detailed description of these relationships and elements that shape them contain a classes derived from classes GroundWaterFlowSystem  and AquiferSystem or are related to them.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation belongsTo Source -> Destination | Public GroundWaterFlowSystem | Public contains HydrogeologicalSystem |  |
| Generalization TheMostComplexFeature Source -> Destination | Public HydrogeologicalSystem | Public HydrogeologicalFeature |  |
| Aggregation belongsTo Source -> Destination | Public AquiferSystem | Public formedBy HydrogeologicalSystem |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| isMain Boolean Public «voidable» |  | Default: |

#### ObservationWell

Type:		Class    Well

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-03-07.  Last modified on 2012-02-16.

GUID:		{F1E98E5F-F9D6-43f3-ACCE-B967B37AF9AF}

-- Definition --

Class GroundWaterObservation realizes class ObservationWell from ISO 19156 standard in hydrogeology domain.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public ObservationWell | Public Well |  |
| Aggregation Source -> Destination | Public ObservationWell | Public GroundWaterMonitoringStation |  |
| Association LinkForNavigation Source -> Destination | Public ObservationWell | Public MonitoringFacility | Invariant {if ( MonitoringFacility.monitoredMedium == groundWater )} |
| Dependency Source -> Destination | Public ObservationWell | Public ObservationType |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| observation ObsrevatinType Public [1..*] «voidable» |  | Default: |

#### WFD_GroundWaterBody

Type:		Class    GroundWaterBody

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-04-13.

GUID:		{70EE2E18-6A47-4aea-918B-6FCB5ED1FB51}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public GrounWaterBodyDescription | Public WFD_GroundWaterBody |  |
| Generalization Source -> Destination | Public WFD_GroundWaterBody | Public GroundWaterBody |  |
| Association LinkForNavigation Source -> Destination | Public WFD_GroundWaterBody | Public WFDGroundWaterBody |  |
| Association LinkForNavigation Source -> Destination | Public WFD_GroundWaterBody | Public ProtectedSite | Invariant {If ProtectedSite is dependent on GroudWaterBody -> if ( GrounWaterBody.isAssociatedWithProtectedAreas == truth )} |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| EU_GWB_code FeatureUniqueEUCodeType Public | Unique Code for the Groundwater Body at EU level | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| GWB_MS_code FeatureUniqueCodeType Public | Unique Code for the Groundwater Body within the Member State | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| GWB_name PT_FreeText Public «voidable» | Locally-used name of groundwater body if known | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| isOutOf_RBD boolean Public «voidable» | Indicate whether or not parts of the Groundwater Body are outside the RBD. Yes or No | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| isTrannsboundary boolean Public «voidable» | Does the Groundwater Body cross a country border? Yes or No | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| URL anyURI Public «voidable» | URL for integration of MS own internet-based information | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| centroid GM_Point Public |  | Default: |

#### Well

Type:		Class    HydrogeologicalObjectManMade

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Core     Keywords:

Detail:		Created on 2011-02-19.  Last modified on 2012-02-16.

GUID:		{5DD7422B-248F-45a8-B462-B2244F293385}

--Definition --

(from GWML) An excavation where the intended use is for location, acquisition, development, or artificial recharge of ground water.

http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-w.pdf

A water well is an excavation or structure created in the ground ? by digging, driving, boring or drilling to access water in underground aquifers.

http://en.wikipedia.org/wiki/Water_well

| Constraints |  |
| --- | --- |
|  | inheritance restriction: geometry is applicable if Borehole is void: (Invariant, Status is Approved) |
|  | inheritance restriction: type of geometry is GM_Point: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public HydrogeologicalProfile | Public Well |  |
| Generalization Source -> Destination | Public ActiveWell (or AbstractionWell) | Public Well |  |
| Generalization Source -> Destination | Public ObservationWell | Public Well |  |
| Generalization Source -> Destination | Public Well | Public HydrogeologicalObjectManMade |  |
| Dependency Source -> Destination | Public Well | Public WellPurposeCode |  |
| Dependency Source -> Destination | Public Well | Public WellUse |  |
| Aggregation Source -> Destination | Public isPartOf Borehole | Public isBasedOn Well |  |
| Dependency Source -> Destination | Public Well | Public WellStatusCode |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| usage WellUse Public |  | Default: |
| wellStatus WellStatusCode Public «voidable» |  | Default: |
| wellPurpose WellPurposeCode Public «voidable» |  | Default: |
| geometry GM_Point Public «voidable - restricted» |  | Default: |

#### HydrogeologyCore_CodeListsAndEnumerations

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_Core

Detail:		Created on 2011-03-09. Last modified on 2011-04-11

GUID:		{63393A05-FD2F-49de-B989-E20F3CC4D597}

##### AquiferMediaType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2011-04-11.  Last modified on 2011-04-11.

GUID:		{38AE0070-57CF-48f6-9F41-BE2D6DDAFE10}

Imported from GWML

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| fractured Public «enum» |  | Default: |
| porous Public «enum» |  | Default: |
| karstic Public «enum» |  | Default: |
| compound Public «enum» |  | Default: |

##### AquiferType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2011-04-11.  Last modified on 2011-04-11.

GUID:		{AD576A81-3DFC-4f6e-B1C1-6653DA4A0006}

Imported from GWML

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| confinedSubArtesian Public «enum» |  | Default: |
| confinedArtesian Public «enum» |  | Default: |
| unconfined Public «enum» |  | Default: |

##### ConditionOfGrounwater

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2011-03-05.  Last modified on 2011-04-12.

GUID:		{6C2D1265-3CDE-45ee-ABEB-3B21A2829D8B}

-- Definition --

GroundWaterFlowSystem and its parts (GroundWaterBody) are variable during the state dependent on external factors, which are the most diverse human activities. Enumeration ConditonOfGroundwater determines the approximate degree of these changes.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public GroundWaterBody | Public ConditionOfGrounwater |  |
| Dependency Destination -> Source | Public ConditionOfGrounwater | Public GroundWaterFlowSystem |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| natural Public «enum» |  | Default: |
| lightlyModified Public «enum» |  | Default: |
| modified Public «enum» |  | Default: |
| stronglyModified Public «enum» |  | Default: |

##### NaturalObjectType

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2011-04-13.  Last modified on 2011-04-13.

GUID:		{AEB6A2C6-4176-48b4-86F2-C89F361388E5}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public HydrogeologicalObjectNatural | Public NaturalObjectType |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| spring Public |  | Default: |
| seep Public |  | Default: |
| swallowHole Public |  | Default: |
| other Public |  | Default: |

##### ObservationType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2011-03-07.  Last modified on 2011-04-11.

GUID:		{2DDBECB1-619F-4701-BDBE-92820D75D35D}

-- Definition --

A type of an observation made on the feature of interest.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public ObservationWell | Public ObservationType |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| diagnostics Public «enum» |  | Default: |
| surveillance Public «enum» |  | Default: |
| research Public «enum» |  | Default: |
| industrial Public «enum» |  | Default: |

##### StationLevel

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2011-03-07.  Last modified on 2011-04-11.

GUID:		{C0339B21-8B11-4644-9A54-5174039C4B89}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public GroundWaterMonitoringStation | Public StationLevel |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| basicNetwork Public «enum» |  | Default: |
| hydrologicalBenchmark Public «enum» |  | Default: |
| baseLine Public «enum» |  | Default: |
| specificNetwork Public «enum» |  | Default: |
| temporalNetwork Public «enum» |  | Default: |

##### StationType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2011-03-07.  Last modified on 2011-04-11.

GUID:		{4EACB3C5-8011-4754-AD63-0FF50DF670FE}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public GroundWaterMonitoringStation | Public StationType |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| quality Public «enum» |  | Default: |
| quantity Public «enum» |  | Default: |

##### TypeOfSourceForGeometry

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2011-02-19.  Last modified on 2011-04-11.

GUID:		{AA8322E6-79A4-49db-B8B2-EDEE047F745D}

-- Definition --

Determination of the feature geometry information source or the way of geometry mark.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Destination -> Source | Public TypeOfSourceForGeometry | Public HydrogeologicalFeatureAppearence |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| borholeM&O <undefined> Public |  | Default: |
| cartographicalM&O <undefined> Public |  | Default: |
| fiat <undefined> Public |  | Default: |
| fromMap <undefined> Public |  | Default: |
| fuzzy <undefined> Public |  | Default: |
| genuine <undefined> Public |  | Default: |
| geophisicalM&O <undefined> Public |  | Default: |
| hipotetical <undefined> Public |  | Default: |
| hipoteticalVerifed <undefined> Public |  | Default: |
| other Public |  | Default: |

##### WaterPersistenceValue

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2008-09-29.  Last modified on 2011-04-11.

GUID:		{346035A4-7320-42cc-A9B7-09729595FF03}

-- Definition --

Categories of hydrological persistence of water.

(from GWML - modified)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | asDictionary = true. Values: true Default: true Description: Encode code list as externally managed dictionary (applies to ISO 19136:2007 encoding rule). Always true in INSPIRE. |
|  | codeList = urn:x-inspire:def:codeList:INSPIRE:HydrologicalPersistenceValue. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007_INSPIRE_Extensions Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public HydrogeologicalObjectNatural | Public WaterPersistenceValue |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| ephemeral Public | -- Definition -- Filled and/or flowing during and immediately after precipitation. -- Description -- SOURCE [DFDD]. | Default: |
| intermittent Public | -- Definition -- Filled and/or flowing for part of the year. -- Description -- SOURCE [DFDD]. | Default: |
| seasonal Public |  | Default: |
| perennial Public | -- Definition -- Filled and/or flowing continuously throughout the year as its bed lies below the water table. -- Description -- SOURCE [DFDD]. | Default: |

##### WaterType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2008-12-05.  Last modified on 2011-04-11.

GUID:		{1CF36D41-91E7-479f-AF3F-73B9619506A4}

-- Definition --

(From GWML) This enumeration class refers to the concept of salinity and its classes in water.

-- Note --

(From GWML) Salinity is the saltiness or dissolved salt content of a body of water. http://en.wikipedia.org/wiki/Salinity

Generally, the concentration of mineral salts dissolved in water. Salinity may be expressed in terms of a concentration or as electrical conductivity. When describing salinity influenced by seawater, salinity often refers to the concentration of chlorides in the water. See also total dissolved solids. http://www.groundwater.water.ca.gov/groundwater_basics/gwb_glossary/index.cfm#ss

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | asDictionary = true. Values: true Default: true Description: Encode code list as externally managed dictionary (applies to ISO 19136:2007 encoding rule). Always true in INSPIRE. |
|  | codeSpace = . Description: URI that identifies the authority for this dictionary and its members |
|  | dictionaryIdentifier = . Description: URI that identifies this dictionary |
|  | memberIdentifierStem = . Description: URI stem used for dictionary members |
|  | xsdEncodingRule = iso19136_2007. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007 Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Destination -> Source | Public WaterType | Public GroundWaterBody |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| ultraFreshWater Public | TBD | Default: [sequenceNumber = 10 ] |
| freshWater Public | A- Freshwater is a word that refers to bodies of water such as ponds, lakes, rivers and streams containing low concentrations of dissolved salts and other total dissolved solids. In other words, the term excludes seawater and brackish water. Freshwater can also be the output of desalinated seawater. http://en.wikipedia.org/wiki/Freshwater Fresh water: Water with less than 0.5 parts per thousand dissolved salts. | Default: [sequenceNumber = 20 ] |
| acratopegae Public | From Ancient Greek akratos ( "pure, unmixed") and PEGE ( "source, fountain). Water with low mineral content or less than mineral water. Translated from : Du grec ancien akratos (" pur, sans melange ") et pege (" source, fontaine "). Qui a, en parlant d'une eau, une faible teneur en sels mineraux ou une teneur moindre que celle d'une eau minerale. http://fr.wiktionary.org/wiki/acratop%C3%A8ge | Default: [sequenceNumber = 30 ] |
| brackishWater Public | Brackish water (less commonly brack water) is water that has more salinity than fresh water, but not as much as seawater. It may result from mixing of seawater with fresh water, as in estuaries, or it may occur in brackish fossil aquifers. The word comes from the Middle Dutch root "brak," meaning "salten" or "salty" http://en.wikipedia.org/wiki/Brackish | Default: [sequenceNumber = 40 ] |
| saltWater Public | Saline water is a general term for water that contains a significant concentration of dissolved salts (NaCl). The concentration is usually expressed in parts per million (ppm) of salt. The salinity concentration level used by United States Geological Survey classifies saline water in three categories. Slightly saline water contains around 1,000 to 3,000 ppm. Moderately saline water contains roughly 3,000 to 10,000 ppm. Highly saline water has around 10,000 to 35,000 ppm of salt. Seawater has a salinity of roughly 35,000 ppm, equivalent to 35 g/L. Technically, brackish water contains between 0.5 to 30 grams of salt per litre?more often expressed as 0.5 to 30 parts per thousand (ppt). Thus, brackish covers a range of salinity regimes and is not considered a precisely defined condition. http://en.wikipedia.org/wiki/Saline_water | Default: [sequenceNumber = 50 ] |
| brineWater Public | Brine (lat. saltus) is water saturated or nearly saturated with salt (NaCl). http://en.wikipedia.org/wiki/Brine | Default: [sequenceNumber = 60 ] |

##### WaterUseCode

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2006-01-24.  Last modified on 2011-04-11.

GUID:		{CCC02192-582D-4e5e-AF98-022235F228D0}

-- Definition --

(from GWML) The use of water may be classified by specific types according to distinctive uses.

http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-w.pdf

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | asDictionary = true. Values: true Default: true Description: Encode code list as externally managed dictionary (applies to ISO 19136:2007 encoding rule). Always true in INSPIRE. |
|  | codeSpace = . Description: URI that identifies the authority for this dictionary and its members |
|  | dictionaryIdentifier = . Description: URI that identifies this dictionary |
|  | memberIdentifierStem = . Description: URI stem used for dictionary members |
|  | xsdEncodingRule = iso19136_2007. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007 Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public ActiveWell (or AbstractionWell) | Public WaterUseCode |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| agriculture Public | Agricultural Use. The use of any tract of land for the production of animal or vegetable life; uses include, but are not limited to, the pasturing, grazing, and watering of livestock and the cropping, cultivation, and harvesting of plants. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-a.pdf | Default: |
| commercial Public | Commercial Water Use (Withdrawals). Water for motels, hotels, restaurants, office buildings, and other commercial facilities and institutions, both civilian and military. The water may be obtained from a public supply or may be self supplied. The terms "water use" and "water withdrawals" are equivalent, but not the same as "Consumptive Use" as they do not account for return flows. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-c.pdf | Default: |
| domestic Public | Domestic Water Use (Withdrawals). Water used normally for residential purposes, including household use, personal hygiene, drinking, washing clothes and dishes, flushing toilets, watering of domestic animals, and outside uses such as car washing, swimming pools, and for lawns, gardens, trees and shrubs. The water may be obtained from a public supply or may be self supplied. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-d.pdf | Default: |
| heatTransfert Public | Water used as an agent to facilitate heat trasfer. Heat Transfer Agent. A liquid or gas that functions in a Heat Exchanger to facilitate the movement of heat from one location to another. For example, the engine coolant in an automobile serves to transfer heat from the engine block to the atmosphere. likewise, water facilitates the movement of heat from the reactor core to the outside of a nuclear reactor. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-h.pdf | Default: |
| industrial Public | Industrial Water Use (Withdrawals) ? Industrial water use includes water used for processing activities, washing, and cooling. Major water-using manufacturing industries include food processing, textile and apparel products, lumber, furniture and wood products, paper production, printing and publishing, chemicals, petroleum, rubber products, stone, clay, glass and concrete products, primary and fabricated metal industries, industrial and commercial equipment and electrical, electronic and measuring equipment and transportation equipment. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-i.pdf | Default: |
| irrigation Public | Irrigation Water Use (Withdrawals). Artificial application of water on lands to assist in the growing of crops and pastures or to maintain vegetative growth on recreational lands, such as parks and golf courses. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-i.pdf | Default: |
| otherUnknow Public | Other Water Use. Water used for such purposes as heating, cooling, irrigation (public-supplied only), lake augmentation, and other nonspecific uses. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-o.pdf | Default: |
| publicRecreation Public | TBD | Default: |
| research Public | TBD | Default: |
| municipal Public | M&I (Municipal and Industrial) Water Withdrawals (Use). Water supplied for municipal and industrial uses provided through a municipal distribution system. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-m.pdf Public Water Use. Water supplied from a Public Water Supply System (PWSS) and used for such purposes as fire fighting, street washing, and municipal parks, golf courses, and swimming pools. Public water use also includes system water losses (water lost to leakage) and brine water discharged from desalination facilities. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-p.pdf | Default: |
| forDrinking Public |  | Default: |

##### WellPurposeCode

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2006-01-24.  Last modified on 2011-04-11.

GUID:		{9B3825A0-746F-4af8-93A7-3C9BF48895DA}

-- Definition --

Code associated to define the purpose of the well.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | asDictionary = true. Values: true Default: true Description: Encode code list as externally managed dictionary (applies to ISO 19136:2007 encoding rule). Always true in INSPIRE. |
|  | codeSpace = . Description: URI that identifies the authority for this dictionary and its members |
|  | dictionaryIdentifier = . Description: URI that identifies this dictionary |
|  | memberIdentifierStem = . Description: URI stem used for dictionary members |
|  | xsdEncodingRule = iso19136_2007. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007 Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public Well | Public WellPurposeCode |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| cathodicProtection Public | Other, less common types of wells include cathodic protection wells. Cathodic protection wells, sometimes called 'deep groundbeds,' house devices to minimize electrolytic corrosion of metallic pipelines, tanks, and other facilities in contact with the ground. http://www.dpla.water.ca.gov/sd/groundwater/california_well_standards/cpws/cpws_introduction.html | Default: |
| coalELog Public | TBD | Default: |
| core Public | TBD | Default: |
| decontamination Public | TBD | Default: |
| dewatering Public | ewatering is the removal of water from solid material or soil by wet classification, centrifugation, filtration, or similar solid-liquid separation processes. Removing or draining water from a riverbed, construction site, caisson, or mine shaft, by pumping or evaporation. This is often done during the site development phase of a major construction project due to a high water table. Usually involves the use of "dewatering" pumps. Methods of dewatering include; Wellpoint, Deep Well and Eductor systems. http://en.wikipedia.org/wiki/Dewatering | Default: |
| disposal Public | A well, often a depleted oil or gas well, into which waste fluids can be injected for safe disposal. Disposal wells typically are subject to regulatory requirements to avoid the contamination of freshwater aquifers. http://www.glossary.oilfield.slb.com/Display.cfm?Term=disposal%20well | Default: |
| flowingShot Public | A flowing shot hole is a drilled (seismic) hole that has entered an underground water source that has sufficient pressure to cause the hole to "overflow". http://www.etsurvey.com/water/h20main.htm | Default: |
| geotechnical Public | A geotechnical well is defined as a hole drilled for the exclusive purpose of collecting geotechnical data, including soil samples, vapour samples, and water samples obtained through bailing, driven sampler or other similar methods. http://www.adwr.state.az.us/dwr/Content/Find_by_Category/Laws_and_Rules/files/SPS/Well%20Construction/Well%20Construction%20and%20LIcensing%20-%20WL7.pdf | Default: |
| mineral Public | A non-E&P well drilled for the purpose of locating and/or extracting a mineral from the subsurface, usually through the injection and/or extraction of mineral-bearing fluids. http://posc.org/technical/reference/POSC_well_purpose.html Mineral test hole- any hole in excess of one hundred (100) feet drilled during the exploration for minerals but shall exclude auger drilling in surficial or otherwise unconsolidated material, drilling in conjunction with mining or quarrying operations, and drill holes for the exploration of oil and/or gas, water, structural foundations, and seismic surveys. http://www.tennessee.gov/sos/rules/0950/0950-01-01.pdf | Default: |
| monitoringLevelHead Public | Monitoring well: A non-pumping well, generally of small diameter, that is used to measure the elevation of a water table or water quality. A piezometer, which is open only at the top and bottom of its casing, is one type of monitoring well. http://www.groundwater.org/gi/gwglossary.html#M monitoring wells or piezometers, are often smaller diameter wells used to Monitor the hydraulic head or sample the groundwater for chemical constituents. Piezometers are monitoring wells completed over a very short section of aquifer. Monitoring wells can also be completed at multiple levels, allowing discrete samples or measurements to be made at different vertical elevations at the same map location. http://en.wikipedia.org/wiki/Monitoring_well | Default: |
| monitoringQuality Public | Most monitoring wells constructed today are used to assess the nature and distribution of pollutants and contaminants in groundwater; The nature and distribution of naturally occurring chemical constituents; Subsurface hydrologic conditions; and, hydraulic properties of strata as they relate to pollutant and contaminant movement. http://www.dpla.water.ca.gov/sd/groundwater/california_well_standards/mws/mws_history.html | Default: |
| oil Public | An oil well is a term for any perforation through the Earth's surface designed to find and release both petroleum oil and gas hydrocarbons. http://en.wikipedia.org/wiki/Oil_well | Default: |
| oilExploratory Public | A exploratory well drilled in an unproved area to test for a new field, a new pay, a deeper reservoir, or a shallower reservoir. Also known as an exploration well. http://posc.org/technical/reference/POSC_well_purpose.html Exploratory Well: A well drilled with a high degree of risk to: A) search for a new reservoir of oil or gas, also known as a Wildcat, B) extend the parameters of an existing field known as a Step Out, C) to prove another zone within an already producing field. http://www.vastenergy.com/definitions.htm | Default: |
| other Public | Any other usage of a well. | Default: |
| recharge Public | a- Aquifer Recharge Wells (5R21) Used to recharge depleted aquifers and may inject fluids from a variety of sources such as lakes, streams, domestic wastewater treatment plants, other aquifers, etc. b- Saline Water Intrusion Barrier Wells (5B22) Used to inject water into fresh water aquifers to prevent intrusion of salt water into fresh water aquifers. Used in highly populated areas. c- Subsidence Control Wells (5S23) Used to inject fluids into a non-oil or gas-producing zone to reduce or eliminate subsidence associated with overdraft of fresh water and not used for the purpose of oil or natural gas production. a, b, c - http://www.epa.gov/Region2/water/compliance/wellclasstypetable_inventoryc_form.pdf | Default: |
| seismic Public | A well used to conduct seismic surveys. | Default: |
| waterExploratory Public | A well drilled to seach for new groundwater | Default: |
| waterSupply Public | Well used to supply water for various usage. | Default: |
| withdrawal Public | TBD | Default: |
| termal Public |  | Default: |

##### WellStatusCode

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2006-01-24.  Last modified on 2011-04-12.

GUID:		{6B4AD8BC-E50A-433c-942B-C03FB5DCB60E}

-- Definition --

(from GWML) Status of the well.

-- Note --

Class adapted from GWML.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | asDictionary = true. Values: true Default: true Description: Encode code list as externally managed dictionary (applies to ISO 19136:2007 encoding rule). Always true in INSPIRE. |
|  | codeSpace = . Description: URI that identifies the authority for this dictionary and its members |
|  | dictionaryIdentifier = . Description: URI that identifies this dictionary |
|  | memberIdentifierStem = . Description: URI stem used for dictionary members |
|  | xsdEncodingRule = iso19136_2007. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007 Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public Well | Public WellStatusCode |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| abandonedDry Public |  | Default: |
| abandonedInsufficient Public |  | Default: |
| abandonedQuality Public |  | Default: |
| deepened Public |  | Default: |
| new Public |  | Default: |
| notInUse Public |  | Default: |
| reconditionned Public |  | Default: |
| standby Public |  | Default: |
| unfinished Public |  | Default: |
| unknown Public |  | Default: |
| abandoned Public |  | Default: |

##### WellUse

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_CodeListsAndEnumerations     Keywords:

Detail:		Created on 2007-08-03.  Last modified on 2011-04-11.

GUID:		{6C898CC8-BC28-4d2c-BD24-76F7494D8A47}

-- Definition --

A use of well (production or pumping wells, monitoring wells, piezometers).

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | asDictionary = true. Values: true Default: true Description: Encode code list as externally managed dictionary (applies to ISO 19136:2007 encoding rule). Always true in INSPIRE. |
|  | codeSpace = . Description: URI that identifies the authority for this dictionary and its members |
|  | dictionaryIdentifier = . Description: URI that identifies this dictionary |
|  | memberIdentifierStem = . Description: URI stem used for dictionary members |
|  | xsdEncodingRule = iso19136_2007. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007 Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public Well | Public WellUse |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| observation Public |  | Default: [sequenceNumber = 10 ] |
| monitoring Public |  | Default: [sequenceNumber = 20 ] |
| piezometry Public |  | Default: [sequenceNumber = 30 ] |
| domestic Public |  | Default: [sequenceNumber = 40 ] |
| exploitation Public |  | Default: [sequenceNumber = 50 ] |
| production Public |  | Default: [sequenceNumber = 60 ] |

#### HydrogeologyCore_GenericAndExternalTypes

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_Core

Detail:		Created on 2011-03-09. Last modified on 2011-04-11

GUID:		{877435E6-EC49-430f-A4E4-4B3CEF15CF31}

##### AquiferDescription

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2007-05-30.  Last modified on 2011-03-12.

GUID:		{35243708-6D87-4199-8783-D80019694CE3}

-- Definition --

(From GWML) Properties of the rock (forming aquifer) that is relevant to the groundwater.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = true. Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xmlSchemaType = . Description: no type shall be generated for the class, but the given type or element referenced |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation DescribesParamitersOf Source -> Destination | Public AquiferDescription | Public Aquitard |  |
| Aggregation Source -> Destination | Public AquiferDescription | Public ProfileInterval |  |
| Aggregation DescribesParametersOf Source -> Destination | Public AquiferDescription | Public Aquifer |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| descriptoin PT_FreeText Public «voidable» |  | Default: |
| capacityCoefficient CGI_NumericRange Public «voidable - estimatedProperty» | Volumetric capacity of an aquifer to hold water. | Default: [sequenceNumber = 10 ] |
| hydraulicConductivityCoefficient CGI_NumericRange Public «voidable - estimatedProperty» | Hydraulic conductivity can be measured by applying Darcy's law on the material. Such experiments can be conducted by creating a hydraulic gradient between two points, and measuring the flow rate (Oosterbaan and Nijland[1]). (Wikipedia) | Default: [sequenceNumber = 20 ] |
| storativityCoefficient CGI_NumericRange Public «voidable - estimatedProperty» | Storativity is the volume of water released from storage per unit decline in hydraulic head in the aquifer, per unit area of the aquifer. http://en.wikipedia.org/wiki/Specific_storage | Default: [sequenceNumber = 30 ] |
| beginLifespanObject DateTime Public «lifeCycleInfo - voidable» |  | Default: |
| endLifespanObject DateTime Public [0..1] «lifeCycleInfo - voidable» |  | Default: |

##### FeatureUniqueCodeType

Type:		Class    string

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-04-13.

GUID:		{C26DCED7-0DE2-40c5-8934-4E65840DCD71}

Type for unique code within the Member State for the feature

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | derivation = restriction. |
|  | maxLength = 40. |
|  | minLength = 1. |
|  | whiteSpace = collapse. |

##### FeatureUniqueEUCodeType

Type:		Class    string

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-04-13.

GUID:		{892FBC1D-4045-4062-BE92-5D025AC97A3F}

Type for EU code, allows up to 42 characters (unique code within MS prefixed with country ISO 2 character code

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | derivation = restriction. |
|  | maxLength = 42. |
|  | minLength = 3. |

##### GEh_ComplexGeometry

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2011-02-17.  Last modified on 2011-03-09.

GUID:		{E29D9CD1-E8F5-484d-8E39-7684234ED1AC}

-- Definition --

Description of hydrogeological feature for hydrogeological modeling needs.

-- Note --

Horizontal extend of hydrogeological feature is expressed by GM_SurfaceBoundary.

Vertical extend diferent in diferent places is limited by floor and roof surfaces.

Surfaces are expressed in CV_ContinousQuadrilatereaGridCoverage form.

| Constraints |  |
| --- | --- |
|  | horizontal extent of floor and roof is limited to value of horizonalExtent attribute: (Invariant, Status is Approved) |
|  | one (floor or roof) must be present: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public GEh_Geometry | Public GEh_ComplexGeometry |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| floor CV_ContinuousQuadrilateralGridCoverage Public «voidable - featureType» |  | Default: |
| horizontalExtend GM_SurfaceBoundary Public |  | Default: |
| roof CV_ContinuousQuadrilateralGridCoverage Public «voidable - featureType» |  | Default: |

##### GEh_Geometry

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2011-02-17.  Last modified on 2011-04-11.

GUID:		{A6037110-1372-4a78-968D-38D754C002C8}

-- Definition --

Common data union for geometry of feature.

-- Note --

In GML application it is realised by choice element.

XML Schema choice element allows only one of the elements contained in the <choice> declaration to be present within the containing element.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public GEh_Geometry | Public GEh_ComplexGeometry |  |
| Dependency Source -> Destination | Public GEh_GeometryType | Public GEh_Geometry |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| complexGeometry GEh_ComplexGeometry Public |  | Default: |
| simpleGeometry GM_Primitive Public |  | Default: |

##### GEh_GeometryType

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2011-03-06.  Last modified on 2011-03-09.

GUID:		{2C0CFC7B-664E-4811-9DE7-F24FC9207F0D}

-- Definition --

Container for union of different types of geometriec

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public GEh_GeometryType | Public GEh_Geometry |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| geometry GEh_Geometry Public «union» | geometry | Default: |

##### PiezometricStateOfGroundwater

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2011-04-12.  Last modified on 2011-04-12.

GUID:		{E87F6D39-3B93-4519-B2BF-C9241B61C19F}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public StateOfGroundwater | Public PiezometricStateOfGroundwater |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| descriptionOfPiezometricState PT_FreeText Public |  | Default: |
| piezometricSurfaceGrid CV_ContinuousQuadrilateralCoverage Public «featureType» |  | Default: |
| piezometricSurfaceCountours CV_SegmentedCurveCoverage Public «featureType» |  | Default: |

##### StateOfGroundwater

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2011-04-12.  Last modified on 2011-04-12.

GUID:		{4FB02906-2AE1-4f45-A036-CADB0FC8DB31}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public StateOfGroundwater | Public PiezometricStateOfGroundwater |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| piezometricState PiezometricStateOfGroundwater Public «union» |  | Default: |

##### WaterPropertyDescription

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2005-11-28.  Last modified on 2011-03-13.

GUID:		{A86A8666-8967-4288-AA79-6318148F8EC0}

-- Definition --

(from GWML)

Water Property description are the properties of the HydrogeologicUnit regarding its water content.  Two aspects are considered, First the quantity (the amount of water that is present, as opposed to the amount of water that can potentially be there) and second, the quality of that water.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = true. Values: true Default: true Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public WaterYieldDescription | Public WaterPropertyDescription |  |
| Generalization Source -> Destination | Public WaterYieldDescription | Public WaterPropertyDescription |  |
| Generalization Source -> Destination | Public WaterQuantityDescription | Public WaterPropertyDescription |  |
| Generalization Source -> Destination | Public WaterQualityDescription | Public WaterPropertyDescription |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| beginLifespanVersion DateTime Public «lifeCycleInfo - voidable» | Time instant or period at which this value has any validity. NOTE: the time at which this observation was made is carried by O&M | Default: [sequenceNumber = 20 ] |
| endLifespanVersion DateTime Public [0..1] «lifeCycleInfo - voidable» |  | Default: |

##### WaterQualityDescription

Type:		Class    WaterPropertyDescription

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2005-11-28.  Last modified on 2011-03-13.

GUID:		{2DEB5464-82B1-4704-800B-6E967FF3C216}

-- Definition --

(from GWML)

A list of common properties related to water quality assessment.  Head of a substitution list that include Qualitative and Quantitative water quality properties

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = true. Values: true Default: true Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public WaterQualityDescription | Public ProfileInterval |  |
| Generalization Source -> Destination | Public WaterQualityDescription | Public WaterPropertyDescription |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| normComplianceResult GEh_ValueType Public «voidable» | Evaluation of the compliance of this result compared to the norm (eg, below the norm, above the norm, meeting, exceeding..) | Default: [sequenceNumber = 10 ] |
| normDocumentation localizedString Public «voidable» | Identification of a norm (if any) that is should be considered to interpret the result. | Default: [sequenceNumber = 20 ] |

##### WaterQuantityDescription

Type:		Class    WaterPropertyDescription

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2005-11-28.  Last modified on 2011-03-13.

GUID:		{F42DBAB5-4493-4605-BED5-8B4D02732913}

-- Definition --

(from GWML)

A class used to describe the the quantity of water that is present in an aquifer.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = true. Values: true Default: true Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public WaterQuantityDescription | Public ProfileInterval |  |
| Generalization Source -> Destination | Public WaterQuantityDescription | Public WaterPropertyDescription |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| qualitativeWaterQuantity GEh_ValueType Public «voidable» | Qualitative description of the water quantity (such as 'wet' or 'dry'). Often the only information recorded in some water well records | Default: [sequenceNumber = 10 ] |

##### WaterYieldDescription

Type:		Class    WaterPropertyDescription

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes     Keywords:

Detail:		Created on 2008-02-05.  Last modified on 2011-03-09.

GUID:		{D21D6A96-626B-48b2-B87F-7CA11D246FD4}

-- Definition --

(from GWML) A class used to describe the different yield parameters of an aquifer.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = true. Values: true Default: true Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public WaterYieldDescription | Public ProfileInterval |  |
| Generalization Source -> Destination | Public WaterYieldDescription | Public WaterPropertyDescription |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| safeYield CGI_NumericRange Public «voidable - estimatedProperty» | -- Definition -- (from GWML) With reference to either a surface- or ground-water supply, the rate of diversion or extraction for Consumptive Use which can be maintained indefinitely, within the limits of economic feasibility, under specified conditions of water-supply development. Also see Perennial Yield. Yield, Perennial ? The amount of usable water of a ground-water reservoir that can be economically withdrawn and consumed each year for an indefinite period of time. It cannot exceed the sum of the Natural Recharge, the Artificial (or Induced) Recharge, and the Incidental Recharge without causing depletion of the groundwater reservoir. Also referred to as Safe Yield. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-y.pdf Amount of water that can be retreived from an aquifer without jeopardising the resources but still involved some impacts (def. to be improved) | Default: [sequenceNumber = 10 ] |
| specificYield CGI_NumericRange Public «voidable - estimatedProperty» | -- Definition -- (from GWML) Specific yield, also known as the drainable porosity, is a ratio, less than or equal to the effective porosity, indicating the volumetric fraction of the bulk aquifer volume that a given aquifer will yield when all the water is allowed to drain out of it under the forces of gravity (http://en.wikipedia.org/wiki/Specific_storage) | Default: [sequenceNumber = 20 ] |
| sustainableYield CGI_NumericRange Public «voidable - estimatedProperty» | --Definition -- (from GWML) The groundwater extraction regime, measured over a specified planning timeframe, that allows acceptable levels of stress and protects dependent economic, social, and environmental values. http://www.environment.gov.au/water/publications/environmental/groundwater/pubs/annex-a.pdf The sustainable yield of natural capital is the ecological yield that can be extracted without reducing the base of capital itself, i.e. the surplus required to maintain nature's services at the same or increasing level over time | Default: [sequenceNumber = 30 ] |

##### From_ISO19156

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	HydrogeologyCore_GenericAndExternalTypes

Detail:		Created on 2011-03-12. Last modified on 2011-03-12

GUID:		{DE370832-39DE-4479-8E3A-02625546B0B4}

###### Interval

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From_ISO19156     Keywords:

Detail:		Created on 2005-01-20.  Last modified on 2011-03-12.

GUID:		{966AAC2A-1954-480f-8651-8230CEA710EB}

-- Definition --

(from ISO 19156)

An Interval is a simple curve, parameterized only by its two end-points.

| Constraints |  |
| --- | --- |
|  | type(shape)=GM_CurveSegment: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: true ¦ false Default: false Description: enforce the 'by value' property pattern without xlinks |
|  | byValuePropertyType = false. Values: true,false Default: false Description: enforce the 'by value' property pattern without xl |
|  | hasXmlLang = false. Values: true ¦ false Default: false Description: axml attribute xml:lang shall be generated for the type representing the class |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | isCollection = false. Values: true,false Default: false Description: attribute group gml:AggregationAttributeGroup is a |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | noPropertyType = false. Values: true,false Default: false Description: suppress automatic creation of *PropertyType compl |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public Interval | Public Profile |  |

###### Measure

Type:		Class

Status:		Proposed.  Version .  Phase .

Package:	From_ISO19156     Keywords:

Detail:		Created on 2008-04-10.  Last modified on 2011-03-12.

GUID:		{CDBC49A7-ED7D-46fb-9196-C808226975F8}

-- Definition --

The result from performing the act or process of ascertaining the extent, dimensions, or quantity of some entity.

A measure is always associated to a unit of measure. Ratio measures where the two base measures are in the same units (often considered unit-less) should be associated to UnitOfMeasure (same meter/meter for map scale) so that conversions to non-unitless ratios can be accomplished (such as miles/inch).

-- Note --

(From ISO 19156 - Observations and Measurements)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | persistence = persistent. |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| value Number Public | The numerical value quantifying the extent or size of some quantity, in the units specified by the associated UnitOfMeasure class. | Default: |

###### ObservationWell

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From_ISO19156     Keywords:

Detail:		Created on 2006-07-18.  Last modified on 2011-03-12.

GUID:		{AC6964E5-3686-41e3-BDDF-EF3BA90D0185}

-- Definition --

(from ISO 19156) Wells, boreholes, drillholes provide access within a solid medium, and are used to sample rock-units, aquifers, etc. Instruments may be inserted into the hole for in-situ observations. Specimens may be retrieved from a well for observations ex-situ. The association between specimens and wells may be indicated using the SamplingFeatureRelation class.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: true ¦ false Default: false Description: enforce the 'by value' property pattern without xlinks |
|  | byValuePropertyType = false. Values: true,false Default: false Description: enforce the 'by value' property pattern without xl |
|  | hasXmlLang = false. Values: true ¦ false Default: false Description: axml attribute xml:lang shall be generated for the type representing the class |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | isCollection = false. Values: true,false Default: false Description: attribute group gml:AggregationAttributeGroup is a |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | noPropertyType = false. Values: true,false Default: false Description: suppress automatic creation of *PropertyType compl |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public Profile | Public ObservationWell |  |

###### Profile

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From_ISO19156     Keywords:

Detail:		Created on 2007-03-14.  Last modified on 2011-03-12.

GUID:		{8658B8BD-3819-4387-BE86-AB7AA94FCC68}

-- Definition --

A profilfe is a curve along which a continuously varying property is sampled, usually at a high spatial sampling rate with an automated instrument.

-- Note --

(From ISO 19156 - Observations and Measurements)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: true ¦ false Default: false Description: enforce the 'by value' property pattern without xlinks |
|  | byValuePropertyType = false. Values: true,false Default: false Description: enforce the 'by value' property pattern without xl |
|  | hasXmlLang = false. Values: true ¦ false Default: false Description: axml attribute xml:lang shall be generated for the type representing the class |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | isCollection = false. Values: true,false Default: false Description: attribute group gml:AggregationAttributeGroup is a |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | noPropertyType = false. Values: true,false Default: false Description: suppress automatic creation of *PropertyType compl |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public Profile | Public ObservationWell |  |
| Aggregation Source -> Destination | Public Interval | Public Profile |  |

###### SF_SamplingCurve

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From_ISO19156     Keywords:

Detail:		Created on 2011-03-06.  Last modified on 2011-03-12.

GUID:		{0ACF4E3C-6936-46b9-A241-0E5AC9434E9A}

-- Definition --

A common mode of sampling is along a curve.

The class SM_SamplingCurve (Figure 11) is an instance of the «metaclass» GF_FeatureType (ISO 19109), which therefore represents a feature type. SM_SamplingCurve shall support one attribute and one association.

-- Note --

From ISO 19156 - Observations and Measurements

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

###### SF_SamplingFeature

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From_ISO19156     Keywords:

Detail:		Created on 2004-12-13.  Last modified on 2011-03-13.

GUID:		{D93ED42F-BF3F-43d0-A1F3-A1BA679D2E44}

- Role of sampling features

Sampling features are artefacts of an observational strategy, and have no significant function outside of their role in the observation process. The physical characteristics of the features themselves are of little interest, except perhaps to the manager of a sampling campaign.

EXAMPLE    A “station” is essentially an identifiable locality where a sensor system or procedure may be deployed and an observation made. In the context of the observation model, it connotes the “world in the vicinity of the station”, so the observed properties relate to the physical medium at the station, and not to any physical artefact such as a mooring, buoy, benchmark, monument, well, etc.

NOTE    A transient sampling feature, such as a ships-track or flight-line, may be identified and described, but is unlikely to be revisited exactly.

A sampling feature is intended to sample some feature of interest in an application domain. However, in some cases the identity, and even the exact type, of the sampled feature may not be known when observations are made using the sampling features.

- Classification of sampling features

A small number of sampling patterns are common across disciplines in observational science. These provide a basis for processing and portrayal tools which are similar across domains, and depend primarily on the geometry of the sample design. Common names for sampling features include specimen, station, profile, transect, path, swath, and scene. These are classified primarily by their topological dimension. Point-located samples are defined in this Clause, extensive samples of various dimensions in Clause 9 and specimens in Clause 10.

| Constraints |  |
| --- | --- |
|  | not sampledFeature->forAll(oclIsKindOf(SF_SamplingFeature)): (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: true,false Default: false Description: enforce the 'by value' property pattern without xl |
|  | byValuePropertyType = false. Values: true ¦ false Default: false Description: enforce the 'by value' property pattern without xlinks |
|  | hasXmlLang = false. Values: true ¦ false Default: false Description: axml attribute xml:lang shall be generated for the type representing the class |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | isCollection = false. Values: true,false Default: false Description: attribute group gml:AggregationAttributeGroup is a |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | noPropertyType = false. Values: true,false Default: false Description: suppress automatic creation of *PropertyType compl |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| parameter NamedValue Public [0..*] | If present, the attributes parameter:NamedValue shall describe an arbitrary parameter associated with the SF_SamplingFeature. This might be a parameter that qualifies the interaction with the sampled feature, or an environmental parameter associated with the sampling process. | Default: [sequenceNumber = 5 ] |
| lineage LI_Lineage Public [0..1] | If present, the attribute lineage:LI_Lineage shall describe the history and provenance of the SF_SamplingFeature. This might include information relating to the handling of the specimen, or details of the survey procedure of a spatial sampling feature. | Default: [sequenceNumber = 2 ] |

###### Station

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From_ISO19156     Keywords:

Detail:		Created on 2007-03-07.  Last modified on 2011-03-12.

GUID:		{ADA10F18-8C87-4797-A4CE-9EE575CCCCDE}

-- Definition --

A station is an identified point location where observations are made. In many cases the station will be continuously monitored or revisted.

-- Note --

From ISO 19156 - Observations and Measurements

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: true ¦ false Default: false Description: enforce the 'by value' property pattern without xlinks |
|  | byValuePropertyType = false. Values: true,false Default: false Description: enforce the 'by value' property pattern without xl |
|  | hasXmlLang = false. Values: true ¦ false Default: false Description: axml attribute xml:lang shall be generated for the type representing the class |
|  | isCollection = false. Values: true,false Default: false Description: attribute group gml:AggregationAttributeGroup is a |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | noPropertyType = false. Values: true,false Default: false Description: suppress automatic creation of *PropertyType compl |

### Hydrogeology_Extensions

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_v.0.4

Detail:		Created on 2011-04-12. Last modified on 2011-04-12

GUID:		{5D7F4E2A-0E71-4997-80F6-0694215845ED}

Hydrogeology_Extensions - (Package diagram)

Created By:	Janusz Michalak on 2011-03-14

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{1153E480-4A05-45e1-983F-1F3D140F6C7B}

Figure: 9

#### <anonymous>

Type:		Text

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Extensions     Keywords:

Detail:		Created on 2007-07-19.  Last modified on 2007-07-19.

GUID:		{52C58360-28AA-4fe8-9CBF-023E8F0FFBD7}

#### Hydrogeology_AquiferSystem

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_Extensions

Detail:		Created on 2011-04-11. Last modified on 2011-04-12

GUID:		{DA3B087D-7D75-42f8-BE64-ED0F283872AF}

Hydrogeology_AquiferSystem - (Logical diagram)

Created By:	Tomasz Nałęcz and Janusz Michalak on 2011-04-11

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{58B2551F-B645-4f2e-AA48-1C0742E0E7FB}

Figure: 10

##### CompoudAquiferSystem

Type:		Class    AquiferSystem

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_AquiferSystem     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-04-13.

GUID:		{3466465C-DBEB-40ac-9F04-5B1856BF919B}

-- Definition --

The set of two or more AquiferSystem belonging to different categories. For example:

- porous and fractured,

- porous and karst

- fractured and karstic

or

- porous, fractured, and karstic.

-- Note --

Group of features Aquifer type create one connected hydrodynamicaly system.

The features the Aquifer type contains have common recharge area and elements of drainage system

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public isContained PorousAquiferSystem | Public contains CompoudAquiferSystem |  |
| Aggregation Source -> Destination | Public isContained FracturedAquiferSystem | Public contains CompoudAquiferSystem |  |
| Generalization Source -> Destination | Public CompoudAquiferSystem | Public AquiferSystem |  |
| Aggregation Source -> Destination | Public isContained KarsticAquiferSystem | Public contains CompoudAquiferSystem |  |

##### FracturedAquiferSystem

Type:		Class    AquiferSystem

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_AquiferSystem     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-04-13.

GUID:		{4A28C7F2-FBDA-44da-A07C-FAC450631FC3}

-- Definition --

Group of features FracturedAquifer type create one connected hydrodynamicaly system.

-- Note --

The features the FracturedAquifer type contains have common recharge area and elements of drainage system (rivers, sources and others)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public isContained FracturedAquiferSystem | Public contains CompoudAquiferSystem |  |
| Generalization Source -> Destination | Public FracturedAquiferSystem | Public AquiferSystem |  |
| Dependency Source -> Destination | Public FracturedAquiferSystem | Public OriginOfFractures |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| genesis OriginOfFractures Public [1..*] «voidable» |  | Default: |

##### KarsticAquiferSystem

Type:		Class    AquiferSystem

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_AquiferSystem     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-04-13.

GUID:		{656C7058-A57B-40fa-9831-9D3E335F36BC}

-- Definition --

The group of feature KarsticAquifer types forming a hydrodynamically connected to the system.

--Note --

The features the KarsticAquifer type contains have common recharge area and elements of drainage system (rivers, sources and others)

These features can be in:

- saturated zone and are then completely filled with water.

or

- in aeration zone and are only partially filled with water.

The degree of filling with water may be seasonal, from the total fulfillment to the conditions when there is a completely lack of water.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public KarsticAquifer | Public KarsticAquiferSystem |  |
| Aggregation Source -> Destination | Public SwallowHole | Public KarsticAquiferSystem |  |
| Aggregation Source -> Destination | Public isContained KarsticAquiferSystem | Public contains CompoudAquiferSystem |  |
| Dependency Destination -> Source | Public SoluteRockType | Public KarsticAquiferSystem |  |
| Aggregation Source -> Destination | Public KarsticSpring | Public KarsticAquiferSystem |  |
| Generalization Source -> Destination | Public KarsticAquiferSystem | Public AquiferSystem |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| soluteRock SoluteRockType Public [1..*] «voidable» |  | Default: |

##### OriginOfFractures

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_AquiferSystem     Keywords:

Detail:		Created on 2011-03-05.  Last modified on 2011-04-12.

GUID:		{C92872C2-EC99-4fad-A2BD-EA58BCE48E62}

-- Definition --

Genesis of cracks forming FracturedSystem, for example:

- related to the fault zone,

- caused by weathering processes,

- other processes.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public FracturedAquiferSystem | Public OriginOfFractures |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| zoneOfShearDisplacementStructure Public «enum» |  | Default: |
| zoneOfWeathering Public «enum» |  | Default: |
| other Public «enum» |  | Default: |

##### PorousAquiferSystem

Type:		Class    AquiferSystem

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_AquiferSystem     Keywords:

Detail:		Created on 2011-01-28.  Last modified on 2011-04-13.

GUID:		{E623A403-8D58-4088-9454-C435F5B3FDA4}

-- Definition --

Feature collection PorousAquifer and PorousAquitard type (often referred to HydrostratigraficSystem - GWML). Borders of PorousSystem create features of Aquiclude type and surface area. It may also consist of only one PorousAquifer.

-- Note --

Frequentally PorousSystem has a considerable horizontal range and vertical layered structure.

Group of features PorousAquifer type create one connected hydrodynamicaly system.

The features consist PorousAquifer type have common recharge area and elements of drainage system (rivers, sources and others)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public PorousAquiferSystem | Public AquiferSystem |  |
| Aggregation Source -> Destination | Public isContained PorousAquiferSystem | Public contains CompoudAquiferSystem |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| isHydrostratigraphic boolean Public «voidable» |  | Default: |
| hasLargeHorizontalExtend boolean Public «voidable» |  | Default: |

#### Hydrogeology_DrinkingGroundWaterProtection

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_Extensions

Detail:		Created on 2011-03-11. Last modified on 2011-03-12

GUID:		{5EA249A3-E105-44f8-959B-B1CC522E6E4D}

Hydrogeology_DrinkingGroundWaterProtection - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-09

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{86E63028-935C-4abc-A727-83A99132CEBF}

Figure: 11

##### ConeOfDepression

Type:		Class    HydrogeologicalFeature, HydrogeologicalZone

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_DrinkingGroundWaterProtection     Keywords:

Detail:		Created on 2007-05-31.  Last modified on 2011-04-11.

GUID:		{D8B51A2B-AA5B-4c9d-853D-0D7FD8E40693}

-- Definition --

(from GWML) A cone of depression occurs in an aquifer when ground water is pumped from a well. In an unconfined (water table) aquifer, this is an actual depression of the water levels. In confined (artesian) aquifers, the cone of depression is a reduction in the pressure head surrounding the pumped well (please see earlier articles for discussion of head).

http://en.wikipedia.org/wiki/Cone_of_depression

A depression in the groundwater table or potentiometric surface that has the shape of an inverted cone and develops around a well from which water is being withdrawn. Its trace (perimeter) on the land surface defines the zone of influence of a well. Also called pumping cone and cone of drawdown. (http://www.wrds.uwyo.edu/wrds/deq/whp/whpgloss.html)

he zone around a well in an unconfined aquifer that is normally saturated, but becomes unsaturated as a well is pumped, leaving an area where the water table dips down to form a cone shape.  The shape of the cone is influenced by porosity and the water yield or pumping rate of the well.  The land surface overlying the cone of depression is referred to as the area of influence.

http://www.groundwater.org/gi/gwglossary.html#C

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: true ¦ false Default: false Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public ConeOfDepression | Public HydrogeologicalZone |  |
| Generalization Source -> Destination | Public ConeOfDepression | Public HydrogeologicalFeature |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| centeredAt GM_Point Public | The planimetric localization of the center of the cone of depression. The planimetric shape of a cone of depression os not necessarily circular. This discussion details size and shape characteristic of a cone of depression. The size and shape (slope) of the cone of depression depends on many factors. The pumping rate in the well will affect the size of the cone. Also, the type of aquifer material, such as whether the aquifer is gravel, sand, silt, fractured rocks, karst, etc., also will affect how far the cone extends. The amount of water in storage and the thickness of the aquifer also will determine the size and shape of the cone of depression. As a well is pumped, the cone of depression will extend out and will continue to expand in a radial fashion until a point of equilibrium occurs. This usually is when the amount of water released from storage equals the rate of pumping. This also can occur when recharge to the aquifer equals the amount of water being pumped. We typically think of a cone of depression as being a circular feature surrounding the pumped well. However, aquifer characteristics can affect the shape of the cone of depression. For example, if there is a steep ground-water gradient in the area of pumpage, the cone will tend to be shorter in the upgradient direction and elongated in the downgradient direction. This is because the water is already flowing towards the well from the upgradient direction, so the cone of depression doesn't need to extend as far out to obtain water, whereas the water is flowing away from the well in the downgradient direction, so the cone of depression needs to reach further to obtain water. http://en.wikipedia.org/wiki/Cone_of_depression | Default: [sequenceNumber = 10 ] |
| depth CGI_NumericValue Public | The depth of a cone of depression in an aquifer. | Default: [sequenceNumber = 20 ] |

##### GroundWaterWellProtectionType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_DrinkingGroundWaterProtection     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-04-12.

GUID:		{BB089A28-28A5-4aa0-91B8-1704498D95A8}

-- Definition --

Types of protected areas around the groundwater intake (WFD_register, MS_direct,MS_ intirect).

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public ProtectionZone | Public GroundWaterWellProtectionType |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| MS_indirect Public «enum» |  | Default: |
| MS_direct Public «enum» |  | Default: |
| WFD_register Public «enum» |  | Default: |
| other Public «enum» |  | Default: |

##### HydrogeologicalZone

Type:		Class    HydrogeologicalFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_DrinkingGroundWaterProtection     Keywords:

Detail:		Created on 2011-04-12.  Last modified on 2011-04-12.

GUID:		{3A5CF07C-A9D2-4f5c-BB00-FFFB653CABE1}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public ZoneOfInfluence | Public HydrogeologicalZone |  |
| Generalization Source -> Destination | Public ConeOfDepression | Public HydrogeologicalZone |  |
| Generalization Source -> Destination | Public ProtectionZone | Public HydrogeologicalZone |  |
| Generalization Source -> Destination | Public HydrogeologicalZone | Public HydrogeologicalFeature |  |
| Association Bi-Directional | Public isProtectedBy ActiveWell (or AbstractionWell) | Public protects HydrogeologicalZone |  |
| Generalization Source -> Destination | Public ZoneOfContribution | Public HydrogeologicalZone |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| beginLifespanObject DateTime Public «lifeCycleInfo - voidable» |  | Default: |
| endLifespanObject DateTime Public [0..1] «lifeCycleInfo - voidable» |  | Default: |
| validFrom DateTime Public «voidable» |  | Default: |
| validTo DateTime Public [0..1] «voidable» |  | Default: |

##### ProtectionZone

Type:		Class    HydrogeologicalZone

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_DrinkingGroundWaterProtection     Keywords:

Detail:		Created on 2007-07-19.  Last modified on 2011-03-12.

GUID:		{2FA72268-9B2A-4dec-BF75-3481459BE175}

-- Definition --

(from GWML)

An area that defines a protection zone around a well.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: true ¦ false Default: false Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public ProtectionZone | Public GroundWaterWellProtectionType |  |
| Generalization Source -> Destination | Public ProtectionZone | Public HydrogeologicalZone |  |
| Association LinkForNavigation Source -> Destination | Public ProtectionZone | Public AMArea | Invariant {if ( AMArea.amAreaType == restrictedAreaAroundDrinkingWaterSource && GroundWaterIntake.waterUsage == forDrinking)} |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| typeOfProtection GroundWaterWellProtectionType Public |  | Default: |

##### ZoneOfContribution

Type:		Class    HydrogeologicalZone

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_DrinkingGroundWaterProtection     Keywords:

Detail:		Created on 2011-03-01.  Last modified on 2011-03-12.

GUID:		{28F5DCC9-44C7-4dfc-AEC6-6AFA38384595}

-- Definition --

(from GWML) The area surrounding a pumping well, spring, or tunnel that encompasses all areas and features that supply groundwater to the well spring, or tunnel. (http://www.wrds.uwyo.edu/wrds/deq/whp/whpgloss.html)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public ZoneOfContribution | Public HydrogeologicalZone |  |

##### ZoneOfInfluence

Type:		Class    HydrogeologicalZone

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_DrinkingGroundWaterProtection     Keywords:

Detail:		Created on 2007-05-31.  Last modified on 2011-03-12.

GUID:		{03082523-65DE-490a-A901-4AB68F8BB313}

-- Definition --

(from GWML)

The area of influence, or cone of depression formed when pumping a well or well field. (http://www.wrds.uwyo.edu/wrds/deq/whp/whpgloss.html)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: true ¦ false Default: false Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public ZoneOfInfluence | Public HydrogeologicalZone |  |

#### Hydrogeology_KarsticSystem

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_Extensions

Detail:		Created on 2011-03-11. Last modified on 2011-03-12

GUID:		{1ACB4296-70F2-4655-B575-942998129B45}

HydrogeologyExtension_KarsticSystem - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-06

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{84AADA10-479B-48f5-BFD3-93D779931A6E}

Figure: 12

##### Cave

Type:		Class    KarsticAquifer

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_KarsticSystem     Keywords:

Detail:		Created on 2011-02-20.  Last modified on 2011-03-15.

GUID:		{10EB0C28-B9E5-440f-BFF2-4C5140D0B108}

-- Definition --

An underground hollow with access from the ground surface or from the sea, often found in limestone areas and on rocky coastlines. (Earth Sciences / Physical Geography)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public KarsticStream | Public Cave |  |
| Generalization Source -> Destination | Public Cave | Public KarsticAquifer |  |

##### KarsticAquifer

Type:		Class    Aquifer

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_KarsticSystem     Keywords:

Detail:		Created on 2011-02-27.  Last modified on 2011-03-15.

GUID:		{6E650507-21E4-412d-91D6-6EEC70219A7B}

-- Definition --

A body of soluble rock that conducts water principally via enhanced (conduit or tertiary) porosity formed by the dissolution of the rock. The aquifers are commonly structured as a branching network of tributary conduits, which connect together to drain a groundwater basin and discharge to a perennial spring.

(Technical Definition of Karst Aquifer - Worthington, 2002)

-- Note --

Must produce useful quantities of water to a spring or well to qualify as an aquifer.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public KarsticAquifer | Public KarsticSpring |  |
| Generalization Source -> Destination | Public KarsticChannel | Public KarsticAquifer |  |
| Generalization Source -> Destination | Public KarsticAquifer | Public Aquifer |  |
| Aggregation Source -> Destination | Public KarsticAquifer | Public KarsticAquiferSystem |  |
| Generalization Source -> Destination | Public Cave | Public KarsticAquifer |  |

##### KarsticChannel

Type:		Class    KarsticAquifer

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_KarsticSystem     Keywords:

Detail:		Created on 2011-02-25.  Last modified on 2011-03-15.

GUID:		{57A2DCB0-D99F-429d-BDE5-DF23E0301BCA}

-- Definition --

The karstic channels are regarded as a separate flow system, having the volume of water Vc with the volumetric flow rate Qc(t), mainly supplied from sinkholes.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public KarsticChannel | Public KarsticAquifer |  |
| Association Bi-Directional | Public KarsticStream | Public KarsticChannel |  |

##### KarsticGroundWaterBody

Type:		Class    GroundWaterBody

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_KarsticSystem     Keywords:

Detail:		Created on 2011-02-17.  Last modified on 2011-03-15.

GUID:		{48174351-5940-464c-B9B5-98BCE663D411}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public KarsticGroundWaterBody | Public GroundWaterBody |  |
| Generalization Source -> Destination | Public KarsticStream | Public KarsticGroundWaterBody |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| persistance WaterPersistenceValue Public «voidable - estimatedProperty» |  | Default: |

##### KarsticSpring

Type:		Class    Spring

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_KarsticSystem     Keywords:

Detail:		Created on 2011-02-17.  Last modified on 2011-03-14.

GUID:		{69235633-CEBB-44fc-B3B3-79EF9AB2815E}

-- Definition --

Spring discharging karstic system

| Constraints |  |
| --- | --- |
|  | if springUsageOrType == karstic: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association LinkForNavigation Source -> Destination | Public KarsticSpring | Public SpringOrSeep |  |
| Association Bi-Directional | Public KarsticAquifer | Public KarsticSpring |  |
| Generalization Source -> Destination | Public KarsticSpring | Public Spring |  |
| Aggregation Source -> Destination | Public KarsticSpring | Public KarsticAquiferSystem |  |

##### KarsticStream

Type:		Class    KarsticGroundWaterBody

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_KarsticSystem     Keywords:

Detail:		Created on 2011-02-26.  Last modified on 2011-03-15.

GUID:		{15397967-9F59-4dac-B055-5E66642A4596}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public KarsticStream | Public KarsticChannel |  |
| Generalization Source -> Destination | Public KarsticStream | Public KarsticGroundWaterBody |  |
| Association Bi-Directional | Public KarsticStream | Public Cave |  |

##### SoluteRockType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_KarsticSystem     Keywords:

Detail:		Created on 2011-03-05.  Last modified on 2011-04-12.

GUID:		{6FD6C368-34E5-42a6-AA77-07A17C1AD9FD}

-- Definition --

Determination of the chemical composition of rocks creating karst processes and, consequently, the nature of the voids that collect water and allow it to flow.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Destination -> Source | Public SoluteRockType | Public KarsticAquiferSystem |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| limstone Public «enum» |  | Default: |
| dolomite Public «enum» |  | Default: |
| gypsum Public «enum» |  | Default: |
| other Public «enum» |  | Default: |

#### Hydrogeology_MineralAndThermalWaters

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_Extensions

Detail:		Created on 2011-03-11. Last modified on 2011-03-14

GUID:		{E2DE4A3E-2380-40af-AB28-77B27DD0979A}

HydrogeologyExtensions_MineralAndThermalWaters - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-10

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{183A3E82-28E9-444e-8AC3-9B607331CC24}

Figure: 13

##### Fresh-SaltClassCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-04-13.  Last modified on 2011-04-13.

GUID:		{8747DCB3-B2AD-4c83-991C-1BEABE68F6D2}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public MineralWaterDescription | Public Fresh-SaltClassCode |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| veryOligohaline Public «enum» |  | Default: |
| oligohaline Public «enum» |  | Default: |
| fresh Public «enum» |  | Default: |
| freshBrackish Public «enum» |  | Default: |
| brackish Public «enum» |  | Default: |
| brackishSalt Public «enum» |  | Default: |

##### HardnessCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-04-13.  Last modified on 2011-04-13.

GUID:		{746FC7F6-EFF8-4960-8BE7-93133B75EBEC}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public MineralWaterDescription | Public HardnessCode |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| verySoft Public «enum» |  | Default: |
| soft Public «enum» |  | Default: |
| moderatelyHard Public «enum» |  | Default: |
| hard Public «enum» |  | Default: |
| veryHard Public «enum» |  | Default: |
| extremelyHard Public «enum» |  | Default: |

##### MineralConcentration

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-04-13.  Last modified on 2011-04-13.

GUID:		{8C4FCB63-2D4C-4e6d-BAC1-209EC4CA27DE}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public MineralWaterDescription | Public MineralConcentration |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| veryLow Public «enum» |  | Default: |
| low Public «enum» |  | Default: |
| intermediate Public «enum» |  | Default: |
| high Public «enum» |  | Default: |

##### MineralContent

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-04-13.  Last modified on 2011-04-13.

GUID:		{D6D90F8E-C3F2-49d9-A3A2-848230F0A810}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public MineralWaterDescription | Public MineralContent |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| bicarbonate Public «enum» |  | Default: |
| sulphate Public «enum» |  | Default: |
| chloride Public «enum» |  | Default: |
| calcium Public «enum» |  | Default: |
| magnesium Public «enum» |  | Default: |
| fluoride Public «enum» |  | Default: |
| iron Public «enum» |  | Default: |
| bivalentIron Public «enum» |  | Default: |
| acidCarbonDioxide Public «enum» |  | Default: |
| sodium Public «enum» |  | Default: |
| lowSodium Public «enum» |  | Default: |

##### MineralGroundWaterBody

Type:		Class    GroundWaterBody

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-02-19.  Last modified on 2011-03-12.

GUID:		{85A4B4FF-609E-48b9-B106-1D51F2B939B4}

(Definition will be added later)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public MineralSpring | Public MineralGroundWaterBody |  |
| Association Bi-Directional | Public MineralWaterAbstractionWell | Public MineralGroundWaterBody |  |
| Dependency Source -> Destination | Public MineralGroundWaterBody | Public MineralWaterDescription |  |
| Association Bi-Directional | Public MineralWaterAquifer | Public MineralGroundWaterBody |  |
| Generalization Source -> Destination | Public MineralGroundWaterBody | Public GroundWaterBody |  |
| Dependency Source -> Destination | Public MineralGroundWaterBody | Public MineralWaterPurpose |  |
| Association LinkForNavigation Source -> Destination | Public MineralGroundWaterBody | Public AMArea | Invariant {if ( AMArea.amAreaType == prospectingAndMiningPermitArea && MineralGroundWaterBody.isProspectingAndMiningPermit == truth)} |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| mineralWaterDescription MineralWaterDescription Public «voidable» |  | Default: |
| puropseOfMineralWater MineralWaterPurpose Public «voidable» |  | Default: |
| isProspectingAndMiningPermit boolean Public «voidable» |  | Default: |

##### MineralSpring

Type:		Class    Spring

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-02-19.  Last modified on 2011-03-13.

GUID:		{4F1A4FEF-06AE-4a48-BFAC-7993215327D0}

-- Definition --

Point of mineral water discharge.

| Constraints |  |
| --- | --- |
|  | springUsage == mineral: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public MineralSpring | Public MineralGroundWaterBody |  |
| Association Bi-Directional | Public MineralSpring | Public MineralWaterAquifer |  |
| Generalization Source -> Destination | Public MineralSpring | Public Spring |  |

##### MineralWaterAbstractionWell

Type:		Class    ActiveWell (or AbstractionWell)

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-03-07.  Last modified on 2011-03-13.

GUID:		{D10C067F-3341-49ae-B75F-7CFEAFF58E7B}

-- Definition --

A WaterWell used for the of abstraction of  mineral water from a mineral groundwater body

| Constraints |  |
| --- | --- |
|  | wellPurpose == mineral: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public MineralWaterAbstractionWell | Public MineralGroundWaterBody |  |
| Generalization Source -> Destination | Public MineralWaterAbstractionWell | Public ActiveWell (or AbstractionWell) |  |
| Association Bi-Directional | Public MineralWaterAbstractionWell | Public MineralWaterAquifer |  |

##### MineralWaterAquifer

Type:		Class    Aquifer

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-15.

GUID:		{AD983AA8-7752-41d1-A4A8-1BEEE7447F76}

-- Definition --

An aquifer bearing groundwater containing over 1000mg/dm3 of dissolved substances of geogenic origin.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public MineralWaterAbstractionWell | Public MineralWaterAquifer |  |
| Generalization Source -> Destination | Public MineralWaterAquifer | Public Aquifer |  |
| Association Bi-Directional | Public MineralSpring | Public MineralWaterAquifer |  |
| Association Bi-Directional | Public MineralWaterAquifer | Public MineralGroundWaterBody |  |

##### MineralWaterDescription

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-04-13.  Last modified on 2011-04-13.

GUID:		{BF327C14-CE37-407f-8F22-24DEE49D2CFC}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public MineralGroundWaterBody | Public MineralWaterDescription |  |
| Dependency Source -> Destination | Public MineralWaterDescription | Public HardnessCode |  |
| Dependency Source -> Destination | Public MineralWaterDescription | Public MineralConcentration |  |
| Dependency Source -> Destination | Public MineralWaterDescription | Public Fresh-SaltClassCode |  |
| Dependency Source -> Destination | Public MineralWaterDescription | Public MineralContent |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| mineralConcentration MineralConcentration Public [0..1] «voidable» |  | Default: |
| mineralContent MineralContent Public [1..*] «voidable» |  | Default: |
| hardnessCode HardnessCode Public «voidable» |  | Default: |
| fres-SaltClassCode Fresh-SaltClassCode Public «voidable» |  | Default: |

##### MineralWaterPurpose

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-04-12.

GUID:		{2DA05EE3-74B0-4928-949A-48192FD9A764}

-- Definition --

The purpose for which a mineral water may be exploited.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public MineralGroundWaterBody | Public MineralWaterPurpose |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| therapeutic Public |  | Default: |
| bath Public |  | Default: |
| recreation Public |  | Default: |
| other Public |  | Default: |

##### ThermalSpring

Type:		Class    Spring

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-02-19.  Last modified on 2011-03-14.

GUID:		{356BDB90-3C29-459a-9076-9F2D859960C5}

-- Definition --

Point of mineral water discharge.

| Constraints |  |
| --- | --- |
|  | springUsage == termal: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public ThermalSpring | Public ThermalWaterPurpose |  |
| Association Bi-Directional | Public ThermalSpring | Public ThermallGroundWaterBody |  |
| Association Bi-Directional | Public ThermalSpring | Public ThermalWaterAquifer |  |
| Generalization Source -> Destination | Public ThermalSpring | Public Spring |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| temperature CGI_NumericRange Public «voidable» |  | Default: |

##### ThermalWaterAbstractionWell

Type:		Class    ActiveWell (or AbstractionWell)

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-03-07.  Last modified on 2011-03-14.

GUID:		{30EC0A76-53B1-4af6-9D09-AE8179E92A9B}

-- Definition --

A WaterWell used for the of abstraction of  mineral water from a mineral groundwater body

| Constraints |  |
| --- | --- |
|  | wellPurpose == termal: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public ThermalWaterAbstractionWell | Public ThermalWaterPurpose |  |
| Association Bi-Directional | Public ThermalWaterAbstractionWell | Public ThermallGroundWaterBody |  |
| Association Bi-Directional | Public ThermalWaterAbstractionWell | Public ThermalWaterAquifer |  |
| Generalization Source -> Destination | Public ThermalWaterAbstractionWell | Public ActiveWell (or AbstractionWell) |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| temperature CGI_NumericRange Public «voidable» |  | Default: |

##### ThermalWaterAquifer

Type:		Class    Aquifer

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-14.

GUID:		{5ADBB81F-3092-4f50-B2AA-728E16B863F4}

?????????????/

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public ThermalWaterAquifer | Public ThermallGroundWaterBody |  |
| Generalization Source -> Destination | Public ThermalWaterAquifer | Public Aquifer |  |
| Association LinkForNavigation Source -> Destination | Public ThermalWaterAquifer | Public GeothermalEnergyAssessmentUnit |  |
| Association Bi-Directional | Public ThermalSpring | Public ThermalWaterAquifer |  |
| Association Bi-Directional | Public ThermalWaterAbstractionWell | Public ThermalWaterAquifer |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| temperature CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |

##### ThermalWaterPurpose

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-04-12.

GUID:		{4A755F30-0D2F-4840-8E9F-B2AABB1BAD84}

-- Definition --

The purpose for which thermal water may be exploited (health reasons, energy production reasons),

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public ThermalSpring | Public ThermalWaterPurpose |  |
| Dependency Source -> Destination | Public ThermalWaterAbstractionWell | Public ThermalWaterPurpose |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| healthReasons Public |  | Default: |
| energyProduction Public |  | Default: |
| industrial Public |  | Default: |
| other Public |  | Default: |

##### ThermallGroundWaterBody

Type:		Class    GroundWaterBody

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_MineralAndThermalWaters     Keywords:

Detail:		Created on 2011-02-19.  Last modified on 2011-03-14.

GUID:		{48B39B97-FA54-4489-B931-3870B19ED9D8}

(Definition will be added later)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public ThermalWaterAquifer | Public ThermallGroundWaterBody |  |
| Generalization Source -> Destination | Public ThermallGroundWaterBody | Public GroundWaterBody |  |
| Association Bi-Directional | Public ThermalSpring | Public ThermallGroundWaterBody |  |
| Association Bi-Directional | Public ThermalWaterAbstractionWell | Public ThermallGroundWaterBody |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| temperature CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |

#### Hydrogeology_ObjectNatural

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_Extensions

Detail:		Created on 2011-04-11. Last modified on 2011-04-12

GUID:		{02AE8C74-96B2-4ac3-8A3A-74A4DD34CAB0}

Hydrogeology_ObjectNatural - (Logical diagram)

Created By:	Tomasz Nałęcz and Janusz Michalak on 2011-04-11

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{316B90F7-4481-4b30-9BF3-1CB140B7BE06}

Figure: 14

##### Seep

Type:		Class    HydrogeologicalObjectNatural

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_ObjectNatural     Keywords:

Detail:		Created on 2011-02-19.  Last modified on 2011-04-11.

GUID:		{493D6C67-CB9F-4449-BA13-AD3AC53B557C}

-- Definition --

Area of groundwater discharge.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public Seep | Public HydrogeologicalObjectNatural |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| quantityOfDischarge CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |

##### Spring

Type:		Class    HydrogeologicalObjectNatural

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_ObjectNatural     Keywords:

Detail:		Created on 2011-02-19.  Last modified on 2011-04-11.

GUID:		{309BC2CA-BF56-4549-980F-0148B03D6DF2}

-- Definition --

Point of groundwater discharge.

| Constraints |  |
| --- | --- |
|  | inheritance restriction: type of geometry is GM_Point: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public Spring | Public SpringTypeOrUsageCode |  |
| Association LinkForNavigation Source -> Destination | Public Spring | Public SpringOrSeep |  |
| Generalization Source -> Destination | Public Spring | Public HydrogeologicalObjectNatural |  |
| Generalization Source -> Destination | Public KarsticSpring | Public Spring |  |
| Generalization Source -> Destination | Public MineralSpring | Public Spring |  |
| Generalization Source -> Destination | Public ThermalSpring | Public Spring |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| quantityOfDischarge CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |
| springUsageOrType SpringTypeOrUsageCode Public «voidable» |  | Default: |

##### SpringTypeOrUsageCode

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_ObjectNatural     Keywords:

Detail:		Created on 2006-01-24.  Last modified on 2011-04-12.

GUID:		{02CB79C6-6643-4516-8007-8268204F25FF}

Code associated to define the type or usage of the spring.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | asDictionary = true. Values: true Default: true Description: Encode code list as externally managed dictionary (applies to ISO 19136:2007 encoding rule). Always true in INSPIRE. |
|  | codeSpace = . Description: URI that identifies the authority for this dictionary and its members |
|  | dictionaryIdentifier = . Description: URI that identifies this dictionary |
|  | memberIdentifierStem = . Description: URI stem used for dictionary members |
|  | xsdEncodingRule = iso19136_2007. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007 Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public Spring | Public SpringTypeOrUsageCode |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| mineral Public | A non-E&P well drilled for the purpose of locating and/or extracting a mineral from the subsurface, usually through the injection and/or extraction of mineral-bearing fluids. http://posc.org/technical/reference/POSC_well_purpose.html Mineral test hole- any hole in excess of one hundred (100) feet drilled during the exploration for minerals but shall exclude auger drilling in surficial or otherwise unconsolidated material, drilling in conjunction with mining or quarrying operations, and drill holes for the exploration of oil and/or gas, water, structural foundations, and seismic surveys. http://www.tennessee.gov/sos/rules/0950/0950-01-01.pdf | Default: |
| monitoringQantityOfDischarge Public | Monitoring well: A non-pumping well, generally of small diameter, that is used to measure the elevation of a water table or water quality. A piezometer, which is open only at the top and bottom of its casing, is one type of monitoring well. http://www.groundwater.org/gi/gwglossary.html#M monitoring wells or piezometers, are often smaller diameter wells used to Monitor the hydraulic head or sample the groundwater for chemical constituents. Piezometers are monitoring wells completed over a very short section of aquifer. Monitoring wells can also be completed at multiple levels, allowing discrete samples or measurements to be made at different vertical elevations at the same map location. http://en.wikipedia.org/wiki/Monitoring_well | Default: |
| monitoringQuality Public | Most monitoring wells constructed today are used to assess the nature and distribution of pollutants and contaminants in groundwater; The nature and distribution of naturally occurring chemical constituents; Subsurface hydrologic conditions; and, hydraulic properties of strata as they relate to pollutant and contaminant movement. http://www.dpla.water.ca.gov/sd/groundwater/california_well_standards/mws/mws_history.html | Default: |
| other Public | Any other usage of a well. | Default: |
| drinkingWaterSupply Public | Well used to supply water for various usage. | Default: |
| karstic Public |  | Default: |
| termal Public |  | Default: |

##### SwallowHole

Type:		Class    HydrogeologicalObjectNatural

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_ObjectNatural     Keywords:

Detail:		Created on 2011-03-07.  Last modified on 2011-04-11.

GUID:		{184AAF19-B6C6-4270-9FCA-F391E00ADB0E}

-- Definition --

Point of groundwater recharge .

-- Note --

In most cases, it recharges a karstic aquifer.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association LinkForNavigation Source -> Destination | Public SwallowHole | Public VanishingPoint |  |
| Generalization Source -> Destination | Public SwallowHole | Public HydrogeologicalObjectNatural |  |
| Aggregation Source -> Destination | Public SwallowHole | Public KarsticAquiferSystem |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| quantityOfRecharge CGI_NumericRange Public «voidable - estimatedProperty» |  | Default: |

#### Hydrogeology_Profile

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_Extensions

Detail:		Created on 2011-04-11. Last modified on 2011-04-13

GUID:		{5FF64F07-B30A-4735-BA2D-BF34CF0649A4}

Hydrogeology_Profile - (Logical diagram)

Created By:	Tomasz Nałęcz and Janusz Michalak on 2011-04-11

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{3FC7EA83-D5C8-4c32-AED2-82920EF67527}

Figure: 15

##### AquiferDescription

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Profile     Keywords:

Detail:		Created on 2007-05-30.  Last modified on 2011-04-11.

GUID:		{0566DBB4-0112-4a39-8A3A-06AFA75F4340}

-- Definition --

(From GWML) Properties of the rock (forming aquifer) that is relevant to the groundwater.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = true. Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xmlSchemaType = . Description: no type shall be generated for the class, but the given type or element referenced |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| descriptoin PT_FreeText Public «voidable» |  | Default: |
| capacityCoefficient GEh_Value Public «voidable - estimatedProperty» | Volumetric capacity of an aquifer to hold water. | Default: [sequenceNumber = 10 ] |
| hydraulicConductivityCoefficient GEh_Value Public «voidable - estimatedProperty» | Hydraulic conductivity can be measured by applying Darcy's law on the material. Such experiments can be conducted by creating a hydraulic gradient between two points, and measuring the flow rate (Oosterbaan and Nijland[1]). (Wikipedia) | Default: [sequenceNumber = 20 ] |
| storativityCoefficient GEh_Value Public «voidable - estimatedProperty» | Storativity is the volume of water released from storage per unit decline in hydraulic head in the aquifer, per unit area of the aquifer. http://en.wikipedia.org/wiki/Specific_storage | Default: [sequenceNumber = 30 ] |
| beginLifespanObject DateTime Public «lifeCycleInfo - voidable» |  | Default: |
| endLifespanObject DateTime Public [0..1] «lifeCycleInfo - voidable» |  | Default: |

##### HydrogeologicalProfile

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Profile     Keywords:

Detail:		Created on 2011-02-25.  Last modified on 2011-04-11.

GUID:		{EBE3EEB4-F159-4945-AF32-1209A4C33820}

-- Definition --

Hydrogeological profile (well log) - A record of the thickness and characteristics of the soil, rock and water-bearing formations encountered during the drilling (sinking) of a well.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public HydrogeologicalProfile | Public Well |  |
| Aggregation Source -> Destination | Public ProfileInterval | Public HydrogeologicalProfile |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| geometry GM_CompositeCurve Public |  | Default: |

##### ProfileInterval

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Profile     Keywords:

Detail:		Created on 2011-02-25.  Last modified on 2011-04-11.

GUID:		{CBA4FE0D-ECFE-476b-8DF8-A81BA32FAAF8}

-- Definition --

Profile interval (section) - Segment (section) of well profile described by the same or similar values of parameters.

ProfileInterval is a specialization of Interval class from ISO 19156.

-- Note --

Example of profile interval is a drilled aquifer from roof to floor.

Definition of Interval cass from ISO 19156:

An Interval is a simple curve, parameterized only by its two end-points.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public WaterQuantityDescription | Public ProfileInterval |  |
| Aggregation Source -> Destination | Public WaterQualityDescription | Public ProfileInterval |  |
| Aggregation Source -> Destination | Public WaterYieldDescription | Public ProfileInterval |  |
| Aggregation Source -> Destination | Public AquiferDescription | Public ProfileInterval |  |
| Aggregation Source -> Destination | Public ProfileInterval | Public HydrogeologicalProfile |  |
| Dependency Source -> Destination | Public ProfileInterval | Public RockMaterial |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| permeabilityCoefficient CGI_NumericRange Public «voidable» |  | Default: |
| geometry GM_Curve Public |  | Default: |
| hydraulicHead CGI_NumericRange Public [0..1] «voidable» | Hydraulic head or piezometric head is a specific measurement of water pressure above a geodetic datum. | Default: |
| rockMaterial RockMaterial Public «voidable» |  | Default: |

##### WaterYieldDescription

Type:		Class    WaterPropertyDescription

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_Profile     Keywords:

Detail:		Created on 2008-02-05.  Last modified on 2011-04-11.

GUID:		{1B0AA338-9C84-4364-9AB8-3AEBC1DE050F}

-- Definition --

(from GWML) A class used to describe the different yield parameters of an aquifer.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = true. Values: true Default: true Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public WaterYieldDescription | Public WaterPropertyDescription |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| safeYield GEh_ValueType Public «voidable - estimatedProperty» | -- Definition -- (from GWML) With reference to either a surface- or ground-water supply, the rate of diversion or extraction for Consumptive Use which can be maintained indefinitely, within the limits of economic feasibility, under specified conditions of water-supply development. Also see Perennial Yield. Yield, Perennial ? The amount of usable water of a ground-water reservoir that can be economically withdrawn and consumed each year for an indefinite period of time. It cannot exceed the sum of the Natural Recharge, the Artificial (or Induced) Recharge, and the Incidental Recharge without causing depletion of the groundwater reservoir. Also referred to as Safe Yield. http://water.nv.gov/WaterPlanning/dict-1/PDFs/wwords-y.pdf Amount of water that can be retreived from an aquifer without jeopardising the resources but still involved some impacts (def. to be improved) | Default: [sequenceNumber = 10 ] |
| specificYield GEh_ValueType Public «voidable - estimatedProperty» | -- Definition -- (from GWML) Specific yield, also known as the drainable porosity, is a ratio, less than or equal to the effective porosity, indicating the volumetric fraction of the bulk aquifer volume that a given aquifer will yield when all the water is allowed to drain out of it under the forces of gravity (http://en.wikipedia.org/wiki/Specific_storage) | Default: [sequenceNumber = 20 ] |
| sustainableYield GEh_ValueType Public «voidable - estimatedProperty» | --Definition -- (from GWML) The groundwater extraction regime, measured over a specified planning timeframe, that allows acceptable levels of stress and protects dependent economic, social, and environmental values. http://www.environment.gov.au/water/publications/environmental/groundwater/pubs/annex-a.pdf The sustainable yield of natural capital is the ecological yield that can be extracted without reducing the base of capital itself, i.e. the surplus required to maintain nature's services at the same or increasing level over time | Default: [sequenceNumber = 30 ] |

### Hydrogeology_LinksToOtherThemes

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_v.0.4

Detail:		Created on 2011-04-12. Last modified on 2011-04-12

GUID:		{F787F713-8F92-493d-869C-1860B3C2AAA2}

Hydrogeology: links to AM - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-14

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{76BD4F59-55BD-428f-AB38-723A2DDD7BB7}

Figure: 16

Hydrogeology: links to EF - (Logical diagram)

Created By:	Janusz Michalak on 2011-02-15

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{D897D2FE-AF15-48dc-B39A-53CAFB94A7CE}

Figure: 17

Hydrogeology: links to ER - (Logical diagram)

Created By:	Janusz Michalak on 2011-02-17

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{9A6C3FBE-3454-4ece-9D9E-7AC56E27E976}

Figure: 18

Hydrogeology: links to GN - (Logical diagram)

Created By:	Janusz Michalak on 2011-02-17

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{508B383F-7EE9-4766-B676-3AF3FB7586F8}

Figure: 19

Hydrogeology: links to HY - (Logical diagram)

Created By:	Janusz Michalak on 2011-02-15

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{33E497FE-A8ED-4aee-98BD-3785D5F3A800}

Figure: 20

Hydrogeology: links to PS - (Logical diagram)

Created By:	Janusz Michalak on 2011-02-15

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{974CD035-5428-4500-8D2B-2444C49E0D64}

Figure: 21

Hydrogeology: links to US - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-14

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{68FA57D8-52F1-4384-BB1E-5455A6E3564D}

Figure: 22

Hydrogeology_LinksToOtherThemes - (Package diagram)

Created By:	Janusz Michalak on 2011-03-14

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{8E482FDB-3C76-49de-B4E1-E615A8AFA6E8}

Figure: 23

#### HydrogeologicalName

Type:		Class    GeographicalName

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes     Keywords:

Detail:		Created on 2011-02-22.  Last modified on 2011-03-14.

GUID:		{0786A93F-112E-4200-B7E0-7A71CC44DC66}

(Definition will be added later)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public HydrogeologicalName | Public GeographicalName |  |
| Aggregation Destination -> Source | Public HydrogeologicalName | Public HydrogeologicalNamesRegister |  |
| Dependency Source -> Destination | Public HydrogeologicalNamedPlace | Public HydrogeologicalName |  |

#### HydrogeologicalNamedPlace

Type:		Class    NamedPlace

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes     Keywords:

Detail:		Created on 2008-07-08.  Last modified on 2011-03-14.

GUID:		{71E45031-F472-4edb-B39E-3A3DBD354C54}

-- Definition --

Any real world entity referred to by one or several proper nouns.

| Constraints |  |
| --- | --- |
|  | GeographicalName is restricted to HydrogeologicalName: (Invariant, Status is Approved) |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. |
|  | isCollection = false. |
|  | noPropertyType = false. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public HydrogeologicalNamedPlace | Public HydrogeologicalName |  |
| Association Bi-Directional | Public HydrogeologicalObject | Public HydrogeologicalNamedPlace |  |
| Association Bi-Directional | Public HydrogeologicalFeature | Public HydrogeologicalNamedPlace |  |
| Generalization Source -> Destination | Public HydrogeologicalNamedPlace | Public NamedPlace |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| name HydrogeologicalName Public [1..*] «restricted» |  | Default: |

#### HydrogeologicalNamesRegister

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes     Keywords:

Detail:		Created on 2011-03-12.  Last modified on 2011-03-14.

GUID:		{BE35DECB-45A7-4fd6-9F4C-2E87B750624D}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Destination -> Source | Public HydrogeologicalName | Public HydrogeologicalNamesRegister |  |

#### From AM - Area management

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes

Detail:		Created on 2011-03-12. Last modified on 2011-03-14

GUID:		{EED95BE7-01E0-4dd7-88F7-9ED602B9EDED}

##### AMArea

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From AM - Area management     Keywords:

Detail:		Created on 2010-07-12.  Last modified on 2011-03-14.

GUID:		{8F34826C-D210-4db1-B7B5-EFEABD7A949E}

-- Name --

Management, restriction or reporting area

-- Definition --

Area managed, regulated or used for reporting at international, European, national, regional and local levels.

-- Description --

SOURCE [INSPIRE Directive]

NOTE This spatial object type covers managed areas, regulated areas and reporting units. The same area can be of several of these types.

EXAMPLE A river basin district as defined in the WFD is a managed area, a regulated area and a reporting unit.

| Constraints |  |
| --- | --- |
|  | management or restriction or reporting info mandatory: (OCL, Status is Approved) /* Either the managementInformation or the restrictedActivity or the reportingInformation association role have to have a minimum multiplicity of 1. */ inv: managementInformation-&gt;notEmpty() or restrictedActivity-&gt;notEmpty() or reportingInformation-&gt;notEmpty() |
|  | geometry or grid cell reference mandatory: (OCL, Status is Approved) /*If the ManagementRestrictionOrReportingArea object does not consist of zones, the geometry or gridCellReference attribute must not be empty.*/ inv: geometry-&gt;notEmpty() or gridCellReference-&gt;notEmpty() |
|  | consistency of zone geometries: (OCL, Status is Approved) /*The geometries of the zones of an management, regulation or reporting area shall not overlap.*/ |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public WFDGroundWaterBody | Public AMArea |  |
| Association LinkForNavigation Source -> Destination | Public ProtectionZone | Public AMArea | Invariant {if ( AMArea.amAreaType == restrictedAreaAroundDrinkingWaterSource && GroundWaterIntake.waterUsage == forDrinking)} |
| Dependency Source -> Destination | Public AMArea | Public AMAreaTypeValues |  |
| Association LinkForNavigation Source -> Destination | Public MineralGroundWaterBody | Public AMArea | Invariant {if ( AMArea.amAreaType == prospectingAndMiningPermitArea && MineralGroundWaterBody.isProspectingAndMiningPermit == truth)} |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| inspireID Identifier Public | -- Name -- INSPIRE identifier. -- Definition -- External object identifier of the spatial object. | Default: |
| geometry GM_Object Public [0..1] | -- Name -- geometry -- Definition -- The geometry of the spatial object. -- Description -- NOTE 1 The geometry can have any geometry type. In most cases, however, it will be either a GM_(Multi)Surface or a GM_(Multi)Solid NOTE 2 The geometry of the management, restriction or reporting area has to be specified using this attribute or by providing a grid cell reference. NOTE 3 The geometry has to be explicitly provided, even if it is derived from the geometry of another spatial object. In this case, the way the geometry was derived can be specified using the geometryOrigin attribute. | Default: |
| gridCellReference CharacterString Public [0..1] | -- Name -- grid cell reference -- Definition -- Reference to a grid cell that represents the geometry of the management, regulation or reporting area. -- Description -- NOTE The corresponding grid is specified in the system of management, regulation or reporting areas the area belongs to. | Default: |
| amAreaIdentifier CharacterString Public [0..1] | -- Name -- identifier of the AM area -- Definition -- Unique thematic identifier for the management, regulation or reporting area. -- Description -- NOTE This will typically be the identifier through which other (e.g. reporting) data will be associated with the management, regulation or reporting area. | Default: |
| legalBasis LegalAct Public [0..1] «voidable» | -- Name -- legal basis -- Definition -- The legal act, in which the management, regulation or reporting area is defined. -- Description -- NOTE This attribute does not need to be provided if the area is defined in the legal act defining the system of management, regulation or reporting areas. | Default: |
| amAreaName GeographicalName Public [0..*] «voidable» | -- Name -- name of the management, regulation or reporting area -- Definition -- Common name used for the management, regulation or reporting area. | Default: |
| amAreaType AMAreaTypeValues Public «voidable» | -- Name -- type of the management, regulation or reporting area. -- Definition -- The type of the management, regulation or reporting area. | Default: |
| geometryOrigin GeometryOrigin Public [0..1] «voidable» | -- Name -- origin of the geometry -- Definition -- Description of the source of the geometry and the way the geometry was derived. -- Description -- EXAMPLE 1 The geometry of a management, regulation or reporting unit can be the same as that of an administrative or statistical unit. EXAMPLE 2 The geometry of a management, regulation or reporting unit can the derived through a buffer around the geometry of a spatial object, e.g. a school, a road or a lake. | Default: |

##### AMAreaTypeValues

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From AM - Area management     Keywords:

Detail:		Created on 2010-07-16.  Last modified on 2011-03-14.

GUID:		{953888E8-8FB3-4324-8AAD-CA8F7E97B01E}

-- Definition --

Code list describing the different types of AM areas.

-- Description --

NOTE This list is currently including the types listed in the definition of the AM theme in the INSPIRE Directive. It will be modified based once a list of AMArea types to be made available by the Member States has been elaborated.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public AMArea | Public AMAreaTypeValues |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| dumpingSite Public | -- Name -- dumping site -- Definition -- A dumping site as defined in ... -- Description -- EXAMPLE ... | Default: |
| restrictedAreaAroundDrinkingWaterSource Public | -- Name -- restricted area around drinking water source -- Definition -- A restricted area around drinking water source as defined in ... -- Description -- EXAMPLE ... | Default: |
| nitrateVulnerableZone Public | -- Name -- nitrate-vulnerable zone -- Definition -- A nitrate-vulnerable zone as defined in ... -- Description -- EXAMPLE ... | Default: |
| regulatedFairwayAtSeaOrLargeInlandWater Public | -- Name -- regulated fairway at sea or large inland water -- Definition -- A regulated fairway at sea or large inland water as defined in ... -- Description -- EXAMPLE ... | Default: |
| areaForDumpingOfWaste Public | -- Name -- area for dumping of waste -- Definition -- An area for dumping of waste as defined in ... -- Description -- EXAMPLE ... | Default: |
| noiseRestrictionZone Public | -- Name -- noise restriction zone -- Definition -- A noise restriction zone as defined in ... -- Description -- EXAMPLE ... | Default: |
| prospectingAndMiningPermitArea Public | -- Name -- prospecting and mining permit area -- Definition -- A dumping site as defined in ... -- Description -- EXAMPLE ... | Default: |
| riverBasinDistrict Public | -- Name -- river basin district -- Definition -- A river basin district as defined in Directive 2000/60/EC (Water Framework Directive). -- Description -- EXAMPLE ... | Default: |
| reportingUnit Public | -- Name -- reporting unit -- Definition -- A reporting unit as defined in the legal act defining the reporting obligation for which the unit is used. -- Description -- EXAMPLE ... | Default: |
| coastalZoneManagementArea Public | -- Name -- coastal zone management area -- Definition -- A coastal zone management area as defined in ... -- Description -- EXAMPLE ... | Default: |

##### WFDGroundWaterBody

Type:		Class    AMArea, WFDWaterBody

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From AM - Area management     Keywords:

Detail:		Created on 2008-08-10.  Last modified on 2011-03-14.

GUID:		{9AF32C28-C9C8-403e-8791-EAD1D44ED8B2}

-- Definition --

A distinct volume of groundwater within an aquifer or aquifers.

-- Description --

SOURCE [2000/60/EC Art. 2(12)].

NOTE All WFD spatial object types are regarded as candidate spatial objects in Annex III theme 'Area management/restriction/regulation zones and reporting units' due to their primary function for reporting under Directive 2000/60/EC.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: false Default: false Description: Create a property type that requires that the instance is encoded inline (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | inspireConcept = FIXME. Default: FIXME Description: URN reference to the feature concept in the INSPIRE Feature Concept Dictionary Register |
|  | isCollection = false. Values: true ¦ false Default: false Description: Identifies the feature type as a feature collection. |
|  | noPropertyType = false. Values: false Default: false Description: Surpress creation of a standard property type that supports inline or by-reference encoding (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007_INSPIRE_Extensions Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association LinkForNavigation Source -> Destination | Public WFD_GroundWaterBody | Public WFDGroundWaterBody |  |
| Generalization Source -> Destination | Public WFDGroundWaterBody | Public WFDWaterBody |  |
| Generalization Source -> Destination | Public WFDGroundWaterBody | Public AMArea |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| geometry GM_Primitive Public «voidable» | -- Definition -- The geometry of the WFD groundwater body. | Default: |

#### From EF - MonitoringFacilities

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes

Detail:		Created on 2011-03-12. Last modified on 2011-03-12

GUID:		{6DAADE89-A2F4-4afb-89A2-F6B29CDD75A0}

##### MonitoringFacility

Type:		Class    MonitoringFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From EF - MonitoringFacilities     Keywords:

Detail:		Created on 2010-10-11.  Last modified on 2011-03-14.

GUID:		{580F6017-1286-490b-B9EC-CF07EC5827F5}

-- Definition --

A facility at/from which instruments are hosted or deployed to undertake environmental monitoring (insitu

or remotely sensed); either a static observation site or a moving platform.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association LinkForNavigation Source -> Destination | Public ObservationWell | Public MonitoringFacility | Invariant {if ( MonitoringFacility.monitoredMedium == groundWater )} |
| Association LinkForNavigation Source -> Destination | Public GroundWaterMonitoringStation | Public MonitoringFacility | Invariant {if ( MonitoringFacility.monitoredMedium == groundWater )} |
| Generalization Source -> Destination | Public MonitoringFacility | Public MonitoringFeature |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| altitude double Public [0..1] | -- Definition -- Vertical extent | Default: |
| hierarchy CodeList Public «voidable» | -- Definition -- granularity (site/station/sensor); as hierarchies of monitoring facilities are allowed; codelist with clear definition of terms and constraints connected to link ParentFacility | Default: |
| monitoredMedium CodeList Public | -- Definition -- Code list on the monitored environmental medium, granularity of code list as common to all has to be clarifies | Default: |
| monitoringType CodeList Public «voidable» | -- Definition -- Could be a codelist (as in UK EOF model). There it can be a computation (out of scope for EF), a field trip, flight or cruise, monitoring network, programme, project... | Default: |
| administrativeArea CharacterString Public | -- Definition -- Administrative unit | Default: |
| previousIndentifier CharacterString Public [0..*] | -- Definition -- if station has vastly changed ID of old feature | Default: |

##### MonitoringFeature

Type:		Class    OM_Process

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From EF - MonitoringFacilities     Keywords:

Detail:		Created on 2010-09-09.  Last modified on 2011-03-14.

GUID:		{A2219545-43A1-4caa-90EC-EED0A8CF870F}

-- Definition --

MonitoringFeature is the BaseObject for MonitoringActivities and MonitoringFacilities

-- Description --

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public MonitoringFeature | Public OM_Process |  |
| Generalization Source -> Destination | Public MonitoringFacility | Public MonitoringFeature |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| inspireId Identifier Public |  | Default: |
| beginLifespan DateTime Public «voidable» | -- Definition -- BeginLifespan of MonitoringFeature (Temporal extent) | Default: |
| endLifespan DateTime Public [0..1] «voidable» | -- Definition -- EndLifespan of MonitoringFeature (Temporal extent) | Default: |
| geometry GM_Object Public |  | Default: |
| name CharacterString Public [0..*] | -- Definition -- Plain text denotation of facility | Default: |
| keyword CharacterString Public [0..*] | -- Definition -- Reference to controlled vocabulary (thesaurus like GEMET,...) | Default: |
| comment CharacterString Public | -- Definition -- Plain text description of additional information not fitting in other attributes | Default: |
| responsibleParty CharacterString Public | -- Definition -- Responsible party for the monitoring | Default: |
| beginLifespanVersion DateTime Public «voidable» |  | Default: |
| endLifespanVersion DateTime Public [0..1] «voidable» |  | Default: |
| legalFoundationDocument CI_Citation Public [1..*] «voidable» | -- Definition -- A URL or text citation referencing the legal act that created, regulates or maintain a MonitoringFeature. (taken from Protected Sites) | Default: |
| process OM_Process Public [0..*] «voidable» | --Definition-- Processes with their corresponding Observations where this MonitoringFeature controls the Processes or is part of a Process-Chain | Default: |

##### OM_Process

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From EF - MonitoringFacilities     Keywords:

Detail:		Created on 2008-07-07.  Last modified on 2011-03-14.

GUID:		{FF9C258F-ABFC-4bcb-AA1A-253A9D88B948}

The class OM_Process (Figure 2) is an instance of the «metaclass» GF_FeatureType (ISO 19109:2005), which therefore represents a feature type. OM_Process is abstract, and has no attributes, operations or associations. It serves as the base class for observation processes. The purpose of an observation process is to generate an observation result. An instance of OM_Process is often an instrument or sensor, but may be a human observer, a simulator, or a process or algorithm applied to more primitive results used as inputs.

NOTE    ISO 19115-2:2008 provides MI_Instrument, LE_Processing and LE_Algorithm, which could all be modelled as specializations of OM_Process. OGC SensorML [10] provides a model which is suitable for many observation procedures.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: true ¦ false Default: false Description: enforce the 'by value' property pattern without xlinks |
|  | isCollection = false. Values: true ¦ false Default: false Description: attribute group gml:AggregationAttributeGroup is added to the complex type of the feature type |
|  | noPropertyType = false. Values: true ¦ false Default: false Description: suppress automatic creation of *PropertyType complexType |
|  | xsdDerivation = true. Values: true ¦ false Default: true Description: class implemented as new type + element, or by applying constraints only |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public MonitoringFeature | Public OM_Process |  |

#### From ER - Energy resources

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes

Detail:		Created on 2011-03-12. Last modified on 2011-03-12

GUID:		{60172B4F-E06D-4feb-A2C0-C926668C31A0}

##### EnergyResource

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From ER - Energy resources     Keywords:

Detail:		Created on 2010-07-21.  Last modified on 2011-03-14.

GUID:		{9DB6C75C-078B-46e5-81E5-D3DEE46225E5}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public EnergyResourceAssessmentUnit | Public EnergyResource |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| beginLifespanVersion DateTime Public «voidable» | -- Definition -- Date and time at which this version of the spatial object was inserted or changed in the spatial data set. | Default: |
| endLifespanVersion DateTime Public [0..1] «voidable» | -- Definition -- Date and time at which this version of the spatial object was superseded or retired in the spatial data set. | Default: |
| geometry GM_Object Public | -- Definition -- Geometry associated to the energy resource deposit. This data specification does not restrict the geometry types. | Default: |
| inspireID Identifier Public | -- Definition -- External object identifier of the spatial object. -- Description -- NOTE An external object identifier is a unique object identifier published by the responsible body, which may be used by external applications to reference the spatial object. The identifier is an identifier of the spatial object, not an identifier of the real-world phenomenon. | Default: |

##### EnergyResourceAssessmentUnit

Type:		Class    EnergyResource

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From ER - Energy resources     Keywords:

Detail:		Created on 2010-07-21.  Last modified on 2011-03-14.

GUID:		{A2BFDF75-7914-4535-9922-DBF2789B8CA3}

-- Definition --

An entity which is used to study, measure or observe natural phenomena from which energy is or might be extracted.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public EnergyResourceAssessmentUnit | Public EnergyResource |  |
| Generalization Source -> Destination | Public GeothermalEnergyAssessmentUnit | Public EnergyResourceAssessmentUnit |  |

##### GeothermalEnergyAssessmentUnit

Type:		Class    EnergyResourceAssessmentUnit

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From ER - Energy resources     Keywords:

Detail:		Created on 2010-09-29.  Last modified on 2011-03-14.

GUID:		{A3398DF8-A4A2-4749-9E4A-AFF87A152609}

-- Definition --

Abstract feature used as embeding structure for characteristics related to geothermal energy (geothermal gradient, heat flow).

-- Description --

Geographical regions where geology and tectonics have determined that geothermal gradient and heat flow are reaching such rates that either warmed up water in place is withdrawn, or water artificialy injected and consequently when it's warmed up withdrawn, in order to use it for power generation or direct heating.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public GeothermalEnergyAssessmentUnit | Public EnergyResourceAssessmentUnit |  |
| Association LinkForNavigation Source -> Destination | Public ThermalWaterAquifer | Public GeothermalEnergyAssessmentUnit |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| heatFlowdensity mW/m2 Public | -- Definition -- Heat flows constantly from its sources within the Earth to the surface. Heat flow Q (W/m2) is calculated value (Q=K.?T/?Z), where K is the thermal conductivity of the rock. | Default: |
| geothermalGradient °C/km Public | -- Definition -- The geothermal gradient is the rate of temperature change (T) with depth (Z), in the Earth. | Default: |

#### From GN - Gepgraphical Names

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes

Detail:		Created on 2011-03-12. Last modified on 2011-03-12

GUID:		{2F567C69-7B6B-4553-80D0-B953BD9D77CC}

##### GeographicalName

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From GN - Gepgraphical Names     Keywords:

Detail:		Created on 2008-09-08.  Last modified on 2011-03-14.

GUID:		{F03C888B-26FF-4745-ACE4-2C73315D2B57}

-- Definition --

Proper noun applied to a real world entity.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | isCollection = false. |
|  | noPropertyType = false. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public NamedPlace | Public GeographicalName |  |
| Generalization Source -> Destination | Public HydrogeologicalName | Public GeographicalName |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| language CharacterString Public «voidable» | -- Definition -- Language of the name, given as a three letters code, in accordance with either ISO 639-3 or ISO 639-5. -- Description -- NOTE 1More precisely, this definition refers to the language used by the community that uses the name. NOTE 2 The code "mul" for "multilingual" should not be used in general. However it can be used in rare cases like official names composed of two names in different languages. For example, "Vitoria-Gasteiz" is such a multilingual official name in Spain. NOTE 3 Even if this attribute is "voidable" for pragmatic reasons, it is of first importance in several use cases in the multi-language context of Europe. | Default: |
| nativeness NativenessValue Public «voidable» | -- Definition -- Information enabling to acknowledge if the name is the one that is/was used in the area where the spatial object is situated at the instant when the name is/was in use. | Default: |
| nameStatus NameStatusValue Public «voidable» | -- Definition -- Qualitative information enabling to discern which credit should be given to the name with respect to its standardisation and/or its topicality. -- Description -- NOTE The Geographical Names application schema does not explicitly make a preference between different names (e.g. official endonyms) of a specific real world entity. The necessary information for making the preference (e.g. the linguistic status of the administrative or geographic area in question), for a certain use case, must be obtained from other data or information sources. For example, the status of the language of the name may be known through queries on the geometries of named places against the geometry of administrative units recorded in a certain source with the language statuses information. | Default: |
| sourceOfName CharacterString Public «voidable» | -- Definition -- Original data source from which the geographical name is taken from and integrated in the data set providing/publishing it. For some named spatial objects it might refer again to the publishing data set if no other information is available. -- Description -- EXAMPLES Gazetteer, geographical names data set. | Default: |
| pronunciation PronunciationOfName Public «voidable» | -- Definition -- Proper, correct or standard (standard within the linguistic community concerned) pronunciation of the geographical name. -- Description -- SOURCE Adapted from [UNGEGN Manual 2006]. | Default: |
| spelling SpellingOfName Public [1..*] | -- Definition -- A proper way of writing the geographical name. -- Description -- NOTE 1 Different spellings should only be used for names rendered in different scripts. . NOTE 2 While a particular GeographicalName should only have one spelling in a given script, providing different spellings in the same script should be done through the provision of different geographical names associated with the same named place. | Default: |
| grammaticalGender GrammaticalGenderValue Public [0..1] «voidable» | -- Definition -- Class of nouns reflected in the behaviour of associated words. -- Description -- NOTE the attribute has cardinality [0..1] and is voidable, which means that: - in case the concept of grammatical gender has no sense for a given name (i.e. the attribute is not applicable), the attribute should not be provided. - in case the concept of grammatical gender has some sense for the name but is unknown, the attribute should be provided but void. | Default: |
| grammaticalNumber GrammaticalNumberValue Public [0..1] «voidable» | -- Definition -- Grammatical category of nouns that expresses count distinctions. -- Description -- NOTE the attribute has cardinality [0..1] and is voidable, which means that: - in case the concept of grammatical number has no sense for a given name (i.e. the attribute is not applicable), the attribute should not be provided. - in case the concept of grammatical number has some sense for the name but is unknown, the attribute should be provided but void. | Default: |

##### NamedPlace

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From GN - Gepgraphical Names     Keywords:

Detail:		Created on 2008-07-08.  Last modified on 2011-03-14.

GUID:		{85F56E0B-E08C-47db-B954-85BB7DF4DEE0}

-- Definition --

Any real world entity referred to by one or several proper nouns.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. |
|  | isCollection = false. |
|  | noPropertyType = false. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public NamedPlace | Public GeographicalName |  |
| Generalization Source -> Destination | Public HydrogeologicalNamedPlace | Public NamedPlace |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| beginLifespanVersion DateTime Public «voidable» | -- Definition -- Date and time at which this version of the spatial object was inserted or changed in the spatial data set. | Default: |
| endLifespanVersion DateTime Public [0..1] «voidable» | -- Definition -- Date and time at which this version of the spatial object was superseded or retired in the spatial data set. | Default: |
| geometry GM_Object Public | -- Definition -- Geometry associated to the named place. This data specification does not restrict the geometry types. -- Description -- NOTE 1 The most common geometry types for a named place are a reference point (modelled as GM_Point), a more precise geometry of the footprint (typically modelled as GM_Curve or GM_Surface), or a bounding box (to be modelled as a GM_Envelope). NOTE 2 If the geometry depicts the spatial footprint of the named place, a reference point and a bounding box could be derived from it. However, this specification does not require the explicit provision of any specific type of geometry such as bounding boxes or reference points. NOTE 3 To avoid any misunderstanding, note that null geometry is not allowed by this specification. NOTE 4 3D geometries are not really required for Geographical Names, but the model allows for it, so a data provider may publish it. | Default: |
| inspireId Identifier Public | -- Definition -- External object identifier of the spatial object. -- Description -- NOTE An external object identifier is a unique object identifier published by the responsible body, which may be used by external applications to reference the spatial object. The identifier is an identifier of the spatial object, not an identifier of the real-world phenomenon. | Default: |
| leastDetailedViewingResolution MD_Resolution Public [0..1] «voidable» | -- Definition -- Resolution, expressed as the inverse of an indicative scale or a ground distance, above which the named place and its associated name(s) should no longer be displayed in a basic viewing service. -- Description -- NOTE 1This information may be used to determine if the names of the named place should be displayed at a given scale of display, only in the context of basic viewing services intending to show the content of the data set containing names. Even if this information is a valuable one for mapping in general, it is only approximate; cartographic services intending to produce high quality maps should certainly rely on other criteria and techniques for selecting names for the map. NOTE 2 Even if this attribute is "voidable" for practical reasons linked to its availability in data sources, this information may be of first importance for viewing services. There are great chances that viewing services will inefficiently manage named places having this attribute void. EXAMPLES The following examples use the equivalentScale attribute of MD_Resolution to express the attribute value. - Names of important cities in Europe may be displayed at all viewing scales greater than 1/5,000,000. In this case, the value of the attribute is 5,000,000 - Names of small hamlets may only be displayed from all viewing scale greater than 1/25,000. In this case, the value of the attribute is 25,000 - Names of countries may be displayed at any small scale. In this case, this attribute is not filled. NOTE 3 If the data set contain multiple representations of the same real world entity represented at different levels of detail, the scale ranges defined by the attributes leastDetailedViewingResolution and mostDetailedViewingResolution should not overlap, in order to avoid displaying the same names several times. NOTE 4 The geometry of the named place should have a level of detail (i.e. resolution, granularity, precision, etc.) roughly compatible with its associated viewing scales. | Default: |
| localType LocalisedCharacterString Public [1..*] «voidable» | -- Definition -- Characterisation of the kind of entity designated by geographical name(s), as defined by the data provider, given in at least in one official language of the European Union. -- Description -- SOURCE Adapted from [UNGEGN Manual 2007]. NOTE Local types may be defined in additional European languages, either EU official languages or other languages such as the language(s) of the geographical names provided. | Default: |
| mostDetailedViewingResolution MD_Resolution Public [0..1] «voidable» | -- Definition -- Resolution, expressed as the inverse of an indicative scale or a ground distance, below which the named place and its associated name(s) should no longer be displayed in a basic viewing service. -- Description -- NOTE See Description of leastDetailedViewingResolution EXAMPLES The following examples use the equivalentScale attribute of MD_Resolution to express the attribute value. - Names of wide areas like mountain ranges may not be displayed at all in viewing scales greater than 1/100,000. In this case, the value of the attribute is 100,000 - Names of small hamlets may be displayed at any large scale. In this case, this attribute is not filled. | Default: |
| name GeographicalName Public [1..*] | -- Definition -- Name of the named place. | Default: |
| relatedSpatialObject Identifier Public [0..*] «voidable» | -- Definition -- Identifier of a spatial object representing the same entity but appearing in other themes of INSPIRE, if any. -- Description -- NOTE If no identifier is provided with features of other INSPIRE themes, those features can of course not be referred by the NamedPlace. | Default: |
| type NamedPlaceTypeValue Public [1..*] «voidable» | -- Definition -- Characterisation of the kind of entity designated by geographical name(s). -- Description -- SOURCE Adapted from [UNGEGN Manual 2007]. NOTE 1 This attribute should be consistent with the attribute 'relatedSpatialObject'. More precisely, if the attribute 'relatedSpatialObject' is filled in, the attribute 'type' should be filled in, and its value(s) should be consistent with the spatial data theme(s) of the related object(s). NOTE 2 Even if this attribute may introduce some redundancy with the attribute 'relatedSpatialObject', it has to be filled in order to allow to use geographical names on their own without accessing to any other INSPIRE data set, which may be necessary in most cases. | Default: |

#### From HY - Hydrography

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes

Detail:		Created on 2011-03-12. Last modified on 2011-03-12

GUID:		{A49AEB75-D536-4006-BDC7-A5F8F99F5B4E}

##### HydroObject

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From HY - Hydrography     Keywords:

Detail:		Created on 2009-06-17.  Last modified on 2011-03-14.

GUID:		{9FBD995E-E94B-4b54-A026-DAC170F3CBDE}

-- Definition --

An identity base for hydrographic (including man-made) objects in the real world.

-- Description --

NOTE Derived 'views' of real-world hydrographic objects are represented through specialisations in other application schemas; all representations of the same real-world object share a common geographic name or hydrographic identifier.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: false Default: false Description: Create a property type that requires that the instance is encoded inline (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | gmlMixin = true. |
|  | inspireConcept = FIXME. Default: FIXME Description: URN reference to the feature concept in the INSPIRE Feature Concept Dictionary Register |
|  | isCollection = false. Values: true ¦ false Default: false Description: Identifies the feature type as a feature collection. |
|  | noPropertyType = false. Values: false Default: false Description: Surpress creation of a standard property type that supports inline or by-reference encoding (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007_INSPIRE_Extensions Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public HydroPointOfInterest | Public HydroObject |  |
| Generalization Source -> Destination | Public SurfaceWater | Public HydroObject |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| geographicalName GeographicalName Public [0..*] «voidable» | -- Definition -- A geographical name that is used to identify a hydrographic object in the real world. It provides a 'key' for implicitly associating different representations of the object. -- Description -- EXAMPLE A standing water in a mapping view may share the same geographical name as a WFD lake in a reporting view, implying they are both representations of the same real world object. | Default: |
| hydroId HydroIdentifier Public [0..*] «voidable» | -- Definition -- An identifier that is used to identify a hydrographic object in the real world. It provides a 'key' for implicitly associating different representations of the object. -- Description -- NOTE 1 The identifier may be a national hydrological identification code. NOTE 2 More than one identifier may be required, for instance a watercourse may be assigned different identifying codes under national and European schemes. EXAMPLE A lock in a mapping view may share the same identifier as a hydro node in a network view, implying they are both representations of the same real world object. | Default: |

##### HydroPointOfInterest

Type:		Class    HydroObject

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From HY - Hydrography     Keywords:

Detail:		Created on 2008-08-12.  Last modified on 2011-03-14.

GUID:		{3941C718-15BA-47ab-ACD6-37C3A361FC87}

-- Definition --

A natural place where water appears, disappears or changes its flow.

-- Description --

EXAMPLE Fluvial points (waterfall, cascade, rapids, breaker), spring/water hole (spring, source, geyser, thermal spring, natural fountain, well, also fumarole, artesian), sinkhole (sinkhole, drainage loss).

NOTE A hydro point of interest may create a flow constriction in the network.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: false Default: false Description: Create a property type that requires that the instance is encoded inline (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | inspireConcept = FIXME. Default: FIXME Description: URN reference to the feature concept in the INSPIRE Feature Concept Dictionary Register |
|  | isCollection = false. Values: true ¦ false Default: false Description: Identifies the feature type as a feature collection. |
|  | noPropertyType = false. Values: false Default: false Description: Surpress creation of a standard property type that supports inline or by-reference encoding (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007_INSPIRE_Extensions Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public SpringOrSeep | Public HydroPointOfInterest |  |
| Generalization Source -> Destination | Public VanishingPoint | Public HydroPointOfInterest |  |
| Generalization Source -> Destination | Public HydroPointOfInterest | Public HydroObject |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| beginLifespanVersion DateTime Public «voidable» | -- Definition -- Date and time at which this version of the spatial object was inserted or changed in the spatial data set. | Default: |
| endLifespanVersion DateTime Public [0..1] «voidable» | -- Definition -- Date and time at which this version of the spatial object was superseded or retired in the spatial data set. | Default: |
| geometry GM_Primitive Public «voidable» | -- Definition -- The geometry of the hydro point of interest, as a point, curve or surface. | Default: |
| inspireId Identifier Public | -- Definition -- External object identifier of the spatial object. -- Description -- NOTE An external object identifier is a unique object identifier published by the responsible body, which may be used by external applications to reference the spatial object. The identifier is an identifier of the spatial object, not an identifier of the real-world phenomenon. | Default: |
| levelOfDetail MD_Resolution Public [0..1] | -- Definition -- Resolution, expressed as the inverse of an indicative scale or a ground distance. -- Description -- NOTE The object is captured at a scale of this level of detail; rules apply for portrayal and visualisation. | Default: |

##### SpringOrSeep

Type:		Class    HydroPointOfInterest

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From HY - Hydrography     Keywords:

Detail:		Created on 2008-08-12.  Last modified on 2011-03-14.

GUID:		{FE4E8B87-0F77-47b5-B072-E5A3C0B75502}

-- Definition --

A natural outflow of water from below the ground surface.

-- Description --

NOTE 1 Corresponds to a 'source' node in a network view.

NOTE 2 Regarded as a placeholder in Annex II theme 'Geology' due to the connection with groundwater.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. |
|  | isCollection = false. |
|  | noPropertyType = false. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public SpringOrSeep | Public HydroPointOfInterest |  |
| Association LinkForNavigation Source -> Destination | Public Spring | Public SpringOrSeep |  |
| Association LinkForNavigation Source -> Destination | Public KarsticSpring | Public SpringOrSeep |  |

##### SurfaceWater

Type:		Class    HydroObject

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From HY - Hydrography     Keywords:

Detail:		Created on 2008-06-25.  Last modified on 2011-03-14.

GUID:		{DD111828-D9CE-492d-9367-5CE245225910}

-- Definition --

Any known inland waterway body.

-- Description --

SOURCE [Based on DFDD].

EXAMPLE Lake/pond, reservoir, river/stream, etc.

NOTE May include islands, represented as 'holes' in its geometry. Islands may be surrounded by a shore and / or land-ware boundary.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: false Default: false Description: Create a property type that requires that the instance is encoded inline (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | inspireConcept = FIXME. Default: FIXME Description: URN reference to the feature concept in the INSPIRE Feature Concept Dictionary Register |
|  | isCollection = false. Values: true ¦ false Default: false Description: Identifies the feature type as a feature collection. |
|  | noPropertyType = false. Values: false Default: false Description: Surpress creation of a standard property type that supports inline or by-reference encoding (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007_INSPIRE_Extensions Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public SurfaceWater | Public HydroObject |  |
| Association BaseOfDrainage (LinksForNavigation) Source -> Destination | Public isDrained Aquifer | Public draines SurfaceWater |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| beginLifespanVersion DateTime Public «voidable» | -- Definition -- Date and time at which this version of the spatial object was inserted or changed in the spatial data set. | Default: |
| endLifespanVersion DateTime Public [0..1] «voidable» | -- Definition -- Date and time at which this version of the spatial object was superseded or retired in the spatial data set. | Default: |
| inspireId Identifier Public | -- Definition -- External object identifier of the spatial object. -- Description -- NOTE An external object identifier is a unique object identifier published by the responsible body, which may be used by external applications to reference the spatial object. The identifier is an identifier of the spatial object, not an identifier of the real-world phenomenon. | Default: |
| levelOfDetail MD_Resolution Public [0..1] | -- Definition -- Resolution, expressed as the inverse of an indicative scale or a ground distance. -- Description -- NOTE The object is captured at a scale of this level of detail; rules apply for portrayal and visualisation. | Default: |
| localType LocalisedCharacterString Public [0..1] «voidable» | -- Definition -- Provides 'local' name for the type of surface water. -- Description -- EXAMPLE Canal, channel, ditch, etc. | Default: |
| origin OriginValue Public «voidable» | -- Definition -- Origin of the surface water. -- Description -- SOURCE [Based on EuroRegionalMap]. EXAMPLE Natural, man-made, etc. NOTE 1 Natural surface waters are natural watercourses (e.g. rivers, streams) and standing waters (e.g. pools, lakes) naturally preserved and that have been canalised and / or dammed either for navigation or for preventing flood disaster. NOTE 2 A man-made surface water is a surface water entirely created by man for drainage, storage or transportation purpose, this includes canals, ditches and reservoirs. | Default: |
| persistence HydrologicalPersistenceValue Public «voidable» | -- Definition -- The degree of persistence of water. -- Description -- SOURCE [Based on DFDD]. | Default: |
| tidal Boolean Public «voidable» | -- Definition -- Identifies whether the surface water is affected by tidal water. | Default: |

##### VanishingPoint

Type:		Class    HydroPointOfInterest

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From HY - Hydrography     Keywords:

Detail:		Created on 2008-08-12.  Last modified on 2011-03-14.

GUID:		{655FB31C-A3E9-4306-85E1-4C241B26452A}

-- Definition --

Location where a watercourse disappears into the terrain or vanishes due to anthropization.

-- Description --

NOTE 1 Corresponds to an 'outlet' node in a network view.

NOTE 2 Regarded as a placeholder in Annex II theme 'Geology' due to the connection with groundwater.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. |
|  | isCollection = false. |
|  | noPropertyType = false. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public VanishingPoint | Public HydroPointOfInterest |  |
| Association LinkForNavigation Source -> Destination | Public SwallowHole | Public VanishingPoint |  |

##### WFDWaterBody

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From HY - Hydrography     Keywords:

Detail:		Created on 2008-08-10.  Last modified on 2011-03-14.

GUID:		{711ED7B0-F473-4f80-976A-FE5DF76FB809}

-- Definition --

Abstract class representing a WFD body of surface water or body of groundwater.

-- Description --

NOTE All WFD spatial object types are regarded as candidate spatial objects in Annex III theme 'Area management/restriction/regulation zones and reporting units' due to their primary function for reporting under Directive 2000/60/EC.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: false Default: false Description: Create a property type that requires that the instance is encoded inline (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | inspireConcept = FIXME. Default: FIXME Description: URN reference to the feature concept in the INSPIRE Feature Concept Dictionary Register |
|  | isCollection = false. Values: true ¦ false Default: false Description: Identifies the feature type as a feature collection. |
|  | noPropertyType = false. Values: false Default: false Description: Surpress creation of a standard property type that supports inline or by-reference encoding (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. Values: iso19136_2007 ¦ iso19139_2007 ¦ iso19136_2007_INSPIRE_Extensions Default: iso19136_2007_INSPIRE_Extensions Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public WFDGroundWaterBody | Public WFDWaterBody |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| beginLifespanVersion DateTime Public «lifeCycleInfo» | -- Definition -- Date and time at which this version of the spatial object was inserted or changed in the spatial data set. | Default: |
| endLifespanVersion DateTime Public [0..1] «lifeCycleInfo» | -- Definition -- Date and time at which this version of the spatial object was superseded or retired in the spatial data set. | Default: |
| inspireId Identifier Public | -- Definition -- External object identifier of the spatial object. -- Description -- NOTE An external object identifier is a unique object identifier published by the responsible body, which may be used by external applications to reference the spatial object. The identifier is an identifier of the spatial object, not an identifier of the real-world phenomenon. | Default: |

#### From PS - Protected Sites

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes

Detail:		Created on 2011-03-12. Last modified on 2011-03-12

GUID:		{FEEF0A47-F401-46b1-965B-88B37C278100}

##### Protected Sites Full

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	From PS - Protected Sites

Detail:		Created on 2011-03-14. Last modified on 2011-03-14

GUID:		{ECFA21B2-B647-411e-8447-6AEAF824D494}

###### ProtectedSite

Type:		Class    ProtectedSite

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Protected Sites Full     Keywords:

Detail:		Created on 2009-08-03.  Last modified on 2011-03-14.

GUID:		{11726C9B-5FC5-407c-B0A7-76E7AAC37BE5}

-- Definition --

An area designated or managed within a framework of international, Community and Member States' legislation to achieve specific conservation objectives.

-- Description --

Each protected site has a boundary defined through formal, legal or administrative agreements or decisions. The establishment of a protected site is normally underpinned by legislation and thus given weight in decisions about land use change and spatial planning. Each Site is normally selected as a representative example of a wider resource and selected through a formal criterion based approach. A protected site can be a contiguous extent of land/sea or a collection of discrete areas that together represent a single formal Protected Site.  This class has the attributes, constraints and associations that are part of the Full application schema.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association LinkForNavigation Source -> Destination | Public WFD_GroundWaterBody | Public ProtectedSite | Invariant {If ProtectedSite is dependent on GroudWaterBody -> if ( GrounWaterBody.isAssociatedWithProtectedAreas == truth )} |
| Association LinkForNavigation Source -> Destination | Public HydrogeologicalObjectNatural | Public ProtectedSite | Invariant {if(hydrogeologicalObjectNatural is protected)} |
| Association LinkForNavigation Source -> Destination | Public WFD_ProtectedArea | Public ProtectedSite |  |
| Generalization Source -> Destination | Public ProtectedSite | Public ProtectedSite |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| siteIdentifier SiteIdentifierType Public [0..*] «voidable» | -- Definition -- The identifier for the Site using some national or international identification scheme. -- Description -- This is distinct from the inspireID, which is a unique identifier for the record. | Default: |
| activitiesAndImpacts ActivitiesAndImpactsType Public [0..*] «voidable» | -- Definition -- Impacts resulting from human activities or natural processes that positively or negatively affect the conservation status of the protected site. -- Description -- This information is needed to inform evaluation of conservation status of a Protected Site. This includes management activities such as grazing or cutting, land uses such as mineral extraction or transport and natural processes such as disease fluvial erosion. | Default: |
| spatialResolution MD_Resolution Public [0..1] «voidable» | -- Definition -- The spatial resolution of the protected site geometry. This may relate to a scale of capture value. | Default: |
| siteDescription LocalisedCharacterString Public [0..1] «voidable» | -- Definition -- A general description of the Site and its characteristics. -- Description -- NOTE This attribute corresponds to item 4.1 of the Natura2000 SDF. | Default: |
| qualityAndImportance LocalisedCharacterString Public [0..1] «voidable» | -- Definition -- An overall indication of the quality and importance of the Site, in view of the conservation objectives of the various Directives. -- Description -- NOTE This attribute corresponds to item 4.2 of the Natura2000 SDF. | Default: |
| vulnerability LocalisedCharacterString Public [0..1] «voidable» | -- Definition -- The nature and extent of pressures on the Site from human and other influences and the fragility of habitats and ecosystems found there. -- Description -- NOTE This attribute corresponds to item 4.3 of the Natura2000 SDF. | Default: |
| ownership LocalisedCharacterString Public [0..1] «voidable» | -- Definition -- A general description of the site ownership. -- Description -- NOTE This attribute corresponds to item 4.5 of the Natura2000 SDF. EXAMPLES Private, State, conservation NGO. | Default: |
| documentation LocalisedCharacterString Public [0..1] «voidable» | -- Definition -- References to publications and scientific data concerning the Protected Site. -- Description -- NOTE 1 This attribute corresponds to item 4.6 of the Natura2000 SDF. NOTE 2 Information entered should be made according to standard conventions for scientific references. Unpublished items or communications referring to the information given in the recording form should be included where ever useful. | Default: |
| legalExpiryDate DateTime Public [0..1] «voidable» | -- Definition -- The date that the protected site was legally destroyed. This is the date that the real world object was destroyed, not the date that its representation in an information system was destroyed or changed. | Default: |
| beginLifespanVersion DateTime Public «voidable» | -- Definition -- Date and time at which this version of the spatial object was inserted or changed in the spatial data set. -- Description -- NOTE This date is recorded to enable the generation of change only update files. | Default: |
| endLifespanVersion DateTime Public [0..1] «voidable» | -- Definition -- Date and time at which this version of the spatial object was superseded or retired in the spatial data set. -- Description -- NOTE This date is recorded primarily for those systems which "close" an entry in the spatial data set in the event of an attribute change. | Default: |
| dataSource CharacterString Public [0..1] «voidable» | -- Definition -- The agency or organisation that is responsible for maintaining and providing the data about the Protected Site. This may be represented in the form of the URL or name and address of the organisation. | Default: |
| officialsiteArea Area Public [0..1] «voidable» | -- Definition -- The official area of the site in hectares. This may not the same as area calculated from the geometry. | Default: |
| siteLength Length Public [0..1] «voidable» | -- Definition -- The length of the site, normally used if the area is not populated. | Default: |
| fundingSource FundingSourceType Public [0..*] «voidable» | -- Definition -- The source(s) of financial support that are being used to implement the management plan on a Protected Site. -- Description -- NOTE Funding of management on Protected Sites is critical to securing desired conservation status. The resources are supplied from a variety of sources, ranging from private land owners to European funding schemes. | Default: |
| siteManagementPlan SiteManagementPlanType Public [0..*] «voidable» | -- Definition -- The Site Management Plans that set out practical actions and measures that are needed to ensure that the features for which the site is designated are maintained. | Default: |
| natura2000Respondent CI_ResponsibleParty Public [0..1] «voidable» | -- Definition -- The person responsible for completing Natura2000 reporting on the Site. -- Description -- NOTE This is likely to be a person from the ResponsibeAgency, but may not be. | Default: |
| protectedEntity ProtectedEntityType Public [0..*] «voidable» | -- Definition -- An entity that is protected by a designated protected site (that is, the object or reason for protection). -- Description -- EXAMPLE Such entities may include habitats, species and geological, archaeological, cultural and other types of entities. | Default: |
| presentHabitat PresentHabitatType Public [0..*] «voidable» | -- Definition -- A habitat that exists on the Site. -- Description -- NOTE This is distinct from the Habitats for which the Site is protected. These are represented in the protectedEntity attribute. | Default: |
| timePeriod TM_OrdinalEra Public [0..1] «voidable» | -- Definition -- For historical or archaeological Sites, the era in which the Site is thought to originate. | Default: |

##### Protected Sites Simple

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	From PS - Protected Sites

Detail:		Created on 2011-03-14. Last modified on 2011-03-14

GUID:		{2A09C0E2-C646-443d-BACA-F6BDECCF19CA}

###### ProtectedSite

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Protected Sites Simple     Keywords:

Detail:		Created on 2008-06-17.  Last modified on 2011-03-14.

GUID:		{44976208-3EF0-4f45-BE99-BB985FE62D36}

-- Definition --

An area designated or managed within a framework of international, Community and Member States' legislation to achieve specific conservation objectives.

-- Description --

Each protected site has a boundary defined through formal, legal or administrative agreements or decisions. The establishment of a protected site is normally underpinned by legislation and thus given weight in decisions about land use change and spatial planning. Each Site is normally selected as a representative example of a wider resource and selected through a formal criterion based approach. A protected site can be a contiguous extent of land/sea or a collection of discrete areas that together represent a single formal Protected Site.  This class has the attributes, constraints and associations that are part of the Simple application schema.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | byValuePropertyType = false. Values: false Default: false Description: Create a property type that requires that the instance is encoded inline (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | inspireConcept = FIXME. Default: FIXME Description: URN reference to the feature concept in the INSPIRE Feature Concept Dictionary Register |
|  | isCollection = false. Values: true ¦ false Default: false Description: Identifies the feature type as a feature collection. |
|  | noPropertyType = false. Values: false Default: false Description: Surpress creation of a standard property type that supports inline or by-reference encoding (applies to ISO 19136:2007 encoding rule). Always set to false in INSPIRE. |
|  | xsdEncodingRule = iso19136_2007_INSPIRE_Extensions. Values: iso19136_2007 ¦ iso19139_2007 Default: iso19136_2007 Description: XML Schema encoding rule to apply |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public ProtectedSite | Public ProtectedSite |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| geometry GM_Object Public | -- Definition -- The geometry defining the boundary of the Protected Site. -- Description -- The geometry may be determined by a wide range of methods, including surveying, digitisation or visual reference to natural features or cadastral boundaries and may be defined by the legal document that creates the protected area. The geometry included in a data set that uses this data model is stored as a fixed geometry by coordinates, not by reference to natural, cadastral or administrative boundaries, although it may originally have been defined from these. | Default: |
| inspireID Identifier Public | -- Definition -- External object identifier of the protected site. -- Description -- NOTE An external object identifier is a unique object identifier published by the responsible body, which may be used by external applications to reference the spatial object. The identifier is an identifier of the spatial object, not an identifier of the real-world phenomenon. | Default: |
| legalFoundationDate DateTime Public «voidable» | -- Definition -- The date that the protected site was legally created. This is the date that the real world object was created, not the date that its representation in an information system was created. -- Description -- NOTE In the case of Natura2000 sites, a protected site may go through several different stages (for example, proposed as SCI, confirmed as SCI, designated as SAC). A new version of the site is created for each of these different stages (because there is a change in the designation), and the new version should have the date on which the new stage was legally assigned as the legalFoundationDate (for example, when designated as a SAC, the version for the SAC would have the designation date as the legalFoundationDate). | Default: |
| legalFoundationDocument CI_Citation Public «voidable» | -- Definition -- A URL or text citation referencing the legal act that created the Protected Site. | Default: |
| siteDesignation DesignationType Public [1..*] «voidable» | -- Definition -- The designation (type) of Protected Site. -- Description -- At least one designation is required, but designations may be available using a number of different designation schemes, all of which can be accommodated in the DesignationSchemeValue codelist. | Default: |
| siteName GeographicalName Public [0..*] «voidable» | -- Definition -- The name of the Protected Site. -- Description -- NOTE 1 Several names in different languages may be expressed. NOTE 2 It is recommended that the language of the name (part of the GeographicalName data type) be filled where ever possible. This is an important identifying attribute of a Protected Site. | Default: |
| siteProtectionClassification ProtectionClassificationValue Public [1..*] «voidable» | -- Definition -- The classification of the protected site based on the purpose for protection. -- Description -- The site may have more than one classification. | Default: |

#### From US - Utility and Governmental Services

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_LinksToOtherThemes

Detail:		Created on 2011-03-14. Last modified on 2011-03-14

GUID:		{F4C13003-3A74-4016-A8CD-48E5CA02A209}

##### UtilityNode

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From US - Utility and Governmental Services     Keywords:

Detail:		Created on 2010-10-06.  Last modified on 2011-03-14.

GUID:		{F30BF0E0-B870-4e6c-9835-B10B77160DCE}

-- Definition --

A point spatial object which is used for connectivity.

-- Description --

Nodes are found at both ends of the UtilityLink.

| Constraints |  |
| --- | --- |
|  | All objects have inspireId: (OCL, Status is Proposed) /* All utility nodes have an external object identifier. */ inv:inspireId-&gt;notEmpty() |

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public WaterAppurtenance | Public UtilityNode |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| containerId Identifier Public «voidable» | -- Definition -- External container object identifier of the spatial object. | Default: |
| headnig Angle Public «voidable» | -- Definition -- Heading (rotation with regards to true North) value of an utility network node. -- Description -- Nodes with heading (rotation) ressemble to vectors. For example azimuth of the directed telecommunications antenna . | Default: |

##### WaterAppurtenance

Type:		Class    UtilityNode

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From US - Utility and Governmental Services     Keywords:

Detail:		Created on 2010-10-09.  Last modified on 2011-03-14.

GUID:		{66470B46-CCBC-4053-8C2C-03CCB1CFCA35}

-- Definition --

A node (point) appurtenance in water networks.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public WaterAppurtenance | Public WaterAppurtenanceValue |  |
| Generalization Source -> Destination | Public WaterAppurtenance | Public UtilityNode |  |
| Association LinkForNavigation Source -> Destination | Public ActiveWell (or AbstractionWell) | Public WaterAppurtenance | Invariant {if ( GroundWaterAbstractionWell.waterUse == forDrinking && WaterAppurtenance. type == well )} |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| type WaterAppurtenanceValue Public «voidable» | -- Definition -- Type of the water network appurtenance. | Default: |

##### WaterAppurtenanceValue

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	From US - Utility and Governmental Services     Keywords:

Detail:		Created on 2010-10-12.  Last modified on 2011-03-14.

GUID:		{35D875E5-F8AD-4aa5-AE0A-8A31000C276C}

-- Definition --

Possible node (point) appurtenance values in water networks.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public WaterAppurtenance | Public WaterAppurtenanceValue |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| anode Public | -- Definition -- Anode. -- Description -- An anode is a feature (specifically, an electrical mechanism) that’s applied to system components for the prevention of rust, pitting, and the corrosion of metal surfaces that are in contact with water or soil. A low-voltage current is applied to the water or soil in contact with the metal, such that the electromotive force renders the metal component cathodic. Corrosion is concentrated on the anodes instead of on the associated (and protected) water system components. This type of corrosion may occur in copper, steel, stainless steel, cast iron, and ductile iron pipes. | Default: |
| clearWell Public | -- Definition -- Clear well. -- Description -- A clear well is an enclosed tank that is associated with a treatment plant. Clear wells are used to store filtered water of sufficient capacity to prevent the need to vary the filtration rate with variations in demand. Clear wells are also used to provide chlorine contact time for disinfection. Pumps are used to move the water from the clear well to the treatment plant or to a distribution system. | Default: |
| controlValve Public | -- Definition -- Control valve. -- Description -- Control valves represent set of valves that operate in special ways. There are three fundamental types of control valves: backflow control, air control, and altitude. | Default: |
| fitting Public | -- Definition -- Fitting. -- Description -- The fitting represents the facility found at the joint between two lines where a transition of some sort must occur. The basic connecting devices between pipes; fittings are rarely used to control the flow of water through the network. | Default: |
| hydrant Public | -- Definition -- Hydrant. -- Description -- A hydrant enables fire fighters to attach fire hoses to the distribution network. Hydrants also have secondary uses that include flushing main lines and laterals, filling tank trucks, and providing a temporary water source for construction jobs. | Default: |
| junction Public | -- Definition -- Junction. -- Description -- The junction is a water network node where two or more pipes combine, or a point where water consumption is allocated and defined as demand. | Default: |
| lateralPoint Public | -- Definition -- Lateral point. -- Description -- A lateral point represents the location of the connection between the customer and the distribution system. | Default: |
| meter Public | -- Definition -- Meter. -- Description -- A meter is a facility that is used to measure water consumption (volume). Being a facility, a meter plays the role of a junction on the active network. NOTE Meters are also much like hydrants as they also have an associated warehouse object, namely, a WarehouseMeter. | Default: |
| pump Public | -- Definition -- Pump. -- Description -- A pump is a piece of equipment that moves, compresses, or alters the pressure of a fluid, such as water or air, being conveyed through a natural or artificial channel. NOTE Pump types include AxialFlow, Centrifugal, Jet, Reciprocating, Rotary, Screw, and Turbine. | Default: |
| pumpStation Public | -- Definition -- Pump station. -- Description -- A pump station is a facility for pumping water on the network to transport to another part of the network (lift pump). | Default: |
| samplingStation Public | -- Definition -- Sampling station. -- Description -- A sampling station is a facility that is used for collecting water samples. Sampling stations may be dedicated sampling devices, or they may be other devices of the system where a sample may be obtained. | Default: |
| scadaSensor Public | -- Definition -- SCADA sensor. -- Description -- The SCADA sensor is a feature that’s used to remotely measure the status of network components as part of a supervisory control and data acquisition (SCADA) system. SCADA systems provide alarms, responses, data acquisition, and control for collection and distribution systems. Operators use the SCADA system to monitor and adjust processes and facilities. | Default: |
| storageBasin Public | -- Definition -- Storage basin. -- Description -- A storage basin represents artificially enclosed area of a river or harbor designed so that the water level remains unaffected by tidal changes. | Default: |
| storageFacility Public | -- Definition -- Enclosed storage facility. | Default: |
| surgeReliefTank Public | -- Definition -- Surge relief tank. -- Description -- A surge relief tank is a piece of equipment used to absorb pressure increases in the water system. Surge relief tanks provide a buffer against throttling within the system by accepting water into a tank through a pressure valve. | Default: |
| systemValve Public | -- Definition -- System valve. -- Description -- A system valve is a facility that is fitted to a pipeline or orifice in which the closure member is either rotated or moved transversely or longitudinally in the waterway so as to control or stop the flow. System valves are used to regulate pressure, isolate, throttle flow, prevent backflow, and relieve pressure. NOTE System valve types include Gate, Plug, Ball, Cone, and Butterfly. These specific types may be classified as isolation valves. | Default: |
| thrustProtection Public | -- Definition -- Thrust protection. -- Description -- The thrust protection represents a type of line protector that’s used to prevent pipe movement. Thrust protection is commonly implemented as thrust blocks (masses of concrete material) that are placed at bends and around valve structures. NOTE The types of thrust protection include Anchor, Blocking, Deadman, and Kicker. | Default: |
| treatmentPlant Public | -- Definition -- Treatment plant. | Default: |
| well Public | -- Definition -- Production well. | Default: |

### Hydrogeology_WFD_GWB

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_v.0.4

Detail:		Created on 2011-03-11. Last modified on 2011-04-12

GUID:		{5BCFF3C9-0006-4d3f-926F-0AE2BAF221A5}

This schema was adapted from the Article 5 schema to include the status information required from the revised GWD1 Reporting sheet. The schema has also been adapted to include attributes defined in the Article 5 resubmission reporting sheet for GIS. The detail of the changes to the schema can be located in topic centre web site location http://water.eionet.europa.eu/schemas/dir????????

2 Oct 2008 YGW

Amended namespace and WFD namespace

Hydrogeology_WFD_GWB - (Package diagram)

Created By:	Janusz Michalak on 2011-03-12

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{9BBFB5B6-2A96-4a44-B26A-79CCEC79D5AF}

Figure: 24

WFD_GWB_LinksFromHydrogeology - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-12

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{AD648555-771A-47c0-B39D-F05BC7E98FB7}

Figure: 25

#### <anonymous>

Type:		Note

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology_WFD_GWB     Keywords:

Detail:		Created on 2012-06-28.  Last modified on 2012-06-28.

GUID:		{F6FBDB74-B2A3-4c75-B772-646C8909154A}

Generated from WFD WISE XSD Schemas by reverse engendering

#### WFD_ GWB_Main

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_WFD_GWB

Detail:		Created on 2011-03-11. Last modified on 2011-03-12

GUID:		{089FD6C6-A885-40de-AF45-E84F573D4512}

WFD_GWB_Main - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-10

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{6BCEC830-F0D0-4857-9590-6A3816AB5305}

Figure: 26

##### BackgroundSubstances

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{A4072AC8-C535-47ed-96E1-C85D074D8B5B}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = choice. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public BackgroundSubstances | Public NaturalBackgroundLevel |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| backgroundNaturalSubstancesIndicators GWBackgoundSubstancesCode Public | Select the natural substance(s) or indicator of pollution with a background level from the enumeration list for which threshold values have been established and for which a natural background level is required. Arsenic, Cadmium, Lead, Mercury, Ammonium, Nitrate, Chloride, Sulphate, Conductivity. | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| backgroundOtherNaturalSubstances PF_FreeText Public | Give the name(s) of the natural substance(s) or indicators for any other relevant pollutant for which threshold values have been established and for which a natural background level is required. | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |

##### ChemicalExemption

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{0BCB2ACB-DEA7-43d5-A6A6-509F0D35008E}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public OtherRelevantPollutantExemption | Public ChemicalExemption |  |
| Aggregation Source -> Destination | Public ChemicalExemption | Public ChemicalStatus |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| chemicalExemptionType ObjectivesExemptionType Public | If an exemption is applied to this Groundwater Body in terms of achievement of good chemical status select from the enumeration list which type(s) of exemption apply. More than one may apply to each Groundwater Body. Exemptions are: Article4(4) -Technical feasibility Article4(4) - Disproportionate cost Article4(4) - Natural conditions Article4(5) -Technical feasibility Article4(5) - Disproportionate cost Article4(6) - Natural causes Article4(6) - Force Majeure Article4(6) - Accidents Article4(7) - New modification Article4(7) - Sustainable human development | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| pollutantCausingExemption GWPollutantsCode Public [0..*] «voidable» | CONDITIONAL. Select the pollutant(s) from the enumeration list that are covered by the exemption. | Default: [maxOccurs = unbounded ] [minOccurs = 0 ] |
| exemptionComment PF_FreeText Public «voidable» | Further comment or explanation | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |

##### ChemicalStatus

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{D8184D9F-9804-4609-A204-1828A15F401A}

Detail required to produce the following maps:

Map 2: Achievement/exceedance of standard for nitrates (value in Annex 1 of GWD or set according to paragraph 3 of Annex 1 GWD, and according to status assessment procedure in Article 4 of GWD);

Map 3: Achievement/exceedance of standard for pesticides (combined total and individual value in Annex 1 of GWD or set according to paragraph 3 of Annex 1 GWD, and according to status assessment procedure in Article 4 of GWD);

Map 4: Achievement/exceedance of threshold values set by Member States for other pollutants (considering in this category the list of substances as contained in Part B of Annex II of GWD and more generally any other pollutants contributing to the characterisation of Groundwater Bodies as being 'at risk', and according to status assessment procedure in Article 4 of GWD).

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public ChemicalExemption | Public ChemicalStatus |  |
| Aggregation Source -> Destination | Public ChemicalStatus | Public GroundwaterBodyStatus |  |
| Aggregation Source -> Destination | Public NaturalBackgroundLevel | Public ChemicalStatus |  |
| Aggregation Source -> Destination | Public PollutantsCausingFailure | Public ChemicalStatus |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| chemicalStatusValue GWStatusCode Public | 2=good, 3=poor, U=unknown | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| commentChemicalStatusValue PF_FreeText Public «voidable» | Further comment or explanation | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| reasonsForFailure GWChemicalFailureCode Public [0..*] «voidable» | CONDITIONAL. If the Groundwater Body is of Poor Status (3) select reasons for failure from the enumeration list. Reasons are: a) Saline or other intrusions; b) Exceedance of one or more quality standard or threshold value; c) Failure to meet environmental objectives in associated surface water bodies or significant diminution of the ecological or chemical status of such bodies d) Significant damage to terrestrial ecosystems which depend directly on the groundwater body e) Deterioration in quality of waters for human consumption. f) Significant impairment of human uses; g) Significant environmental risk from pollutants across the Groundwater Body | Default: [maxOccurs = unbounded ] [minOccurs = 0 ] |

##### GWProtectedAreaDetails

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-14.

GUID:		{3ADBBA6C-A6CD-438f-ADEF-30AA2C6A82D4}

CONDITIONAL. If there are Protected Areas associated with this Water Body (PROT_AREA_ASSOC is "Yes") then indicate here the types of Protected Area associated with the Water Body.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public GWProtectedAreaDetails | Public StatusProtectedAreas |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| typeOfProtectedArea ProtectedAreaGWType Public | Indicate the type of Protected Area as defined in the enumeration list. This should be repeated for each type of Protected Area associated with the water body. This comprises Birds Habitats Nitrates Article 7 Waters bodies used for the abstraction of drinking water European other National Local | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| protectedAreaCode FeatureUniqueEUCodeType Public [1..*] | Input each previously reported Protected Area code (i.e. previously reported, before March 2010, under other directives). For each newly reported Protected Area input a code and use PROT_AREA schema to provide details. | Default: [maxOccurs = unbounded ] [minOccurs = 1 ] |
| typeOfAssociation ProtAreaAssociationType Public | Choose from the enumeration list the type of association (water bodies and protected areas will not always overlap 1:1. Protected Areas may include more than water body and a water body may overlap more than one Protected Area): a) water body within a Protected Area, b) water body overlapping (partly within) a Protected Area or c) water body dynamically connected to a Protected Area e.g. river/groundwater | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| valueStatusProtectedArea SWStatusProtAreaCode Public «voidable» | CONDITIONAL: In general reporting is required for all protected areas except for areas designated under nitrates Directive, if status is reported under those other directives and a link can be made with the protected area Code. Reporting is in any case required for all protected areas WFD Article 7 Protected Areas as these are not reported under any other directive. See further explanatory text in the Schema User Guide Section 2.2.5 for reporting the status of Article 7 and other Protected Areas. | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| commentValueStatusProtArea PF_FreeText Public «voidable» | CONDITIONAL: For WFD Article 7 Protected Areas, indicate the reasons for failure to reach Good status and the description of the Quality Elements/determinands that fail. In case of protected areas designated under Habitats or Birds Directives provide information about the habitats or species that are causing the failure. Further guidance on what should be reported here is given in section 2.2.5 of the user guide. | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| protAreaExemptions ObjectivesExemptionType Public [0..*] «voidable» | Report Exemptions from the enumeration list. Exemptions are Article4(4) -Technical feasibility Article4(4) - Disproportionate cost Article4(4) - Natural conditions Article4(5) -Technical feasibility Article4(5) - Disproportionate cost Article4(6) - Natural causes Article4(6) - Force Majeure Article4(6) - Accidents Article4(7) - New modification Article4(7) - Sustainable human development | Default: [minOccurs = 0 ] [maxOccurs = 10 ] |
| protAreaExemptionComment PF_FreeText Public «voidable» | Further comment or explanation | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |

##### GrounWaterBodyDescription

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{1446FD3F-0B89-4d89-A1FF-563592700B13}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public HydrogeologicalCharacteristics | Public GrounWaterBodyDescription |  |
| Aggregation Source -> Destination | Public GroundwaterBodyStatus | Public GrounWaterBodyDescription |  |
| Aggregation Source -> Destination | Public PressuresAndImpacts | Public GrounWaterBodyDescription |  |
| Aggregation Source -> Destination | Public StatusProtectedAreas | Public GrounWaterBodyDescription |  |
| Aggregation Source -> Destination | Public GrounWaterBodyDescription | Public WFD_GroundWaterBody |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| countruCode CountryCode Public | Two-letter ISO country code | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| EU_RBD_code FeatureUniqueEUCodeType Public | Unique EU code of the parent River Basin District. Add the two-letter ISO Country code followed by the Member State unique id | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| URL anyURI Public «voidable» | URL for integration of your own internet-based information | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| creationDate dateTime Public |  | Default: [use = required ] |
| creator PF_FreeText Public |  | Default: [use = required ] |
| email styring Public «voidable» |  | Default: [use = optional ] |
| description PF_FreeText Public «voidable» |  | Default: [use = optional ] |
| generatedBy PF_FreeText Public «voidable» |  | Default: [use = optional ] |
| MD_classificationCode DataConfidentialityClassificationCode Public «voidable» |  | Default: [use = optional ] |

##### GroundwaterBodyStatus

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{A0E12298-49DD-4fc4-B822-AA74F360ED96}

Groundwater Quantitative and Groundwater Chemical status are to be reported

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public GroundwaterBodyStatus | Public GrounWaterBodyDescription |  |
| Aggregation Source -> Destination | Public SignificantUpwardTrends | Public GroundwaterBodyStatus |  |
| Aggregation Source -> Destination | Public QuantitativeStatus | Public GroundwaterBodyStatus |  |
| Aggregation Source -> Destination | Public ChemicalStatus | Public GroundwaterBodyStatus |  |
| Aggregation Source -> Destination | Public SignificantTrendReversal | Public GroundwaterBodyStatus |  |

##### HydrogeologicalCharacteristics

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{FAAE2FAD-6197-4d97-B25E-0C4E4D73690B}

Provide the relevant hydrogeological aspects used to characterise the Groundwater Body.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public HydrogeologicalCharacteristics | Public GrounWaterBodyDescription |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| isLayered boolean Public «voidable» | Is the Groundwater Body layered? Y=Yes, N=No | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| area double Public «voidable» | Surface area of the Groundwater Body in km2 | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| scale ScaleType Public «voidable» | Reference map scale at which the area or length of the Water Body was calculated. Express as 1:nnnnnn e.g. 1:50,000. | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| scaleExplanation PF_FreeText Public «voidable» | CONDITIONAL. Provide brief explanation of the scale used if necessary. | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| averageDepth double Public «voidable» | Average depth from surface to Groundwater Body in metres. | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| averageThickness double Public «voidable» | Average thickness of Groundwater Body in metres. | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| depthRange GWDepthRangeCode Public «voidable» | Assignment to a depth range at the point where the main part of the Groundwater Body is situated (depth ranges: 0-20m, 20-50m, 50-200m, >200m) | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| areLinkedSurfaceWaterBodies boolean Public «voidable» | Is the Groundwater Body dynamically linked to Surface Water Bodies? Y=yes, N=No | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| areLinkTerrestrialEcosystems boolean Public «voidable» | Is the Groundwater Body dynamically linked to terrestrial ecosystems? Y=yes, N=No | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| geologicalFormation GWGeologicalFormationCode Public «voidable» | What is the main geological formation of the Aquifer Type? Select one from the enumeration list Porous - highly productive, Porous - moderately productive, Fissured aquifers including karst - highly productive, Fissured aquifers including karst - moderately productive, Insignificant aquifers - local and limited groundwater | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| verticalOrientation GWVerticalOrientationCode Public «voidable» | What is the vertical orientation of the Groundwater Body? Please select one from the enumeration list and refer to the "GIS guidance on reporting of groundwater bodies under WFD and GWD" for pictures of the options. Horizontal, mainly continuous body; Horizontal, mainly smaller, individual formations; Declining, mainly continuous body; Declining, mainly smaller, individual formations; Declining and cumulative, mainly continuous body; Declining and cumulative, mainly smaller, individual formations; Boat form, mainly continuous body; Smaller individual boat formed formations; Other | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| capacity double Public «voidable» | What is the volume of the aquifer in 1000m3. | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |

##### NaturalBackgroundLevel

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{BE31E62E-82BC-400d-A0A3-5AC19BA768BC}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public BackgroundSubstances | Public NaturalBackgroundLevel |  |
| Aggregation Source -> Destination | Public NaturalBackgroundLevel | Public ChemicalStatus |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| concentrationLevel double Public | Numeric value of the natural background level. | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| concentrationUnits GWConcentrationUnitsCode Public | Select the relevant units for the natural background concentrations/levels. Units are mg/l, µg/l, ng/l and Sm-1 (the reporting unit of conductivity is milli Siemens per metre) | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |

##### OtherRelevantPollutant

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{B88C9992-B470-4f0e-984D-6B6016FFC468}

Please give the name of the pollutant or indicator of pollution. Please give CAS number of the pollutant if available and/or relevant

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public UpwardTrendPollutant | Public OtherRelevantPollutant |  |
| Dependency Source -> Destination | Public TrendReversalPollutant | Public OtherRelevantPollutant |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| otherRelevantPollutantCASNumber ChemicalCASNumberType Public |  | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| otherRelevantPollutantName PF_FreeText Public |  | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |

##### OtherRelevantPollutantExceedance

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{723FC574-7321-49d0-B120-E8BF2186523C}

Please give the name of the other pollutant(s) or indicator(s) of pollution which is/are causing the exceedance of national standards or threshold values. Please give CAS number of the pollutant if available and/or relevant

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public PollutantsCausingFailure | Public OtherRelevantPollutantExceedance |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| otherRelevantPollutantCASNumber ChemicalCASNumberType Public |  | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| otherRelevantPollutantName PF_FreeText Public |  | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |

##### OtherRelevantPollutantExemption

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{0DD4A8BE-1E0C-4392-896D-11635C1BB3E6}

CONDITIONAL. Input the details of the other relevant pollutants that are covered by the exemption. Please give CAS number of the pollutant if available and/or relevant

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public OtherRelevantPollutantExemption | Public ChemicalExemption |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| otherRelevantPollutantCASNumber ChemicalCASNumberType Public |  | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| otherRelevantPollutantName PF_FreeText Public |  | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |

##### PollutantsCausingFailure

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{D3AB650B-EC16-4605-BAE5-7C03D5F0F313}

CONDITIONAL. If the Groundwater Body is of Poor Status define each of the pollutants or indicators of pollution that have caused the Groundwater Body to fail to reach good status

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = choice. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public PollutantsCausingFailure | Public OtherRelevantPollutantExceedance |  |
| Aggregation Source -> Destination | Public PollutantsCausingFailure | Public ChemicalStatus |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| pollutantCausingFailure GWPollutantsCode Public | Please select the pollutant(s) or indicator(s) of pollution that is/are causing the failure of the Groundwater Body to reach good chemical status. | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| otherRelevantPollutantExceedance OtherRelevantPollutantExceedance Public |  | Default: |

##### PressuresAndImpacts

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{CE8FA02D-039E-46f5-A99D-EE9DEBD07A93}

Describe the significant sources of pressures on Groundwater Bodies and a summary of the human impacts

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public PressuresAndImpacts | Public GrounWaterBodyDescription |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| significantPresureTypes GWPressureType Public [0..*] «voidable» | CONDITIONAL. Only required where significant pressures exist. Select the relevant and significant pressure(s) (if any) from the enumeration list. See Guidance Document No.21 'Guidance for reporting under the Water Framework Directive' for further information. | Default: [maxOccurs = unbounded ] [minOccurs = 0 ] |
| otherPressureDescription PF_FreeText Public «voidable» | Only to be used if the Pressure Type is not in the enumeration list | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| significantImpactTypes GWImpactTypes Public [0..*] «voidable» | CONDITIONAL. Give a review of the human impacts on Groundwater Bodies to support the determination of chemical and quantitative status according to WFD Annex 5. NOTE: 2010 data should be an update of that already reported in WISE. Select the relevant human impacts form the enumeration list. See Guidance Document No.21 'Guidance for reporting under the Water Framework Directive' for further information. | Default: [maxOccurs = unbounded ] [minOccurs = 0 ] |
| otherImpactDescription PF_FreeText Public «voidable» | Only to be used if the Impact Type is not in the enumeration list | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |

##### QuantitativeStatus

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-14.

GUID:		{D0C9EA5C-379C-42cd-9651-759C91CA1E08}

Map 1: Quantitative status – Identification of bodies that are at “good quantitative status” and those at “poor quantitative status”

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public QuantitativeStatus | Public GroundwaterBodyStatus |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| quantitativeStatusValue GWStatusCode Public | 2=good, 3=poor, U=Unknown | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| commentQuantitativeStatusValue PF_FreeText Public «voidable» | Further comment or explanation | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| reasonsForFailure GWQuantitativeFailureCode Public [0..*] «voidable» | CONDITIONAL. If Groundwater Body is of Poor Status select reasons for failure from the enumeration list. The reason(s) for failing good quantitative status are: a) Exceedance of available groundwater resource by long-term annual average rate of abstraction that may result in a decrease of groundwater levels; b) Failure to achieve environmental objectives (Article 4 WFD) for associated surface waters; c) Significant diminution of the status of surface waters; d) Significant damage to terrestrial ecosystems directly depending on groundwater; e) Saline or other intrusion | Default: [minOccurs = 0 ] [maxOccurs = unbounded ] |
| goodQuantitativeStatusExemption ObjectivesExemptionType Public [0..*] «voidable» | CONDITIONAL. If exemptions apply to the achievement of good quantitative status for this Groundwater Body, please select the reason(s) for the exemption(s) from the enumeration list (more than one may apply to each Groundwater Body). Exemptions are: Article4(4) -Technical feasibility Article4(4) - Disproportionate cost Article4(4) - Natural conditions Article4(5) -Technical feasibility Article4(5) - Disproportionate cost Article4(6) - Natural causes Article4(6) - Force Majeure Article4(6) - Accidents Article4(7) - New modification Article4(7) - Sustainable human development (NOTE: This exemption is not relevant for GWB but has been left in the list for simplicity of schema construction). | Default: [maxOccurs = 10 ] [minOccurs = 0 ] |
| goodQuantitativeStatusExemptionComment PF_FreeText Public «voidable» | Further comment or explanation | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |

##### SignificantTrendReversal

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{58A8D978-1385-449d-9B90-427D03284F47}

Map 5: Trends - Identification of: groundwater bodies in which trends have been reversed

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public TrendReversalPollutant | Public SignificantTrendReversal |  |
| Aggregation Source -> Destination | Public SignificantTrendReversal | Public GroundwaterBodyStatus |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| trendReversal boolean Public | Yes or No. If Yes, the pollutant/indicator of pollution that is showing a trend reversal MUST be reported | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| trendReversalComment PF_FreeText Public «voidable» | Provide extra detail if relevant | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |

##### SignificantUpwardTrends

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-14.

GUID:		{7A6A00B3-77A7-4c12-AF25-3D6304D24F38}

Map 5: Trends - Identification of Groundwater Bodies with environmentally significant and sustained upward trends in pollutant concentrations

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public SignificantUpwardTrends | Public GroundwaterBodyStatus |  |
| Aggregation Source -> Destination | Public UpwardTrendPollutant | Public SignificantUpwardTrends |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| upwardTrend boolean Public «voidable» | Yes or No or Unknown. If Yes, the pollutant/indicator of pollution that is showing a significant and sustained upward trend MUST be reported | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| upwardTrendComment PF_FreeText Public «voidable» | Provide extra detail if relevant | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |

##### StatusProtectedAreas

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-14.

GUID:		{AD022BC1-BB37-4e4d-9024-6EFE7E94618B}

Data for Map 3: Status for Protected Areas (see page 13 CIS Guidance document No. 21).

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public StatusProtectedAreas | Public GrounWaterBodyDescription |  |
| Aggregation Source -> Destination | Public GWProtectedAreaDetails | Public StatusProtectedAreas |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| isAssociatedWithProtectedAreas boolean Public «voidable» | This part of the schema deals with Protected Areas, in particular: those covered by Article 7: water bodies used for the abstraction of drinking water which are not covered under any other directives. In addition: those Protected Areas that have NOT been reported under other directives before March 2010. Is the Water Body associated with a Protected Area? Answer Yes/No/Unknown | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |

##### TrendReversalPollutant

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{BD3B09FF-BCA6-42b5-9932-2455F7E7543F}

CONDITIONAL. Required if TrendReversal is Yes. List the pollutant(s) or indicator(s) of pollution for which a significant trend reversal has/have been identified.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = choice. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public TrendReversalPollutant | Public SignificantTrendReversal |  |
| Dependency Source -> Destination | Public TrendReversalPollutant | Public OtherRelevantPollutant |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| pollutant GWPollutantsCode Public | Choose from enumeration list. The list has been derived from the Groundwater Directive 2006/118/EC with an indicative list of pesticides. | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| otherReleventPollutant OtherReleventPollutan Public |  | Default: |

##### UpwardTrendPollutant

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_ GWB_Main     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{860B69AF-83BA-41f6-BDD3-5BE8E113713F}

CONDITIONAL. Required if UpwardTrend is Yes. Identify the pollutant(s) or indicator(s) of pollution that is/are causing the significant and sustained upward trend.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = choice. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Dependency Source -> Destination | Public UpwardTrendPollutant | Public OtherRelevantPollutant |  |
| Aggregation Source -> Destination | Public UpwardTrendPollutant | Public SignificantUpwardTrends |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| pollutant GWPollutantsCode Public | Choose from enumeration list. The list has been derived from the Groundwater Directive 2006/118/EC with an indicative list of pesticides. | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| otherReleventPollutant OtherReleventPollutant Public |  | Default: |

#### WFD_GWB_EnumerationsAndSimpleTypes

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_WFD_GWB

Detail:		Created on 2011-03-11. Last modified on 2011-03-15

GUID:		{F2FF2DAF-B5C5-4c73-9D28-E52133EEA8B9}

This schema defines the common types used by  WFD reporting schemas. It is a reference schema only and does not have its own instance.

History:

17 Mar 2005    YGW

Added the following

String250Type                    Mandatory String of up to 250 characters

DecimalType                        Decimal with 3 significant places

PercentageType                    Percentage with 3 significant decimal places

YesNoCode                        Yes/No Code

YesNoUnknownCode            For Yes, No or Unknown/Not Applicable

CoordinateType                    Format Degrees Minutes, Decimal fraction of minutes (-)DDMM,MM

ScaleType                        Format for Scale: 1:nnnnnnnn

AltitudeTypeCode                Code for Altitude types (NOT USED)

ConfidenceLevelCode            Confidence Level (NOT USED)

RiskStatusCode                    Codes for Risk status

WaterBodyTypeCode            All water body types

SurfaceWaterBodyTypeCode    SurfaceWater Type

SystemTypeCode                System Type

DesignationCode                    Designation Code

String2000Type                    Type to enter string of 1 to 2000 characters

ImportanceCode                    Importance

EcoregionCode                    Codes for Ecoregions as defined in Directive

CostRecoveryLevelCode            Codes for cost recovery levels

ProtectedAreaCode                Codes for Protected area types (NOT USED)

PollutantCategoryCode            Category Codes for the Priority Substances and Annex VIII  pollutant groups. Codes 1 -12 match categories provided in AnnexVIII of the WFD

ClassificationCode                Codes for data security classification - relevant ISO 19115 terms

NitrogenMethodCode            Codes for methods of Nitrogen measurement

20 May 2005    YGW

Deprecated type definitions moved to end of file

CountryCodeType                Deprecated

NationalInternationalType        Deprecated

MSCodeType                        Deprecated

YesNoType                        Deprecated

ProtectedAreaCode                Deprecated

CountryCode                        2 character Country codes

NationalInternationalCode        Code for national or international

Changed name WaterBodyTypeCode to WaterBodyCategoryCode

Changed name SurfaceWaterBodyTypeCode to SurfaceWaterBodyCategoryCode

Removed element PollutantCategoryCode as can be inferred from the CAS number

CoordinateType pattern changed to (-)dd.ddddd

HR added to CountryCode and MSCode Lists

23 Oct 2005 YGW

Lists amend to cater for situations where values are Not Yet Determined, or Not Applicable

ConfidenceLevelCode             Added U (Unknown) and UM (yet to be measured)

NitrogenMethodCode            Added OT (Other), U (Unknown)

RiskStatusCode                    Added 0 (Yet to be measured), 9 (Unknown)

SystemTypeCode                Added NA (Not Applicable)

YesNoUnknown                    Added NA (not applicable)

ExceptionTypes                    Added to define numerical codes: -9999 (Unknown), -8888 (Yet to be measured) -7777 (not applicable)

DecimalBaseType                Added base type to define non-negative decimal value

DecimalType                    Amended to allow numerical codes for exceptions

PercentageBaseTYpe            Added base type to define percentage

PercentageType                    Amended to allow numerical codes for exceptions

NonNegativeIntegerType            Added to allow non-negative integer with options for exceptions

11 nov 2005

Amended coordinate type to the . character in the coordinate type element because . means any character

25 Nov 2005

CoordinateType                    amended to allow greater flexibility

RiskStatusType                    Amended to include 9 - Unknown, 0 - Yet to be                                                                 determined

20 Mar 2005    YGW

CountryCode                    removed JR (used for test purposes only)

added GR - Greece

10 Jun 2006        YGW

FlowDir                            amended to include U - Unknown

26 Jun 2006        YGW

EcoRegionCode                    added 26 Cyprus

also added 27, 28, 29 to cater for the French DOM                                                                 (Guyane, Guadeloupe, Martinique, Reunion). These have                                                                 yet to be decided

--------------------------------------------------------------------------------------------------------------------------------------------------

Amendments to support Article 8 (Monitoring) reporting requirements

07 Jul 2006        YGW

QualityElementCode                added codes for Quality Elements

WellOrSpringCode                added codes to deteremine if a GW station is a well, spring                                                             or other

InvestigativeMonitoringTypeCode    added codes for types of investigative monitoring

31 Aug 2006    YGW

GroundwaterElementCode        Codes for groundwater element/parameter identifiers

DepthCode                        Codes for depths at which groundwater monitoring occurs

WellOrSpringCode                Codes to identify if a GW station is a well, spring or other

SubsiteTypeCode                Codes to describe the type of subsites

InternationalMonitoringNetworkCode    Codes for key international monitoring networks

ProgrammeLevelCode            Codes for level at which the programme is designed

ReportingLevelCode                Codes to describe the reporting level

---------------------------------------------------------------------------------------------------------------------------------------------------

Amendments following article 8 testing

15/11/2006            YGW

QualityElementCode                added QE3 (missing)

InternationalMonitoringNetworkCode    added new to list

21/01/2007            YGW

QualityElementCode                added QE2-6-3 - structure of intertidal zone

GroundwaterElementCode        added GE1-1 - groundwater level

added GE1-2 - groundwater yield

------------------------------------------------------------------------------------------------------------------------------------------------

Article 13 RBMP additions

November 2006        JDC

String5000Type                    added to support RBMP submissions

YesNoNotApplicableCode        added to support RBMP submissions

AssociatedReferenceStructure    added to support RBMP submissions

SWEcologicalStatusTypeCode    added to supoort surface water body status reporting

SWChemicalStatusTypeCode    added to supoort surface water body status reporting

GWStatusTypeCode                added to supoort groundwater body status reporting

------------------------------------------------------------------------------------------------------------------------------------------------

Streamlining additions

August 2008        JDC

CASNumberType                added to support classification and status submission

StatusOrPotentialType            added to support status submission

HeavyMetalTypeCode            added to support status submission

PesticidesTypeCode                added to supportstatus submission

PrioritySubstancesTypeCode    added to support classification and status submission

OtherPollutantsTypeCode        added to support classification and status submission

RiverIntercalibrationTypeCode    added to support classification and status submission

LakeIntercalibrationTypeCode    added to support classification and status submission

TransitionalIntercalibrationTypeCode    added to support classification and status submission

CoastalIntercalibrationTypeCode    added to support classification and status submission

ProtectedAreaType                added to support status submission

ExemptionType                    added to support status submission

EQSConfidenceType            added to support status submission

SWPointSourcePressureType    added to support pressure and status submission

SWDiffuseSourcePressureType    added to support pressure and status submission

SWAbstractionPressureType    added to pressure and status submission

SWWaterFlowMorphType        added to support pressure and status submission

SWOtherPressureType            added to support pressure and status submission

GWPointSourcePressureType    added to support pressure and status submission

GWDiffuseSourcePressureType    added to support pressure and status submission

GSWAbstractionPressureType    added to support pressure and status submission

GWArtificialRechargeType        added to support pressure and status submission

GWIntrusionType                added to support pressure and status submission

GWImpactType                    added to support pressure and status submission

GWClassificationType            added to support classification submission

BasicMeasuresType                added to support POM submission

OtherBasicMeasuresType        added to support POM submission

HowMeasuredType                added to support Classification submission

PrioritySubstanceAndCasNumberType    added to support Classification submission

------------------------------------------------------------------------------------------------------------------------------------------------

Changes arising from final review by the Commission and Member States based on 30 March 2009 meeting.

April 2009        JDC

1. Element renaming following JRR and MS comments

BasicMeasyresType                renamed to            MeasuresBasicType

CASNumberType                renamed to            ChemicalCASNumberType

ChemicalMeasurementType        renamed to            ChemicalMatrixType

ClassificationCode                renamed to            DataConfidentilityClassificationCode

CoastalBilogicalQEType            renamed to            QECoastalBiologicalCode

CoastalHydromorphQEType        renamed to            QECoastalHydromorphCode

CoastalIntercallibrationTypeCode    renamed to            TypologyCoastalIntercallibrationCode

CodeType                        renamed to            FeatureUniqueCode

CoordinateType                    renamed to            GeoCoordinateType

DecimalBaseType                renamed to            NumberDecimalBaseType

DecimalCostType                renamed to            CostDecimalType

DecimalType                    renamed to            NumberDecimalType

DepthCode                        renamed to            MonitoringDepthCode

EQSConfidenceType            renamed to            StatusConfidenceType

EUCode    Type                    renamed to            FeatureUniqueEUCodeType

ExceptionType                    renamed to            NumberExceptionType

ExemptionType                    renamed to            ObjectivesExemptionType

GroundwaterElementCode        renamed to            GECode

GWClassificationType            renamed to            GWPollutantType

GWStatusCodeType                renamed to            GWStatusCode

GWClassificationType            renamed to            GWPollutantType

GWQuantativeFailureCode         renamed to             GWQuantitativeFailureCode

HowMeasuredType                renamed to            MethodLoadType

InternationalMonitoringNetworkCode    renamed to        MonitoringInternationalNetworkType

InvestigativeMonitoringTypeCode    renamed to            MonitoringInvestigativeCode

LakeHydromorphQEType        renamed to            QELakeHydromorphCode

LakeIntercallibrationTypeCode    renamed to            TypologyLakeIntercallibrationCode

NonNegativeIntegerType            renamed to            NumberNonNegativeIntegerType

NonPriorityPollutantCode            renamed to            ChemicalNonPrioritySubstancesCASCode

OtherBasicMeasureType        renamed to            MeasureBasicOtherType

OtherPhysioChemQEType        renamed to            QEPhysicoChemOtherType

PercentageBaseType            renamed to            NumberPercentageBaseType

PercentageType                    renamed to            NumberPercentageType

PrioritySubstancesAndCasNumberType    renamed to    ChemicalPrioritySubstanceType

ProgrammeLevelCode            renamed to            MonitoringProgrammeLevelCode

QualityElementCode                renamed to            QECode

ProtectedAreaTypeCode            now use            ProtectedAreaType

RiverHydromorphQEType        renamed to            QERiverHydromorphCode

RiverIntercallibrationTypeCode    renamed to            TypologyRiverIntercallibrationCode

RiverLakeBiologicalQEType        renamed to            QERiverLakeBiologicalCode

RiverPhysioChemQEType        renamed to            QEPhysicoChemRiverType

SubsiteTypeCode                renamed to            MonitoringSubsiteCode

SurfaceWaterBodyCategoryCode     renamed to        SWCategoryCode

SWStatusChemicalCode            renamed to            SWStatusChemicalCode

SWBilogicalStatusTypeCode        now use            SWStatusEcologicalCode (no N option)

SWEcologicalStatusTypeCode    renamed to            SWStatusEcologicalCode

SWGeneralChemicalStatusTypeCode      renamed to    SWStatusGeneralPhysicoChemicalCode

SWHydromorphStatusTypeCode    renamed to            SWStatusHydromorphCode

SWNonPriorityChemicallStatusType    renamed to        SWStatusNonPrioritySubstancelCode

SWPressuresRequiringSupplementaryOrAdditionalMeasures    simplified to     SWPressureType

TransitionalBiologicalQEType        renamed to            QETransitionalBiologicalCode

TransitionalHydromorphQEType    renamed to            QETransitionalHydromorphCode

TransitionalIntercallibrationTypeCode    renamed to        TypologyTransitionalIntercallibrationCode

WellOrSpringCode                renamed to            GWWellOrSpringCode

2. Rationalisation of QE and GE enumeration list elelments

All QE enumeration lists have been changed to QE number plus the description e.g. QE1-1 Phytoplankton.

Ditto GE enumeration lists.

All QE elelments are now in common and include QECode, RiverQECode, LakeQECode, TransitionalQECode,                 CoastalQECode, QEcologicalCode, QEBiologicalRiverCode, QEBiologicalLakeCode, QEBiologicalTransitionalCode,             QEBiologicalCoastalCode, QEHydrologicalRiverCode, QEHydrologicalLakeCode, QEHydrologicalTransitionalCode and             QEHydrologicalCoastalCode.

A new single consolidated QE3-1ParameterType element has been added to replace the QE3-1-1MeasurementType to

QE3-1-6MeasurementType elements.

The following QE Enumeration list elements were no longer required following restructuring; QE3-1-1MeasurementType,

QE3-1-2MeasurementType, QE3-1-3MeasurementType, QE3-1-4MeasurementType, QE3-1-5MeasurementType,

QE3-1-6MeasurementType, RiverPhysioChemQEType, OtherPhysioChemQEType and ChemicalMeasurementType.

3. Elements consolidated into a single enumeration list.

GWPollutantsCode - new element created as a single enumeration list of GWIndividualPesticideCode and                 GWAnnexIIPollutantsCode. Each item being prefixed by a hierarchy code of 1, 1.1, 1.2 etc. New level 1 entries were             created.

ChemicalAllSubstancesType - new element created as a single enumeration list of HeavyMetalType, PesticidesType,             PrioritySubstancesOrganicType and OtherPollutantType. Each item being prefixed by a hierarchy code of 1, 1.1, 1.2 etc.             New level 1 entries were created. Also 3.3 Pentabromodiphenylether was changed to 3.3 Brominated diphenylether.

SWPressuresType - new element created as a single enumeration list of SWAbstractionPressureType,                 SWBiologicalExceedenceStatusTypeCode, SWBiologicalStatusTypeCode, SWDiffuseSourcePressureType,                 SWOtherPressureType, SWPointSourcePressureType and SWWaterFlowMorphType. A two level enumeration list was             created prefixed by a hierarchy number 1, 1.1, 1.2, 1.2.1, 1.2.2 etc. New level 1 entries were created. The list was also             changed to reflect the new definitions within the Reporting Sheet.

Note that

SWAbstractionPressureType is retained but renamed SWPressureAbstractionType

An element called SWPressureAggregatedType of all SW level 1 pressures has been added

GWPressuresType - new element created as a single enumeration list of GWAbstractionPressureType,                 GWArtificialRechargeType, GWDiffuseSourcePressureType, GWIntusionType, GWPointSourcePressureType,

A two level enumeration list was created prefixed by a hierarchy number 1, 1.1, 1.2, 1.2.1, 1.2.2 etc. New level 1 entries         were created.

Note that

GWAbstractionPressureType is retained but renamed GWPressureAbstractionType

GWArtificialRechargeType is retained but renamed GWPressureRechargeType

An element called GWPressureAggregatedType of all GW level 1 pressures has been added

4. Elements removed after consolidating separate enumeration lists into one

HeavyMetalType, PesticidesType, PrioritySubstancesOrganicType and OtherPollutantType into                     ChemicalAllSubstancesType.

SWAbstractionPressureType, SWBiologicalExceedenceStatusTypeCode,     SWBiologicalStatusTypeCode,                 SWDiffuseSourcePressureType, SWOtherPressureType, SWPointSourcePressureType and SWWaterFlowMorphType,

GWAbstractionPressureType, GWArtificialRechargeType, GWDiffuseSourcePressureType, GWIntusionType and             GWPointSourcePressureType.

IndividualPesticideCode and AnnexIIPollutantsCode.

5. Elements whose enumeration lists have been altered

All QE enumeration lists have been changed to QE number plus the description e.g. QE1-1 Phytoplankton.

Ditto GE enumeration lists.

CountryCode - GR changed to EL for Greece.

ProtectedAreaType - UWWT, Drinking and Abstraction have been removed from the enumeration list. The item "Article 7"             has been changed to "Article 7 - abstraction for drinking water"

SWPressureType changed to reflect the new requirements within the Reporting Sheet. The ful list is defined in teh             annotation associated with SWPressureType. Effectively all codes have changed as they have been prefixed with a             hierarchy.

ChemicalPrioritySubstanceType CAS number now prefixes substance name.

TypologyRiversIntercalibrationCodes - the following codes were removed R-E3, R-E5, R-E6, R-M3, R-N2, R-N7 and R-N9

TypologyLakesIntercalibrationCodes - the following codes were removed L-A3 and L-M1

TypologyTransitionalntercalibrationCodes - the following codes were removed TW-M5, TW-M6 and TW-M7

TypologyCoastalIntercalibrationCodes - the following codes were removed CW-M1, CW-M2, CW-M3 and CW-M4

TypologyCoastalIntercalibrationCodes - the following codes were added NEA1/26e

New elelment SWStatusProtAreaCode added. Enumeration list is as follows 1=high, 2=good, 3=failing, U=unknown

6. Elements added to common

ProtectedAreaSWType - Same as ProtectedAreaType but with UWWT removed.

ProtectedAreaGWType - The list is the same as ProtectedAreaType but with UWWT as an extra entry in the enumeration         list

GWPollutantAnnexI-IIType - a subset of GWPollutantType used in GWMethods GWClassification element                 PolluntantOrIndicator

7. Elements added to common which had previously been described as enumeration lists in separate schemas.

CARoleCode, ChemicalCadmiumClassType, ChemicalDeOxygenatingSubstanceCode, ChemicalNutrientType,                 Economic2009to2015Code, EconomicWaterServiceType.GWAnnexIIPollutantsCode, GWBackgoundSubstancesCode,             GWChemicalFailureCode,GWConcentrationUnitsCode, GWDepthRangeCode, QEEcologicalCode,                     GWGeologicalFormationCode, GWIndividualPesticideCode, GWMeasureLevelCode, GWQuantativeFailureCode,                 GWVerticalOrientationCode,  MeasureCostAggregationType, MeasureCostClassificationType, MeasureCoverageType,

MeasureRBDorNationalCode, MeasureSWSupplementaryType, PressureGWAbstractionPeriodType,                     PressureGWAbstractionUnitType,  PressureLoadUnitType, ProtAreaAssociationType, QE3-1-1MeasurementType,

QE3-1-2MeasurementType, QE3-1-3MeasurementType, QE3-1-4MeasurementType, QE3-1-5MeasurementType,

QE3-1-6MeasurementType, SWNaturalCode.

8. Elements removed from Common

AltitudeTypeCode, ConfidenceLevelCode, CostRecoveryLevelCode, CountryCodeType, DesignationCode,                 EcoregionCode,     FlowDirCode, GWPollutantType, ImportanceCode, MSCodeType, NationalInternationalCode,                 NationalInternationalType, NitrogenMethodType, OptionalString1000Type, OptionalString100Type, OptionalString250Type,             OptionalString500Type, OptionalString50Type, ProtectedAreaCode, ProtectedAreaTypeCode,  RiskStatusCode, SystemTypeCode,         WaterBodyCategoryCode ,YesNoType.

The following elements were created, renamed and then removed during the compilation of the streamlined schemas.

CoastalQualityElementCode        renamed to            QECoastalCode

LakeQualityElementCode            renamed to            QELakeCode

RiverQualityElementCode        renamed to            QERiverCode                                                  TransitionalQualityElementCode    renamed to            QETransitionalCode

------------------------------------------------------------------------------------------------------------------------------------------------

Changes arising from outcomes of second testing perion 22-oct-2009

1.    SWStatusNonPrioritySubstanceCode - Add enumeration '1' corresponding to 'High status'

2. GWPollutantAnnexI-IIType  - Eliminate the option IndividualPesticides

3. QE3-1ParameterType  - Add the following:

QE3-1-4 Chloride

QE3-1-4 Sulfate

QE3-1-6 N mineral

QE3-1-3 Dissolved organic carbon

4. Economic2009to2015Code  - Add the option "2009-2015"

5. GWStatusCode - changed annotation - unclassified to unknown

6. EconomicWaterServiceType - replace enumeration list with

-water supply for households

-water supply for industry

-water supply for agriculture

-wastewater collection and treatment for households

-wastewater collection and treatment for industry

-Other

##### WFD_GWB_EnumAndSimpleTypes_Supplementary

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes

Detail:		Created on 2011-03-11. Last modified on 2011-03-12

GUID:		{9C67A0B0-1513-485d-BF3C-D6D858AFFC00}

WFD_GWB_EnumAndSimpleTypes_Supplementary - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-11

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{13D9E51F-D087-4c82-BA8F-9C26B614E359}

Figure: 27

###### ChemicalAllSubstancesType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{C239555F-8AA4-424f-A5E1-078584E7EC6A}

Contains Heavy Metals, Pesticides, Priority Substances and Other Pollutants

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 1 Heavy Metals - aggregated string Public |  | Default: |
| 1.1 Cadmium string Public |  | Default: |
| 1.2 Lead string Public |  | Default: |
| 1.3 Mercury string Public |  | Default: |
| 1.4 Nickel string Public |  | Default: |
| 2 Pesticides - aggregated string Public |  | Default: |
| 2.1 Alachlor string Public |  | Default: |
| 2.2 Atrazine string Public |  | Default: |
| 2.3 Chlorpyriphos string Public |  | Default: |
| 2.4 Chlorvenfinphos string Public |  | Default: |
| 2.5 Diuron string Public |  | Default: |
| 2.6 Endosulfan string Public |  | Default: |
| 2.7 Isoproturon string Public |  | Default: |
| 2.8 Hexachlorocyclohexane string Public |  | Default: |
| 2.9 Pentachlorobenzene string Public |  | Default: |
| 2.10 Simazine string Public |  | Default: |
| 2.11 Trifluralin string Public |  | Default: |
| 3 Industrial Pollutants - aggregated string Public |  | Default: |
| 3.1 Anthracene string Public |  | Default: |
| 3.2 Benzene string Public |  | Default: |
| 3.3 Brominated diphenylether string Public |  | Default: |
| 3.4 Carbon tetrachloride string Public |  | Default: |
| 3.5 C10-13-chloroalkanes string Public |  | Default: |
| 3.6 1,2-Dichloroethane string Public |  | Default: |
| 3.7 Dichloromethane string Public |  | Default: |
| 3.8 Di(2-ethylhexyl)phthalate (DEHP) string Public |  | Default: |
| 3.9 Naphthalene string Public |  | Default: |
| 3.10 Nonylphenol string Public |  | Default: |
| 3.11 Octylphenol string Public |  | Default: |
| 3.12 Tetrachloroethylene string Public |  | Default: |
| 3.13 Trichloroethylene string Public |  | Default: |
| 3.14 Trichloromethane string Public |  | Default: |
| 4 Other pollutants - aggregated string Public |  | Default: |
| 4.1 Aldrin string Public |  | Default: |
| 4.2 Dieldrin string Public |  | Default: |
| 4.3 Endrin string Public |  | Default: |
| 4.4 Isodrin string Public |  | Default: |
| 4.5 DDT Total string Public |  | Default: |
| 4.6 para-para-DDT string Public |  | Default: |
| 4.7 Flouranthene string Public |  | Default: |
| 4.8 Hexachlorobenzene string Public |  | Default: |
| 4.9 Hexachlorobutadiene string Public |  | Default: |
| 4.10 Pentachlorophenol string Public |  | Default: |
| 4.11 Polyaromatic hydrocarbons string Public |  | Default: |
| 4.12 Benzo(a)pyrene string Public |  | Default: |
| 4.13 Benzo(b)fluoranthene string Public |  | Default: |
| 4.14 Benzo(k)fluoranthene string Public |  | Default: |
| 4.15 Benzo(g,h,i)perylene string Public |  | Default: |
| 4.16 Indeno(1,2,3-cd)pyrene string Public |  | Default: |
| 4.17 Tributyltin compounds string Public |  | Default: |
| 4.18 Trichlorobenzenes (all isomers) string Public |  | Default: |

###### ChemicalMatrixType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{5FCD57F6-C918-4c48-B264-D4A1B38C679A}

Defines the types of chemical measurement methods

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Embedded Elements

| Element | Detail | Notes |
| --- | --- | --- |
| Port Port1 | Version: 1.0 |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| water string Public |  | Default: |
| settled sediment string Public |  | Default: |
| suspended sediment string Public |  | Default: |
| biota string Public |  | Default: |

###### EconomicWaterServiceType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{43ED35C7-4886-4c63-89C4-B4E3AC09A7E6}

An enumeration list of

Water supply for households

Water supply for industry

Water supply for agriculture

Wastewater collection and treatment for households

Wastewater collection and treatment for industry

Other

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Water supply for households string Public |  | Default: |
| Water supply for industry string Public |  | Default: |
| Water supply for agriculture string Public |  | Default: |
| Wastewater collection and treatment for households string Public |  | Default: |
| Wastewater collection and treatment for industry string Public |  | Default: |
| Other string Public |  | Default: |

###### GWMeasureLevelCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{8D930CD5-EFA5-4e1b-9FC3-555B51F63D11}

An enumeration list of Member State, International RBD, national RBD, part of RBD, Groundwater body

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Member State string Public |  | Default: |
| International RBD string Public |  | Default: |
| National string Public |  | Default: |
| RBD string Public |  | Default: |
| Part of RBD string Public |  | Default: |
| Groundwater body string Public |  | Default: |

###### GWPollutantAnnexI-IIType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{5B0FAB4D-790D-4731-A95F-D7CD12D34703}

Defines the types of Annex I - II pollutants (of Groundwater Directive 2006/118/EEC) or indicators for Groundwater classification

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Nitrates string Public |  | Default: |
| Arsenic string Public |  | Default: |
| Cadmium string Public |  | Default: |
| Lead string Public |  | Default: |
| Mercury string Public |  | Default: |
| Ammonium string Public |  | Default: |
| Chloride string Public |  | Default: |
| Sulphate string Public |  | Default: |
| Trichloroethylene string Public |  | Default: |
| Tetrachloroethylene string Public |  | Default: |
| Conductivity string Public |  | Default: |
| TotalPesticides string Public |  | Default: |

###### GWPressureAbstractionType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{BB0220B7-859B-458a-937B-CEE774379BE7}

Defines the list of GW Abstraction pressures

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 3 Abstraction string Public |  | Default: |
| 3.1 Abstraction - Agriculture string Public |  | Default: |
| 3.2 Abstraction - Public Water Supply string Public |  | Default: |
| 3.3 Abstraction - Industry (aggregated) string Public |  | Default: |
| 3.3.1 Abstraction - IPPC industries string Public |  | Default: |
| 3.3.2 Abstraction - Non-IPPC industries string Public |  | Default: |
| 3.4 Abstraction - Quarries string Public |  | Default: |
| 3.5 Abstraction - Other string Public |  | Default: |

###### GWPressureAggregatedType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{33E7BE1C-6F5B-44b5-A54F-A25C3981EC1D}

Defines the list of groundwater aggregated pressures

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 1 Point Sources string Public |  | Default: |
| 2 Diffuse Sources string Public |  | Default: |
| 3 Abstraction string Public |  | Default: |
| 4 Artificial Recharge string Public |  | Default: |
| 5 Saltwater Intrusion string Public |  | Default: |
| 6 Other Pressures string Public |  | Default: |

###### GWPressureRechargeType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{36B3D614-54AA-4ce5-9022-7EF2BEA7EEF4}

Defines the list of GW Recharge pressures

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 4 Artificial Recharge string Public |  | Default: |
| 4.1 Recharge - Discharges string Public |  | Default: |
| 4.2 Recharge - Returns string Public |  | Default: |
| 4.3 Recharge - Mine water rebound string Public |  | Default: |
| 4.4 Recharge - Other string Public |  | Default: |

###### PressureGWAbstractionPeriodType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{D05E98E7-6390-4378-8400-01A6923564AC}

An enumeration list of yearly, winter, spring, summer, autumn

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| yearly string Public |  | Default: |
| winter string Public |  | Default: |
| spring string Public |  | Default: |
| summer string Public |  | Default: |
| autumn string Public |  | Default: |

###### PressureGWAbstractionUnitType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumAndSimpleTypes_Supplementary     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{38B5186E-045E-46f8-BAA3-930B0B93A16C}

An enumeration list of m3, 1000m3, 1000000m3

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| m3 string Public |  | Default: |
| 1000m3 string Public |  | Default: |
| 1000000m3 string Public |  | Default: |

##### WFD_GWB_EnumerationsAndSimpleTypes

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes

Detail:		Created on 2011-03-12. Last modified on 2011-04-13

GUID:		{E2870466-9DE9-433b-9877-3203F2D6FF84}

WFD_GWB_EnumerationsAndSimpleTypes - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-10

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{731B73F8-5EA2-426b-B484-532655EC76D7}

Figure: 28

###### CountryCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{951AAF1C-49F4-46dd-A600-700AA8B7750E}

Unique 2 character ISO Code for the EU Member States

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| AT string Public |  | Default: |
| BE string Public |  | Default: |
| BG string Public |  | Default: |
| CH string Public |  | Default: |
| CZ string Public |  | Default: |
| CY string Public |  | Default: |
| DE string Public |  | Default: |
| DK string Public |  | Default: |
| EE string Public |  | Default: |
| EL string Public |  | Default: |
| ES string Public |  | Default: |
| FI string Public |  | Default: |
| FR string Public |  | Default: |
| HR string Public |  | Default: |
| HU string Public |  | Default: |
| IE string Public |  | Default: |
| IS string Public |  | Default: |
| IT string Public |  | Default: |
| LT string Public |  | Default: |
| LU string Public |  | Default: |
| LV string Public |  | Default: |
| MT string Public |  | Default: |
| NO string Public |  | Default: |
| NL string Public |  | Default: |
| PL string Public |  | Default: |
| PT string Public |  | Default: |
| RO string Public |  | Default: |
| SE string Public |  | Default: |
| SI string Public |  | Default: |
| SK string Public |  | Default: |
| TR string Public |  | Default: |
| UK string Public |  | Default: |

###### DataConfidentialityClassificationCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{82FC2DE7-6FA3-46e6-B7D4-3E4CE383413C}

Codes for data security classification:

001 = Unclassified - available for general circulation

003 = Confidential - available for EC reporting only

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 001 string Public |  | Default: |
| 003 string Public |  | Default: |

###### GECode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{2B09C28E-CAD0-4042-8803-6270EBF1EBD5}

Codes for groundwater parameter/elements used in groundwater monitoring programmes

Groundwater level and yield GE1

Groundwater level GE1-1

Groundwater yield GE1-2

General parameters – all sub-elements below are determined (as described by Annex 5) GE2

Oxygen content GE2-1

pH Value GE2-2

Conductivity GE2-3

Nitrate GE2-4

Ammonium GE2-5

Other pollutants (e.g. other pollutants not covered above) are determined GE3

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| GE1 Groundwater level and yield string Public |  | Default: |
| GE1-1 Groundwater level string Public |  | Default: |
| GE1-2 Groundwater yield string Public |  | Default: |
| GE2 General parameters string Public |  | Default: |
| GE2-1 Oxygen content string Public |  | Default: |
| GE2-2 pH Value string Public |  | Default: |
| GE2-3 Conductivity string Public |  | Default: |
| GE2-4 Nitrate string Public |  | Default: |
| GE2-5 Ammonium string Public |  | Default: |
| GE3 Other pollutants string Public |  | Default: |

###### GWBackgoundSubstancesCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{92EED1EA-0458-425a-BB77-4DFEA4D9AB6E}

An enumeration list of Arsenic, Cadmium, Lead, Mercury, Ammonium, Nitrate, Chloride, Sulphate, Conductivity.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Arsenic string Public |  | Default: |
| Cadmium string Public |  | Default: |
| Lead string Public |  | Default: |
| Mercury string Public |  | Default: |
| Ammonium string Public |  | Default: |
| Chloride string Public |  | Default: |
| Sulphate string Public |  | Default: |
| Conductivity string Public |  | Default: |
| Nitrate string Public |  | Default: |

###### GWChemicalFailureCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{333AD9B4-BA28-4cb1-B429-565E456FBD1B}

An enumeration list of saline intrusions, exceedance, environmental, terrestrial, water quality, human impairment and pollutant risk

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| saline intrusions string Public |  | Default: |
| exceedance string Public |  | Default: |
| environmental string Public |  | Default: |
| terrestrial string Public |  | Default: |
| water quality string Public |  | Default: |
| human impairment string Public |  | Default: |
| pollutant risk string Public |  | Default: |

###### GWConcentrationUnitsCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{BB5E7E47-E273-4717-83C9-5F4C5EAD500A}

An enumeration list of mg/l, µg/l, ng/l, S m-1, µS/cm and Other

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| mg/l string Public |  | Default: |
| µg/l string Public |  | Default: |
| ng/l string Public |  | Default: |
| S m-1 string Public |  | Default: |
| µS/cm string Public |  | Default: |
| Other string Public |  | Default: |

###### GWDepthRangeCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{83E6056C-25EB-4abc-A4BD-306874057260}

Assignment to a depth range at the point where the main part of the Groundwater Body is situated (depth ranges: 0-20m, 20-50m, 50-200m, >200m)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 0-20 string Public |  | Default: |
| 20-50 string Public |  | Default: |
| 50-200 string Public |  | Default: |
| 200+ string Public |  | Default: |

###### GWGeologicalFormationCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{6F732E7A-0121-48d3-9E3A-7665E71C64FC}

An enumeration list

Porous - highly productive ,

Porous - moderately productive,

Fissured aquifers including karst - highly productive,

Fissured aquifers including karst - moderately productive,

Insignificant aquifers - local and limited groundwater

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Porous high string Public |  | Default: |
| Porous moderate string Public |  | Default: |
| Fissured high string Public |  | Default: |
| Fissured moderate string Public |  | Default: |
| Insignificant string Public |  | Default: |

###### GWImpactTypes

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{0B832DFF-821F-4f8f-84EE-27D1E1022F38}

Defines the types of Groundwater Impacts

Anthropogenic alterations of the level of groundwater leading to significant diminution of the ecological and qualitative status of associated surface water bodies;

Chemical composition of groundwater leading to significant diminution of the ecological and qualitative status of associated surface water bodies;

Anthropogenic alterations of the level of groundwater leading to significant damage to terrestrial ecosystems which depend directly on the GWB;

Chemical composition of groundwater leading to significant damage to terrestrial ecosystems which depend directly on the GWB;

Altered habitat in dependent surface water or terrestrial ecosystems;

Substitution of populations;

Other impacts.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Anthropogenic Diminution string Public |  | Default: |
| Chemical Diminution string Public |  | Default: |
| Anthropogenic Damage string Public |  | Default: |
| Chemical Damage string Public |  | Default: |
| Altered Habitat string Public |  | Default: |
| Population substitution string Public |  | Default: |
| Other Groundwater Impact string Public |  | Default: |

###### GWPollutantsCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{AC83B971-C29B-46ea-AE46-270EC5BBA772}

An enumeration list of 1. Nitrates; 2. Pesticides - Alachlor, Atrazine, Endosulphan, Isoproturon, Hexachlorocyclohexane, Pentachlorobenzene, Simazine, Trifluralin; 3. Annex II pollutants - Arsenic, Cadmium, Lead, Mercury, Ammonium, Chloride, Sulphate, Trichloroethylene, Tetrachloroethylene, Conductivity.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 1 Nitrates string Public |  | Default: |
| 2 Pesticides string Public |  | Default: |
| 2.1 Alachlor string Public |  | Default: |
| 2.2 Atrazine string Public |  | Default: |
| 2.3 Endosulfan string Public |  | Default: |
| 2.4 Isoproturon string Public |  | Default: |
| 2.5 Hexachlorocyclohexane string Public |  | Default: |
| 2.6 Pentachlorobenzene string Public |  | Default: |
| 2.7 Simazine string Public |  | Default: |
| 2.8 Trifluralin string Public |  | Default: |
| 3 Annex II pollutant string Public |  | Default: |
| 3.1 Arsenic string Public |  | Default: |
| 3.2 Cadmium string Public |  | Default: |
| 3.3 Lead string Public |  | Default: |
| 3.4 Mercury string Public |  | Default: |
| 3.5 Ammonium string Public |  | Default: |
| 3.6 Chloride string Public |  | Default: |
| 3.7 Sulphate string Public |  | Default: |
| 3.8 Trichloroethylene string Public |  | Default: |
| 3.9 Tetrachloroethylene string Public |  | Default: |
| 3.10 Conductivity string Public |  | Default: |

###### GWPressureType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{CE0A7C84-D138-4db4-A958-0044942F7CB3}

Defines the list of groundwater pressures requiring supplementary or additional measures

POINT SOURCES

- Leakages from contaminated sites;

- Leakages from waste disposal sites (landfill and agricultural waste disposal);

- Leakages associated with oil industry infrastructure;

- Mine water discharges;

- Discharges to ground such as disposal of contaminated water to soakaways;

- Other relevant point sources.

DIFFUSE SOURCES

- Due to agricultural activities (e.g. fertilizer and pesticide use, live stock etc.);

- Due to non-sewered population;

- Urban land use;

- Other relevant diffuse sources.

ABSTRACTION OF WATER

- Abstractions for agriculture;

- Abstractions for public water supply;

Abstractions by industry including:

- IPPC industries;

- Non-IPPC industries;

- Abstractions by quarries/open cast coal sites;

- Other major abstractions.

ARTIFICAL RECHARGES

- Discharges to groundwater for artificial recharge purposes;

- Returns of groundwater to GWB from which it was abstracted (e.g. for sand and gravel washing);

- Mine water rebound;

- Other major recharges

OTHER

- Saltwater intrusion;

- Other intrusion.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 1 Point Sources string Public |  | Default: |
| 2 Diffuse Sources string Public |  | Default: |
| 3 Abstraction string Public |  | Default: |
| 4 Artificial Recharge string Public |  | Default: |
| 5 Saltwater Intrusion string Public |  | Default: |
| 6 Other Pressures string Public |  | Default: |
| 1.1 Point - Contaminated Sites string Public |  | Default: |
| 1.2 Point - Waste Disposal Sites string Public |  | Default: |
| 1.3 Point - Oil Industry string Public |  | Default: |
| 1.4 Point - Mine Waters string Public |  | Default: |
| 1.5 Point - Discharges To Ground string Public |  | Default: |
| 1.6 Point - Other string Public |  | Default: |
| 2.1 Diffuse - Agriculture string Public |  | Default: |
| 2.2 Diffuse - Non-sewered Population string Public |  | Default: |
| 2.3 Diffuse - Urban Land Use string Public |  | Default: |
| 2.4 Diffuse - Other string Public |  | Default: |
| 3.1 Abstraction - Agriculture string Public |  | Default: |
| 3.2 Abstraction - Public Water Supply string Public |  | Default: |
| 3.3 Abstraction - Industry (aggregated) string Public |  | Default: |
| 3.3.1 Abstraction - IPPC industries string Public |  | Default: |
| 3.3.2 Abstraction - Non-IPPC industries string Public |  | Default: |
| 3.4 Abstraction - Quarries string Public |  | Default: |
| 3.5 Abstraction - Other string Public |  | Default: |
| 4.1 Recharge - Discharges string Public |  | Default: |
| 4.2 Recharge - Returns string Public |  | Default: |
| 4.3 Recharge - Mine water rebound string Public |  | Default: |
| 4.4 Recharge - Other string Public |  | Default: |
| 5.1 Saltwater intrusion string Public |  | Default: |
| 5.2 Other intrusion string Public |  | Default: |

###### GWQuantitativeFailureCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{67E8D607-BCE1-4896-BA2F-081DE57EF506}

An enumeration list of exceedance, environmental, diminution, damage and intrusion.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| exceedance string Public |  | Default: |
| environmental string Public |  | Default: |
| diminution string Public |  | Default: |
| damage string Public |  | Default: |
| intrusion string Public |  | Default: |

###### GWStatusCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{243474E4-4295-4e45-85BA-02F22A27CC45}

Codes for groundwater status 2=good, 3=poor, U=unknown

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 2 string Public |  | Default: |
| 3 string Public |  | Default: |
| U string Public |  | Default: |

###### GWVerticalOrientationCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{09634388-F531-44b6-A6EF-E563E8655ABC}

An enumeration list. Horizontal, mainly continuous body; Horizontal, mainly smaller, individual formations;  Declining, mainly continuous body; Declining, mainly smaller, individual formations; Declining and cumulative, mainly continuous body; Declining and cumulative, mainly smaller, individual formations; Boat form, mainly continuous body; Smaller individual boat formed formations; Other

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Horizontal continuous string Public |  | Default: |
| Horizontal individual string Public |  | Default: |
| Declining, mainly continuous string Public |  | Default: |
| Declining cumulative string Public |  | Default: |
| Declining individual string Public |  | Default: |
| Boat string Public |  | Default: |
| Smaller boat string Public |  | Default: |
| Other string Public |  | Default: |

###### GWWellOrSpringCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{5EAC7941-5056-404a-B2B7-1B5819F8BA22}

Codes for well or spring

W = Well

S  = Spring

O = Other

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| W string Public |  | Default: |
| S string Public |  | Default: |
| O string Public |  | Default: |

###### MonitoringDepthCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{B55C0767-C6DF-43fd-B742-1EE48A06B3ED}

Codes for depth at which monitoring occurs

U = Upper

M = Middle

L = Lower

A = Mixed

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| U string Public |  | Default: |
| M string Public |  | Default: |
| L string Public |  | Default: |
| A string Public |  | Default: |

###### MonitoringInternationalNetworkType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{7AD8EAD7-C8FB-4a11-9887-5A67CAA1B1B9}

International monitoring networks

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| EIONET string Public |  | Default: |
| Helsinki Convention(HELCOM) string Public |  | Default: |
| Barcelona Convention/Mediterranean Action Plan(MEDPOL) string Public |  | Default: |
| Arctic Monitoring and Assessment Programme string Public |  | Default: |
| North East Atlantic (OSPAR) string Public |  | Default: |
| Bucharest Convention/Black Sea Commission string Public |  | Default: |
| Transboundary Waters Convention string Public |  | Default: |
| International Commission for the Meuse string Public |  | Default: |
| International Scheldt Commission string Public |  | Default: |
| International Commission for the Elbe string Public |  | Default: |
| International Commission for the Protection of the Danube string Public |  | Default: |
| International Commission for the Protection of the Odra/Oder string Public |  | Default: |
| International Commission for the Protection of the Rhine against Pollution (ICPR) string Public |  | Default: |
| international commission for the protection of the Moselle against pollution string Public |  | Default: |

###### MonitoringSubsiteCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{CAB993AB-AE04-4ad7-866C-8AAD277C565A}

Codes to define the type of subsite: A= Area, T=Transect, M=Multipoint, N=Not Applicable/None

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| A string Public |  | Default: |
| T string Public |  | Default: |
| M string Public |  | Default: |
| N string Public |  | Default: |

###### ObjectivesExemptionType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{27E8162F-BA05-4bb7-BC36-910E13675FE4}

Defines the Articles against which exemptions are made

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Article4(4) -Technical feasibility string Public |  | Default: |
| Article4(4) - Disproportionate cost string Public |  | Default: |
| Article4(4) - Natural conditions string Public |  | Default: |
| Article4(5) -Technical feasibility string Public |  | Default: |
| Article4(5) - Disproportionate cost string Public |  | Default: |
| Article4(6) - Natural causes string Public |  | Default: |
| Article4(6) - Force Majeure string Public |  | Default: |
| Article4(6) - Accidents string Public |  | Default: |
| Article4(7) - New modification string Public |  | Default: |
| Article4(7) - Sustainable human development string Public |  | Default: |

###### ProtAreaAssociationType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{7A4B9412-0359-42fa-9DA8-7E1CB4A29B8F}

Defines the type of protected Area Association. Options are within protected area, overlapping (partly within) and dynamically connected.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| within protected area string Public |  | Default: |
| overlapping (partly within) string Public |  | Default: |
| dynamically connected string Public |  | Default: |

###### ProtectedAreaGWType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{D1CA9712-DAE8-409c-94E0-6FFDFCD75ADF}

Defines Protected Areas types relevant to the classification of Protected Areas associated with Groundwater Bodies

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Birds string Public |  | Default: |
| Habitats string Public |  | Default: |
| Nitrates string Public |  | Default: |
| Article 7 Abstraction for drinking water string Public |  | Default: |
| European other string Public |  | Default: |
| National string Public |  | Default: |
| Local string Public |  | Default: |

###### ProtectedAreaType

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{7911EE32-DDB6-4abd-B5D9-9697E13D7792}

Defines all Protected Area types for use in the Register of Protected Areas (Article 6)

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| Bathing string Public |  | Default: |
| Birds string Public |  | Default: |
| Fish string Public |  | Default: |
| Shellfish string Public |  | Default: |
| Habitats string Public |  | Default: |
| Nitrates string Public |  | Default: |
| UWWT string Public |  | Default: |
| Article 7 Abstraction for drinking water string Public |  | Default: |
| EuropeanOther string Public |  | Default: |
| National string Public |  | Default: |
| Local string Public |  | Default: |

###### ReportingLevelCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{43509E95-0927-42a1-8596-097404BF3331}

Codes for describing the reporting level: I = international RBD,  N= National part of international RBD, R = national RBD, S  = regional part or sub-unit of RBD

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| I string Public |  | Default: |
| N string Public |  | Default: |
| R string Public |  | Default: |
| S string Public |  | Default: |

###### SWStatusProtAreaCode

Type:		Enumeration

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-12.

GUID:		{386C6BAA-8254-4d17-9A67-E34ADB08A367}

Codes for reporting the status of Protected Areas associated with Surface Water Bodies (see section 2.2.5 of this guide for further information):

For Article 7 drinking water and Nitrates Directive Protected Areas: 2=good, 3=failing to achieve good, U=unknown.

For Freshwater Fish and Shellfish Directive Protected Areas: 1=high, 2=good, 3=failing to achieve good, U = unknown.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| 1 string Public |  | Default: |
| 2 string Public |  | Default: |
| 3 string Public |  | Default: |
| U string Public |  | Default: |

###### ScaleType

Type:		Class    string

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_EnumerationsAndSimpleTypes     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-03-12.

GUID:		{0275D062-0BFD-4cee-AB3F-408F4F50691A}

Format for Scale: 1:nnnnnnnn

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | derivation = restriction. |
|  | pattern = 1:[0-9]{1,8}. |

#### WFD_GWB_Monitoring

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_WFD_GWB

Detail:		Created on 2011-03-11. Last modified on 2011-03-12

GUID:		{47CA5BB1-3FBD-41b6-A2AD-A773ED0946F3}

WFD_GWB_Monitoring - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-11

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{A8EC22AF-40F6-4fe3-8F7E-D94B2A79308C}

Figure: 29

##### AssociatedGroudWaterBody

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_Monitoring     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-03-11.

GUID:		{1586CB10-814C-43c2-B94A-223D8C26AF0B}

Waterbodies at which this QE is measured. only required if this is representative of WB other than that identifed as the WB location

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Source -> Destination | Public WFD_GW_MonitoringStation | Public AssociatedGroudWaterBody |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| WFD_GWB_code FeatureUniqueEUCodeType Public [1..*] | Unique code of the groundwater body at EU level as reported in Article 5. | Default: [minOccurs = 1 ] [maxOccurs = unbounded ] |

##### GW_MonitoringInternationalNetwork

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_Monitoring     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-03-11.

GUID:		{6A4789A6-656A-404a-A893-C37402F10CF7}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public WFD_GW_MonitoringStation | Public GW_MonitoringInternationalNetwork |  |
| Association Bi-Directional | Public GW_MonitoringInternationalNetwork | Public WFD_GW_MonitoringStation |  |
| Aggregation Source -> Destination | Public GW_MonitoringStationDescription | Public GW_MonitoringInternationalNetwork |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| name MonitoringInternationalNetworkType Public | Name of the international network, e.g. EIONET | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| siteCode PF_FreeText Public «voidable» | Unique code for the station in this international network | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |

##### GW_MonitoringStationDescription

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_Monitoring     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-03-11.

GUID:		{5F39272C-A49E-4d93-8DCC-D285CCAB1BFC}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | memberNames = . Values: qualified ¦ unqualified Description: Overrides the package-level default for naming complexType definitions |
|  | mixed = false. Values: true ¦ false Default: false Description: Determines whether this element may contain mixed element and character content. Refer to the W3C XML Schema recommendation |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public GW_MonitoringStationDescription | Public GW_MonitoringInternationalNetwork |  |
| Aggregation Source -> Destination | Public GW_MonitoringStationDescription | Public WFD_GW_MonitoringStation |  |
| Aggregation Source -> Destination | Public GW_MonitoringStationDescription | Public WFD_GW_MonitoringStation |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| countryCode CountryCode Public | Unique 2 character code for the country | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| EU_RBD_code FeatureUniqueEUCodeType Public | The unique EU code of the parent river basin district | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| WFD_RBD_name PF_FreeText Public | Name of RBD | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| reportingLevel ReportingLevelCode Public | Reporting level - international RBD, National part of international RBD, national RBD, regional part or sub-unit of RBD | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| reportingLevelDescription PF_FreeText Public «voidable» | Short description of thr reporting level. | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| URL anyURI Public «voidable» | URL for integration of your own internet-based information | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| creationDate DateTime Public |  | Default: [use = required ] |
| creator PF_FreeText Public |  | Default: [use = required ] |
| email PF_FreeText Public |  | Default: [use = optional ] |
| description PF_FreeText Public |  | Default: [use = optional ] |
| generatedBy PF_FreeText Public |  | Default: [use = optional ] |
| MD_classificationCode DataConfidentialityClassificationCode Public |  | Default: [use = optional ] |

##### Parameter

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_Monitoring     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-03-11.

GUID:		{202A3322-C67E-4868-AE7A-CBB98E2ED2C8}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public Parameter | Public WFD_GW_MonitoringStation |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| parameterCode GECode Public | Code or name of the groundwater element. | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| measuredAs PF_FreeText Public «voidable» | How the parameter is measured at this site | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| isQuantitative boolean Public «voidable» | Parameter is measured for quantitative purposes at this site | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| isChemicalSurveillance boolean Public «voidable» | Parameter is measuered for chemical surveillance purposes at this site | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| isChemicalOperational boolean Public «voidable» | Parameter is measured for chemical operational purposes at this site | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| subsites MonitoringSubsiteCode Public «voidable» | Subsite Type: Transect, Area , Multipoint or Not Applicable/None | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| frequency int Public «voidable» | Frequency at which the parameter is measured at this station at this site if varies from general programme frequency. | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| cycle int Public «voidable» | Monitoring cycle for this parameter if varies from general programme frequency. Use 0 to indicate that the parameter is only measured once | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| cycleDescription PF_FreeText Public «voidable» | If the cycle is dependant on the results of monitoring please summarise here | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| reason PF_FreeText Public «voidable» | Reason for deviation in frequency or cycle from main programme if appropriate | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |

##### WFD_GW_MonitoringStation

Type:		Class    GroundWaterMonitoringStation

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_Monitoring     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-03-12.

GUID:		{D56258E6-F021-4e84-9148-0DE336C48D51}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public WFD_GW_MonitoringStation | Public GW_MonitoringInternationalNetwork |  |
| Association Bi-Directional | Public GW_MonitoringInternationalNetwork | Public WFD_GW_MonitoringStation |  |
| Aggregation Source -> Destination | Public GW_MonitoringStationDescription | Public WFD_GW_MonitoringStation |  |
| Aggregation Source -> Destination | Public Parameter | Public WFD_GW_MonitoringStation |  |
| Association Source -> Destination | Public WFD_GW_MonitoringStation | Public AssociatedGroudWaterBody |  |
| Aggregation Source -> Destination | Public GW_MonitoringStationDescription | Public WFD_GW_MonitoringStation |  |
| Generalization Source -> Destination | Public WFD_GW_MonitoringStation | Public GroundWaterMonitoringStation |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| EU_code FeatureUniqueEUCodeType Public | Unique Code for the monitoring station at EU level | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| MS_code FeatureUniqueCodeType Public | Unique Code for the Monitoring station within the MS | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| location GM_Point Public | Longitude in ETRS89 of the monitoring station | Default: [minOccurs = 1 ] [nillable = true ] [maxOccurs = 1 ] |
| GWB_location FeatureUniqueEUCodeType Public | Unique code at EU level as reported in Article 5 of the ground water body which the monitoring station is physically located on or near. | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| name PF_FreeText Public «voidable» | Locally used name for the monitoring station | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| wellOrSpring GWWellOrSpringCode Public | Is the station a well or a spring or other? | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| isQuantitative boolean Public | Station is used for quantitative monitoring | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| isChemicalSurveillance boolean Public | Station is used for chemical surveillance monitoring | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| isChemicalOperational boolean Public | Station is used for chemical operational monitoring | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| numberOfSubsites int Public | Number of subsites normally associated with this station. | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| depth MonitoringDepthCode Public «voidable» | Groundwater layer at which sampling occurs - upper, medium, lower or mixed. | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| isMonitoring boolean Public «voidable» | Site is used for monitoring of groundwater status | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| isForDrinkingWater boolean Public «voidable» | Site is used for drinking water abstraction | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| additionalRequirements PF_FreeText Public «voidable» | Additional requirements if the station is located in drinking water abstraction areas. | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| isForIndustrialSupply boolean Public «voidable» | Site is used for industrial supply | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| isForIrrigation boolean Public «voidable» | Site is used for irrigation | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| isForOtherPurposes boolean Public «voidable» | Site is used for other purposes. | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| otherNetworks PF_FreeText Public «voidable» | Brief description of any other networks that this station is a part of | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| URL anyURI Public «voidable» | URL for integration of your own internet-based information for this site | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |

#### WFD_GWB_ProtectedArea

Type:		Package «applicationSchema»

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology_WFD_GWB

Detail:		Created on 2011-03-11. Last modified on 2011-03-12

GUID:		{506F8FB3-B8C9-4652-88C9-C771169765DD}

WFD_GWB_ProtectedArea - (Logical diagram)

Created By:	Janusz Michalak on 2011-03-11

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{72FCCEDF-B28F-4979-8B16-5282757F096B}

Figure: 30

##### ProtectedAreaDescription

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_ProtectedArea     Keywords:

Detail:		Created on 2011-03-10.  Last modified on 2011-03-11.

GUID:		{7FA75AE9-8994-46d9-8599-D82C0818E1B7}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public ProtectedAreaDescription | Public WFD_ProtectedArea |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| countruCode CountryCode Public | Two-letter ISO country code | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| EU_RBD_code FeatureUniqueEUCodeType Public | Unique EU code of the parent River Basin District. Add the two-letter ISO Country code followed by the Member State unique id | Default: [minOccurs = 1 ] [maxOccurs = 1 ] |
| URL anyURI Public «voidable» | URL for integration of your own internet-based information | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| creationDate dateTime Public |  | Default: [use = required ] |
| creator PF_FreeText Public |  | Default: [use = required ] |
| email styring Public «voidable» |  | Default: [use = optional ] |
| description PF_FreeText Public «voidable» |  | Default: [use = optional ] |
| generatedBy PF_FreeText Public «voidable» |  | Default: [use = optional ] |
| MD_classificationCode DataConfidentialityClassificationCode Public «voidable» |  | Default: [use = optional ] |

##### RegisterProtectedAreas

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_ProtectedArea     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-03-12.

GUID:		{9DBA2A90-587D-42ce-920B-A7258186FE66}

A Register of Protected Areas is required under Art 6 of the WFD. Annex IV of WFD specifies which types of Protected Areas should be included in the register and what the summary of the register (which should be part of the RBMP) should include.

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Association Bi-Directional | Public WFD_ProtectedArea | Public RegisterProtectedAreas |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| legislation PF_FreeText Public | Provide a list of the Community, national or local legislation under which the Protected Areas have been designated. | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| linkToLegislationReference AssociatedReferenceStructure Public «voidable» | Hyperlink or reference to more detailed information or further documentation | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |

##### Size

Type:		Class

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_ProtectedArea     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-03-12.

GUID:		{8AE31564-73D6-4573-B577-DCADC7F1E31C}

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = choice. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Aggregation Source -> Destination | Public Size | Public WFD_ProtectedArea |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| area double Public | Area of the Protected Area in km2 (if more appropriate, length in km should be provided - see LENGTH element) | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| leng double Public | Length of the Protected Area in km (if more appropriate, area in km2 should be provided - see AREA element) | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |

##### WFD_ProtectedArea

Type:		Class    HydrogeologicalFeature

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	WFD_GWB_ProtectedArea     Keywords:

Detail:		Created on 2011-03-11.  Last modified on 2011-03-14.

GUID:		{6EA2D32E-8343-4b0c-A4AF-D950676A5704}

Only to be used when adding new Protected Area details for an RBD

| Custom Properties |  |
| --- | --- |
|  | isActive = False |

| Tagged Values |  |
| --- | --- |
|  | modelGroup = sequence. |

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Generalization Source -> Destination | Public WFD_ProtectedArea | Public HydrogeologicalFeature |  |
| Association Bi-Directional | Public WFD_ProtectedArea | Public RegisterProtectedAreas |  |
| Aggregation Source -> Destination | Public Size | Public WFD_ProtectedArea |  |
| Aggregation Source -> Destination | Public ProtectedAreaDescription | Public WFD_ProtectedArea |  |
| Association LinkForNavigation Source -> Destination | Public WFD_ProtectedArea | Public ProtectedSite |  |

Attributes

| Attribute | Notes | Constraints and tags |
| --- | --- | --- |
| EU_protectedAreaCode FeatureUniqueEUCodeType Public | Unique code for the Protected Area at EU level. In the case of Protected Areas reported under other directives please use the same code, preceded by the two-letter ISO Country code if necessary. It is expected that the same and/or overlapping Protected Areas would have different identifying codes under the different Directives under which they are designated. | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| MS_code FeatureUniqueCodeType Public | Unique code for the Protected Area within the MS | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| localName PF_FreeText Public «voidable» | Local name of the Protected Area | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |
| centroid GM_Point Public | Latitude in ETRS89 of the centroid of the Protected Area. When linear or area entities are represented as points (centroids) these should be ‘geometric’ centroids in the sense that the point should fall inside a polygon representation or for linear features be a point on the line. | Default: [minOccurs = 1 ] [nillable = true ] [maxOccurs = 1 ] |
| legislationReference PF_FreeText Public «voidable» | Hyperlink or reference to the appropriate European / National / Local Legislation under which the area is protected if appropriate | Default: [maxOccurs = 1 ] [minOccurs = 0 ] |
| protectedAreaType ProtectedAreaType Public | Choose the appropriate Protected Area type from the enumeration list. | Default: [maxOccurs = 1 ] [minOccurs = 1 ] |
| URL anyURI Public «voidable» | URL for integration of your own internet-based information | Default: [minOccurs = 0 ] [maxOccurs = 1 ] |

### Hydrogeology Use Case Model

Type:		Package

Status:		Proposed. Version . Phase 1.0.

Package:	Hydrogeology_v.0.4

Detail:		Created on 2011-03-15. Last modified on 2011-03-15

GUID:		{6886487E-0E77-46e9-B7E6-489CB8338EE8}

Hydrogeological data model use cases - (Use Case diagram)

Created By:	Piotr Wojda on 2005-11-19

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{DC6BC96A-A18E-4340-A245-2FB5CC8B887A}

Figure: 31

#### <anonymous>

Type:		Note

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeology Use Case Model     Keywords:

Detail:		Created on 2006-01-05.  Last modified on 2006-01-05.

GUID:		{0BDF5489-BC08-488d-AEDD-4B6743455EF3}

Actors are the users of the system being modeled. Each Actor will have a well-defined role, and in the context of that role have useful interactions with the system.

A person may perform the role of more than one Actor, although they will only assume one role during one use case interaction.

An Actor role may be performed by a non-human system, such as another computer program.

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| NoteLink | Public Actors | Public <anonymous> |  |

#### Actors

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology Use Case Model

Detail:		Created on 2005-11-19. Last modified on 2011-02-25

GUID:		{8B46A6A6-1210-44df-B3CD-4A7BB4670167}

Actors - (Use Case diagram)

Created By:	Piotr Wojda on 2005-11-19

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{EF90DC7B-E074-45ce-BD2B-04B4634403F7}

Figure: 32

##### Hydrogeologist

Type:		Actor

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Actors     Keywords:

Detail:		Created on 2005-11-19.  Last modified on 2011-02-25.

GUID:		{F99A17F8-3CC3-49d3-8373-21B7ACA58E71}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Identify available hydrogeological data web services |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Access groundwater protection zones data |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Insert a new groundwater intake data |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Connect to web discovery/catalogue services |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Create a groundwater flow model for a new groundwater intake |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Access groundwater flow system and groundwater bodies data |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Connect a GIS application to hydrogeological data web services |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Access aquifer system and aquifers data |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Access wells and piezometers data |  |

##### Investor

Type:		Actor

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Actors     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{C9941555-CF27-4bb2-B265-3545C9CCB83C}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Investor | Public Connect a GIS application to hydrogeological data web services |  |
| UseCaseLink Source -> Destination | Public Investor | Public Access groundwater protection zones data |  |
| UseCaseLink Source -> Destination | Public Investor | Public Publish an investment project localised within protection zones |  |
| UseCaseLink Source -> Destination | Public Investor | Public Identify available hydrogeological data web services |  |
| UseCaseLink Source -> Destination | Public Investor | Public Connect to web discovery/catalogue services |  |

##### MS-User

Type:		Actor

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Actors     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{1E8438F4-5927-4abf-BC58-DEB7AAE73F5B}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public MS-User | Public Investigate the project implementation from a hydrogeological point of view |  |
| UseCaseLink Source -> Destination | Public MS-User | Public Propose a new groundwater sampling scheme |  |

#### Hydrogeological Use Cases

Type:		Package

Status:		Proposed. Version 1.0. Phase 1.0.

Package:	Hydrogeology Use Case Model

Detail:		Created on 2005-11-19. Last modified on 2011-02-23

GUID:		{4B33C825-94C7-4c77-A7AC-4E6BE2F4983B}

A new groundwater intake study - (Use Case diagram)

Created By:	Piotr Wojda on 2005-11-19

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{AF317802-0F5E-43f5-8533-20562D2AE9BF}

Figure: 33

Protection zones for a groundwater intake - (Use Case diagram)

Created By:	Piotr Wojda on 2011-02-23

Last Modified:	2012-06-28

Version:		1.0. Locked: False

GUID:		{9CAC820C-DACF-4a93-ADF6-300FB82D50CF}

Figure: 34

##### Access aquifer system and aquifers data

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{0CD5B122-9654-4c3e-B22D-75DCBF5688EB}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Extend Source -> Destination | Public Access geological/lithological data | Public Access aquifer system and aquifers data |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Access aquifer system and aquifers data |  |

##### Access existing groundwater flow models

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{301DCB93-ECE2-4f87-8C83-58B2F1516C83}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Extend Source -> Destination | Public Access existing groundwater flow models | Public Access groundwater flow system and groundwater bodies data |  |

##### Access geological/lithological data

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{67D88754-D631-4221-8BB1-096084C4AE75}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Extend Source -> Destination | Public Access geological/lithological data | Public Access aquifer system and aquifers data |  |

##### Access groundwater flow system and groundwater bodies data

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{276AB90B-F9D6-45a5-8C25-2DA6B7599D8D}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Access groundwater flow system and groundwater bodies data |  |
| Extend Source -> Destination | Public Access existing groundwater flow models | Public Access groundwater flow system and groundwater bodies data |  |

##### Access groundwater monitoring stations network

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{003AC8E2-9834-4886-A158-3C5F2863EC8F}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Extend Source -> Destination | Public Access groundwater monitoring stations network | Public Propose a new groundwater sampling scheme |  |
| Extend Source -> Destination | Public Access groundwater monitoring stations network | Public Access wells and piezometers data |  |

##### Access groundwater protection zones data

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{7D3AA2C1-8FE7-4b9f-BC97-DF183D1DE7B2}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Access groundwater protection zones data |  |
| UseCaseLink Source -> Destination | Public Investor | Public Access groundwater protection zones data |  |
| Extend Source -> Destination | Public Access protected sites data | Public Access groundwater protection zones data |  |
| Extend Source -> Destination | Public Access legislation on groundwater intake protection zones | Public Access groundwater protection zones data |  |

##### Access legislation on groundwater intake protection zones

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{331B325C-F618-42c1-9F29-689A64D460D7}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Extend Source -> Destination | Public Access legislation on groundwater intake protection zones | Public Access groundwater protection zones data |  |

##### Access protected sites data

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{B07D340C-1803-43e5-8792-953858D17F3A}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Extend Source -> Destination | Public Access protected sites data | Public Access groundwater protection zones data |  |

##### Access wells and piezometers data

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{A95A6783-143A-4d33-8D7A-E19DC43E9DDA}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Extend Source -> Destination | Public Access groundwater monitoring stations network | Public Access wells and piezometers data |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Access wells and piezometers data |  |

##### Connect a GIS application to environmental data web services

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{55AB495F-E303-4374-B980-9772A523656E}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Extend Source -> Destination | Public Connect a GIS application to environmental data web services | Public Connect a GIS application to hydrogeological data web services |  |

##### Connect a GIS application to hydrogeological data web services

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{DD994AC8-1FE4-4e4d-926F-B80669957D4F}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Investor | Public Connect a GIS application to hydrogeological data web services |  |
| Extend Source -> Destination | Public Connect a GIS application to environmental data web services | Public Connect a GIS application to hydrogeological data web services |  |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Connect a GIS application to hydrogeological data web services |  |

##### Connect to web discovery/catalogue services

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2005-11-19.  Last modified on 2011-02-25.

GUID:		{A3438FFF-0B15-4c8a-8FBD-8843C9B00D3B}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Connect to web discovery/catalogue services |  |
| UseCaseLink Source -> Destination | Public Investor | Public Connect to web discovery/catalogue services |  |

Sequence diagram: Use Case1

###### Object1

Type:		Sequence

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2006-01-05.  Last modified on 2006-01-05.

GUID:		{C2B2CF8E-C566-4372-956B-DC13A66F5F35}

##### Create a groundwater flow model for a new groundwater intake

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{C1342D9F-885D-4574-81AC-427820E74CD6}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Create a groundwater flow model for a new groundwater intake |  |

##### Identify available hydrogeological data web services

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2005-11-20.  Last modified on 2011-02-25.

GUID:		{058C12C7-8D2E-44e6-BB92-01CE1FB1A1E1}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Identify available hydrogeological data web services |  |
| UseCaseLink Source -> Destination | Public Investor | Public Identify available hydrogeological data web services |  |
| Extend Source -> Destination | Public Identify environmental data web services | Public Identify available hydrogeological data web services |  |

##### Identify environmental data web services

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{766FAE2F-B4DE-4575-BA18-F1F9FF82CBBE}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| Extend Source -> Destination | Public Identify environmental data web services | Public Identify available hydrogeological data web services |  |

##### Insert a new groundwater intake data

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{958BCEF0-629D-44f6-AD4D-45F8B84D57FC}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Hydrogeologist | Public Insert a new groundwater intake data |  |

##### Investigate the project implementation from a hydrogeological point of view

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{ED5E5A43-453B-4b0f-9E18-D66BE42DD98B}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public MS-User | Public Investigate the project implementation from a hydrogeological point of view |  |

##### Propose a new groundwater sampling scheme

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{94E6B224-8E4E-4400-B22D-2B6E073A97D5}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public MS-User | Public Propose a new groundwater sampling scheme |  |
| Extend Source -> Destination | Public Access groundwater monitoring stations network | Public Propose a new groundwater sampling scheme |  |

##### Publish an investment project localised within protection zones

Type:		UseCase

Status:		Proposed.  Version 1.0.  Phase 1.0.

Package:	Hydrogeological Use Cases     Keywords:

Detail:		Created on 2011-02-23.  Last modified on 2011-02-25.

GUID:		{EC7F11D0-274F-4e5b-A526-810C5DEB4B7F}

Connections

| Connector | Source | Target | Notes |
| --- | --- | --- | --- |
| UseCaseLink Source -> Destination | Public Investor | Public Publish an investment project localised within protection zones |  |